Camera Streaming Interface

CSI_TX_IF

MODULE SPECIFICATION

Recipient agrees that it will not knowingly export or re-export, directly or indirectly, any product or technical data (as defined by the U.S. Export Administration Regulations) including software, or any controlled product (including software) restricted by other applicable national regulations, received from Disclosing party under this Agreement, or any direct product of such technical data, to any destination to which such export or re-export is restricted or prohibited by U.S. or other applicable law, without obtaining prior authorization from U.S. Department of Commerce and other competent government authorities to the extent required by those laws. This provision shall survive termination or expiration of this Agreement. The technology or goods covered by the present agreement are classified according to the following Export Control Code Number (ECCN) in conformance with the US department of Commerce regulation and the European Export Control regulation:

-US ECCN: EAR99

and may require export or re-export license for shipping it in compliance with certain countries regulations.

| TI-Ref | Csi_tx_if |
| --- | --- |
| Document version | 1.0.21 |
| Release date | January 14, 2019 |
| Department | Embedded Processing – Processor |
| Owner | Tony Lell |

Revision History

| Revision | Author | Date | Comments |
| --- | --- | --- | --- |
| x.y.s |  |  |  |
| 0.0.1 | Michael Lachmayr | 2016 DEC20 | Draft |
| 0.1.0 | Michael Lachmayr | 2017 Apr 10 | CSI_TX_IF 0.1 release… |
| 0.1.1 | Michael Lachmayr | 2017 Apr 12 | Adding some missing topics via RAMP review. |
| 0.1.2 | Tony Lell | 9/7/2017 | Further addition of details. |
| 0.1.3 | Tony Lell | 9/14/2017 | Details on dma interface and retransmit paths |
| 0.1.4 | Tony Lell | 9/27/2017 | Various updats |
| 0.1.5 | Tony Lell | 9/28/2017 | Yuv420-8 legacy not supported, updates on retransmit limitations |
| 0.1.6 | Tony Lell | 10/6/2017 | Retransmit updates |
| 1.0.0 | Tony Lell | 10/16/2017 | Updates for sign off. |
| 1.0.1 | Tony Lell | 10/17/2017 | Updates for non interleaved restrictions |
| 1.0.2 | Tony Lell | 10/18/2017 | Added more software details and restrictions in section 9.1 |
| 1.0.3 | Tony Lell | 12/5/2017 | Added mmrs for pixel interface reset. |
| 1.0.4 | Tony Lell | 12/6/2017 | Added memory ecc comments |
| 1.0.5 | Tony Lell | 12/8/2017 | Added safety requirement |
| 1.0.6 | Tony Lell | 12/11/2017 | Updated ecc ram # in mmr |
| 1.0.7 | Tony Lell | 12/13/2017 | Spec feedback updates |
| 1.0.8 | Tony Lell | 12/22/2017 | Updated data type mmr description |
| 1.0.9 | Tony Lell | 12/28/2017 | Updated safety info for rams. |
| 1.0.10 | Tony Lell | 1/31/2018 | Minor change to reg name. data type to data sel to remove ambiguity. |
| 1.0.11 | Tony Lell | 2/15/2018 | Updated ram ecc info |
| 1.0.12 | Tony Lell | 4/9/2018 | Updated color bar mmr descriptions |
| 1.0.13 | Tony Lell | 4/16/2018 | Updated user defined types, stream idle mmrs, retransmit programming clarifications, power down sequence. |
| 1.0.14 | Tony Lell | 7/3/2018 | Added missing asf registers in mmr region |
| 1.0.15 | Tony Lell | 8/14/2018 | Added second ecc aggregator |
| 1.0.16 | Tony Lell | 8/17/2018 | Added more coments on which eggr endpoints map to aggr vector |
| 1.0.17 | Tony Lell | 8/23/2018 | Removed specifics on lane support as that is in cadence spec. |
| 1.0.18 | Tony Lell | 9/19/2018 | Updated pid. |
| 1.0.19 | Tony Lell | 10/3/2018 | Updated comments/usage on cdns ram correctable interrupts |
| 1.0.20 | Tony Lell | 11/28/2018 | Updated cdns reg descriptions |
| 1.0.21 | Tony Lell | 1/14/2018 | Updated underflow error recovery |

Contents

1.	Overview and Feature List	12

1.1	Standards	12

1.2	Features Supported	12

1.3	Features Not Supported	13

1.4	Safety	13

1.4.1	Memories	13

1.4.2	ECC aggregators	13

1.4.3	Cadence interface	14

2. Functional Description	15

2.1	Memory organization of data.	17

2.2	Functional Operation	21

3. Interrupt Conditions	21

3.1	CPU Interrupts	21

3.2	Debug Events	21

3.3	Interrupt Description	21

3.3.1	csi_level_intr/csi_pulse_intr	21

3.3.2	csi_interrupt_intr	21

3.3.3	csi_fatal_intr	21

3.3.4	csi_nonfatal_intr	21

3.3.5	uncorr_pulse_intr/uncorr_level_intr	21

4. Memory Map	23

4.1	Legend	23

4.2	MMR Region Summary	23

4.3	MMR Region: mmr__vbusp_mmr__csi2txif_regs	23

4.3.1	Register Map	24

4.3.2	Register Definitions	24

4.3.2.1	CSITX_ID (0x0)	24

4.3.2.2	COLOR_CNTL (color bar control register), (0x4)	24

4.3.2.3	COLOR_PARAM (color bar frame parameters), (0x8)	25

4.3.2.4	COLOR_START_DELAY (delay from starting first line after enabling), (0xc)	25

4.3.2.5	COLOR_LINE_DELAY (last line start to next line start delay), (0x20)	25

4.3.2.6	COLOR_FRAME_DELAY (line start to next frame start delay), (0x24)	25

4.3.2.7	RETRANS_CNTL (retransmit control register), (0x28)	25

4.3.2.8	CNTL (control register for csi rx wrapper), (0x2c)	25

4.3.2.9	F2F_DELAY[a] (delay in number of main clock cyles from start of last frame line to start of next frame first line), (0x40+(a*0x4)), a=0..15	26

4.3.2.10	CNTL[a]_DMACNTX (DMA Channel Context), (0x100+(a*0x20)), a=0..31	26

4.3.2.11	CNTL[a]_L2L_DELAY (delay in main clock cycles from start of one line to start of another line), (0x100+(a*0x20)+0x4), a=0..31	27

4.4	MMR Region: vbus2apb_wrap__vbusp_apb__csi2tx_regs	28

4.4.1	Register Map	28

4.4.2	Register Definitions	29

4.4.2.1	DEVICE_CONFIG (0x0)	29

4.4.2.2	STATUS (0x4)	30

4.4.2.3	IRQ (0x8)	30

4.4.2.4	IRQ_MASK (0xc)	32

4.4.2.5	DPHY_IRQ (0x10)	35

4.4.2.6	DPHY_IRQ_MASK (0x14)	36

4.4.2.7	TX_CONF (0x20)	36

4.4.2.8	WAIT_BURST_TIME (0x24)	36

4.4.2.9	DPHY_CFG (0x28)	37

4.4.2.10	DPHY_CLK_WAKEUP (0x2c)	37

4.4.2.11	DPHY_ULPS_WAKEUP (0x30)	38

4.4.2.12	VC0_CFG (0x40)	38

4.4.2.13	VC1_CFG (0x44)	38

4.4.2.14	VC2_CFG (0x48)	38

4.4.2.15	VC3_CFG (0x4c)	39

4.4.2.16	VC4_CFG (0x50)	39

4.4.2.17	VC5_CFG (0x54)	39

4.4.2.18	VC6_CFG (0x58)	39

4.4.2.19	VC7_CFG (0x5c)	39

4.4.2.20	VC8_CFG (0x60)	40

4.4.2.21	VC9_CFG (0x64)	40

4.4.2.22	VC10_CFG (0x68)	40

4.4.2.23	VC11_CFG (0x6c)	40

4.4.2.24	VC12_CFG (0x70)	41

4.4.2.25	VC13_CFG (0x74)	41

4.4.2.26	VC14_CFG (0x78)	41

4.4.2.27	VC15_CFG (0x7c)	41

4.4.2.28	DT0_CFG (0x80)	42

4.4.2.29	DT0_FORMAT (0x84)	42

4.4.2.30	DT1_CFG (0x88)	42

4.4.2.31	DT1_FORMAT (0x8c)	43

4.4.2.32	DT2_CFG (0x90)	43

4.4.2.33	DT2_FORMAT (0x94)	44

4.4.2.34	DT3_CFG (0x98)	44

4.4.2.35	DT3_FORMAT (0x9c)	45

4.4.2.36	DT4_CFG (0xa0)	45

4.4.2.37	DT4_FORMAT (0xa4)	46

4.4.2.38	DT5_CFG (0xa8)	46

4.4.2.39	DT5_FORMAT (0xac)	46

4.4.2.40	DT6_CFG (0xb0)	46

4.4.2.41	DT6_FORMAT (0xb4)	47

4.4.2.42	DT7_CFG (0xb8)	47

4.4.2.43	DT7_FORMAT (0xbc)	48

4.4.2.44	STREAM_IF_0_CFG (0x100)	48

4.4.2.45	STREAM_IF_1_CFG (0x104)	48

4.4.2.46	STREAM_IF_2_CFG (0x108)	48

4.4.2.47	STREAM_IF_3_CFG (0x10c)	49

4.4.2.48	DEBUG_CFG (0x110)	49

4.4.2.49	DEBUG_LN_FSM (0x114)	49

4.4.2.50	DEBUG_CLK_LN_FSM (0x118)	49

4.4.2.51	DEBUG_DATA_LN_FSM (0x11c)	50

4.4.2.52	DEBUG_PROT0_FSM (0x120)	51

4.4.2.53	DEBUG_PROT1_FSM (0x124)	52

4.4.2.54	DEBUG_PROT2_FSM (0x128)	52

4.4.2.55	DEBUG_PROT3_FSM (0x12c)	53

4.4.2.56	DPHY_STATUS (0x130)	53

4.4.2.57	DPHY_CFG1 (0x134)	53

4.4.2.58	GENERIC (0x13c)	54

4.4.2.59	ASF_INT_STATUS (0x200)	54

4.4.2.60	ASF_INT_RAW_STATUS (0x204)	54

4.4.2.61	ASF_INT_MASK (0x208)	55

4.4.2.62	ASF_INT_TEST (0x20c)	55

4.4.2.63	ASF_FATAL_NONFATAL_SELECT (0x210)	56

4.4.2.64	ASF_SRAM_CORR_FAULT_STATUS (0x220)	56

4.4.2.65	ASF_SRAM_UNCORR_FAULT_STATUS (0x224)	56

4.4.2.66	ASF_SRAM_FAULT_STATS (0x228)	56

4.4.2.67	ASF_TRANS_TO_CTRL (0x230)	57

4.4.2.68	ASF_TRANS_TO_FAULT_MASK (0x234)	57

4.4.2.69	ASF_TRANS_TO_FAULT_STATUS (0x238)	57

4.4.2.70	ASF_PROTOCOL_FAULT_MASK (0x240)	58

4.4.2.71	ASF_PROTOCOL_FAULT_STATUS (0x244)	58

4.4.2.72	ID_PROD_VER (0xffc)	59

4.5	MMR Region: cp_intd__intd_cfg__intd_cfg	60

4.5.1	Register Map	60

4.5.2	Register Definitions	60

4.5.2.1	REVISION (0x0)	60

4.5.2.2	CONTROL (0x4)	60

4.5.2.3	EOI_REG (0x10)	60

4.5.2.4	INTR_VECTOR_REG (0x14)	61

4.5.2.5	ENABLE_REG_LEVEL_0 (0x100)	61

4.5.2.6	ENABLE_REG_PULSE_0 (0x104)	61

4.5.2.7	ENABLE_CLR_REG_LEVEL_0 (0x300)	61

4.5.2.8	ENABLE_CLR_REG_PULSE_0 (0x304)	61

4.5.2.9	STATUS_REG_LEVEL_0 (0x500)	61

4.5.2.10	STATUS_REG_PULSE_0 (0x504)	62

4.5.2.11	STATUS_CLR_REG_LEVEL_0 (0x700)	62

4.5.2.12	STATUS_CLR_REG_PULSE_0 (0x704)	62

4.5.2.13	INTR_VECTOR_REG_LEVEL (0xa80)	62

4.5.2.14	INTR_VECTOR_REG_PULSE (0xa84)	62

4.6	MMR Region: ecc_aggr__cfg__regs	63

4.6.1	Register Map	63

4.6.2	Register Definitions	63

4.6.2.1	AGGR_REVISION (Aggregator Revision Register), (0x0)	63

4.6.2.2	ECC_VECTOR (ECC Vector Register), (0x8)	63

4.6.2.3	MISC_STATUS (Misc Status), (0xc)	64

4.6.2.4	SEC_EOI_REG (EOI Register), (0x3c)	64

4.6.2.5	SEC_STATUS_REG0 (Interrupt Status Register 0), (0x40)	64

4.6.2.6	SEC_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x80)	64

4.6.2.7	SEC_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0xc0)	64

4.6.2.8	DED_EOI_REG (EOI Register), (0x13c)	65

4.6.2.9	DED_STATUS_REG0 (Interrupt Status Register 0), (0x140)	65

4.6.2.10	DED_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x180)	65

4.6.2.11	DED_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x1c0)	65

4.6.2.12	AGGR_ENABLE_SET (AGGR interrupt enable set Register), (0x200)	65

4.6.2.13	AGGR_ENABLE_CLR (AGGR interrupt enable clear Register), (0x204)	66

4.6.2.14	AGGR_STATUS_SET (AGGR interrupt status set Register), (0x208)	66

4.6.2.15	AGGR_STATUS_CLR (AGGR interrupt status clear Register), (0x20c)	66

4.7	MMR Region: ecc_aggr_byte__cfg__regs	66

4.7.1	Register Map	66

4.7.2	Register Definitions	67

4.7.2.1	AGGR_REVISION (Aggregator Revision Register), (0x400)	67

4.7.2.2	ECC_VECTOR (ECC Vector Register), (0x408)	67

4.7.2.3	MISC_STATUS (Misc Status), (0x40c)	67

4.7.2.4	RESERVED_SVBUS[a] (Reserved Area for Serial VBUS Registers), (0x410+(a*0x4)), a=0..7	68

4.7.2.5	SEC_EOI_REG (EOI Register), (0x43c)	68

4.7.2.6	SEC_STATUS_REG0 (Interrupt Status Register 0), (0x440)	68

4.7.2.7	SEC_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x480)	68

4.7.2.8	SEC_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x4c0)	68

4.7.2.9	DED_EOI_REG (EOI Register), (0x53c)	69

4.7.2.10	DED_STATUS_REG0 (Interrupt Status Register 0), (0x540)	69

4.7.2.11	DED_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x580)	69

4.7.2.12	DED_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x5c0)	69

4.7.2.13	AGGR_ENABLE_SET (AGGR interrupt enable set Register), (0x600)	70

4.7.2.14	AGGR_ENABLE_CLR (AGGR interrupt enable clear Register), (0x604)	70

4.7.2.15	AGGR_STATUS_SET (AGGR interrupt status set Register), (0x608)	70

4.7.2.16	AGGR_STATUS_CLR (AGGR interrupt status clear Register), (0x60c)	70

5. Submodule information	71

6. Integration Overview	71

6.1	Parameterization Requirements	71

6.2	Memories Embedded in the Peripheral	71

6.2.1	Error Correcting Code (ECC) Support	71

6.2.2	ECC Memories	72

6.3	PLL Requirements	72

6.4	Overall Design For Test	72

6.5	Target Processes	72

6.6	Area Requirements	72

6.7	Power Requirements	72

6.8	Performance Requirements	72

6.8.1.1	MAIN CLK	72

6.8.1.2	VBUSP_CLK	73

1.1.1.1	Byte Clock	73

1.1.1.1	ESC Clock	73

7. I/O Description	73

7.1	Module I/O	73

7.1.1	synchronous Reset Interface For csi	74

7.1.2	MAIN_CLK free running clock for csi_tx_if	75

7.1.3	MAIN_CLK free running clock for csi_tx_if	75

7.1.4	ESC_CLK low speed clock for LP low power mode domm	75

7.1.5	Clock Power Stop	75

7.1.6	Power Clock Enable for csi	75

7.1.7	DFT_CLK clock for escape Clock Domain during Text... muxed in for DFT	75

7.1.8	DFT_CLK clock for dphy_tx_byte_clk Clock Domain during Text... muxed in for DFT	75

7.1.9	DFT Interface	75

7.1.10	DFT Interface	76

7.1.11	Interrupt servicing all CSI TI generated RTL aggregated by INTD	76

7.1.12	Interrupt servicing all CSI TI generated RTL aggregated by INTD	76

7.1.13	CSI interupt from CSI2TX submodule	76

7.1.14	CSI fatal interrupt from CSI2TX submodule	76

7.1.15	CSI non fatal interrupt from CSI2TX submodule	76

7.1.16	Vbusp configuration port	76

7.1.17	RETRAN retransmission stream interface from csi_rx_if to csi_tx_if	77

7.1.18	DPHY PPI interface between csi_tx_if and DPHY	78

7.1.19	PSI-L Streaming Interface	80

7.1.20	Tx FIFO RAM 0 DFT Interface	81

7.1.21	mem gpio interface	82

7.1.22	mem gpio interface	82

7.1.23	mem gpio interface	82

7.1.24	mem gpio interface	82

7.1.25	mem gpio interface	82

7.1.26	CPU 4 uncorrectable error, pulse	82

7.1.27	CPU 4 uncorrectable error, level	82

7.1.28	CPU 4 correctable error, pulse	82

7.1.29	CPU 4 correctable error, level	82

7.1.30	CPU 3 uncorrectable error, pulse	83

7.1.31	CPU 3 uncorrectable error, level	83

7.1.32	CPU 3 correctable error, pulse	83

7.1.33	CPU 3 correctable error, level	83

7.2	External Pin IO	83

7.3	Data Flow Diagram	83

8. Physical Implementation	85

8.1	Clocking	85

8.2	Clock Diagram	86

8.3	Reset	87

8.4	Latches	87

8.5	Placement and Hierarchy Information	87

8.6	Hard Macro Information	87

8.7	Power / Voltage	87

8.8	Interrupt	87

9. DFT	88

9.1	Scan Implementation	88

9.2	IDDQ Modes	88

9.3	Memory BIST	88

9.4	I/O DFT	88

10. Programmer’s guide	90

10.1	Mmr configuration guide for PSIL	90

10.2	MMR configuration for retransmit	90

10.3	MMR configuration for COLOR BAR	91

10.4	Initialization sequence	91

10.5	Real-time operating requirements	91

10.6	Power up/down sequence	91

List of Figures

Figure 1 functional block diagram	15

Figure 2 cadence stream to PSIL thread mapping	15

Figure 3 yuv422-8 memory data organization	17

Figure 4 yuv422-10 memory data organization	18

Figure 5 yuv420-8 memory data organization	18

Figure 6 yuv420-10 memory data organization	19

Figure 7 raw memory data organization(unpacked).	19

Figure 8 rgb memory data organization	20

Figure 9 raw12 -packed memory data organization	20

Figure 11 : Data Flow Diagram	83

List of Tables

Table 1 safety associated with each memory	13

Table 2 ecc aggregator conections	13

Table 3 cadence interfaces with safety injection external	14

Table 4 : RAM	71

Table 5 : ECC Memory Table	72

Table 6 : Inter-clock dependency table	85

# Reference documents

| Sl |  | Specification | Revision |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Csitx user guide |  |  | Tbd |
|  | Csirx user guide |  |  | tbd |

# Acronyms Abbreviations and Definitions

| Term | Definition |
| --- | --- |
| EOL | End of Line |
| SOL | Start of Line |
| EOF | End of frame |
| EOL | End of line |
| EOP | End of packet(EOF) |
| Stream | Cadence streaming interface |
| psil | TI streaming interface |
| CSI | Camera streaming interface(MIPI STANDARD) |
| Short packet | 1 set of data that contains meta data and control information |
| Line | A set of data, usually a single horizontal slice of an image. |
| Frame | 1 short packet or multiple lines of image data |
| Data type(DT) | Type of data, RAW, YUV, RGB, ETC… |
| thread | PSIL context. Combination of virtual channel and data type. |

SECTION I: FUNCTIONAL OVERVIEW

# Overview and Feature List

This spec was written with the assumption of some knowledge required of the csi tx controller ip provided from Cadence.

## Standards

| Document | Link | Description |
| --- | --- | --- |
| MIPI CSI2 ver1.3 |  | MIPI Spec |
| MIPI CSI2 ver2.0 |  | MIPI Spec |
| MIPI D-PHY ver1.2 |  | MIPI Spec |

## Features Supported

- CSI rev 2.0+ (16 virtual channel)

- Safety supported within the CSITX controller will be enabled.

  - Ecc on external csi-tx rams

- CSI2TX (protocol layer, interfaces to D-PHY)

  - 16 virtual channel support

    - DOES NOT SUPPORT INTERLEAVED VIRTUAL CHANNELS PER STREAM, CSITX IP limitation for J7. Working on cadence ip update for later devices.

  - DPHY Interface

    - Up to 4 Lane

  - StreamIF0: PSI_L DMA

    - 128bit wide, pixel data , bursting.

    - ByteValid per byte in Last Data Phase

    - 32 PSIL thread ID context support mapped back to CSI protocol (virtual channel & data type combinations). This is done through mmr programming.

    - Unpacks PSIL data and converts to csi tx cadence stream video format.

    - Internal FF based FIFO (Minimal size to cross clock domain)

    - External (TI) Buffer, RAM based

    - Cadence ip does not support interleaved data within a frame per stream interface. Therefore cannot interleave PSIL threads. Csi-tx ip will only allow current active thread to go through till end of frame. This is limitation on J7. Attempting to get csi tx ip updated to enable interleaving per cadence stream.

    - Arbitration to next thread is done by lowest thread id number. Care needs to be taken by software that it does not send too many threads at any given time as that could cause delays for higher thread id’s.

  - StreamIF1: Color Bar

    - 2 Pixel Wide

    - YUV422 8bit format only

    - Frame/line size configurable via mmr

    - 1 virtual channel(programmable)

    - Internal FF based FIFO (Minimal size to cross clock domain)

    - No External Buffer

  - StreamIF2: re-transmit from CSI2RX

    - Limitations to CSI protocol(cadence csi tx ip limitations)

      - DOES NOT SUPPORT Interleaved channels.  Only 1 virtual channel supported.

      - Only support RAW formats

    - Internal csi tx based FIFO, 2kx32

    - Csi tx and csi rx main clocks must be running at the same frequency and synchronous to each other.

    - Programmable mmr for which single VC is used from CSI rx to retransmit.

  - StreamIF3:

    - Second retransmit interface for second VC.

    - Same limitations of IF2

    - Programmable mmr for which single VC is used from CSI rx to retransmit.

    - WHEN CSI TX IS UPGRADED TO SUPPORT 16 VC’S INTERLEAVED,THIS INTERFACE WILL GO AWAY!

## Features Not Supported

- CSI2 ver2.0 scrambling

- CSI2 ver2.0 optional no LP (low power state) between packets

- Cropping

- Pixel processing

- Yuv420-8 legacy not supported.

- Interleaved virtual channels(csi tx limitation)

## Safety

Will target Asil B as a safety requirement.

### Memories

Table 1 safety associated with each memory

| Ram instance | safety | Comments |
| --- | --- | --- |
| Csi tx ram 0 | Inject only | Cadence ip does parity |
| Csi tx ram 1 | Inject only | Cadence ip does parity |
| Csi tx ram 2 | Inject only | Cadence ip does parity |
| Csi tx ram 3 | Inject only | Cadence ip does parity |
| Dma fifo | Ecc with No writeback | Read only once |
| Psil endpoint ram | Ecc No writeback | Read only once |

### ECC aggregators

There are 2 ecc aggregators, 1 for byte clock and the other one for all other clocks.

Table 2 ecc aggregator conections

| aggregator | connection | comment |
| --- | --- | --- |
| Ecc_aggr | Edc_ctrl | Vector 0 |
| Ecc_aggr | Psil fifo | Vector 1 |
| Ecc_aggr | Dma fifo | Vector 2 |
| Ecc_aggr_byte | Cadence stream 0 ram | Vector 0 |
| Ecc_aggr_byte | Cadence stream 1 ram | Vector 1 |
| Ecc_aggr_byte | Cadence stream 2 ram | Vector 2 |
| Ecc_aggr_byte | Cadence stream 3 ram | Vector 3 |

### Cadence interface

Cadence as a number of interfaces that have parity inputs along with the data. these are hooked up to edc_ctrl through a small piece of logic that can insert errors.

Table 3 cadence interfaces with safety injection external

| Cadence input | Comments |
| --- | --- |
| asf_pixel_data_if0_par |  |
| asf_pixel_data_if1_par |  |
| asf_pixel_data_if2_par |  |
| asf_pixel_data_if3_par |  |
| asf_paddr_par |  |
| asf_wdata_par |  |

# Functional Description

Figure 1 functional block diagram

The TI logic consists of 3 sections.

- PSIL to CDNS stream interface(shim)

This bridge converts 128(4x32) bits pixel data and converts it to single 32-bit CDNS streaming protocol of pixel data. Each 128 bit set of PSIL data is split into N M-bit data transfers based on the size_cfg mmr setting in the dma context register for a given thread. See memory organization for data and mmr programming guide for details. Via mmr mapping in the dma context register the PSIL thread id is mapped to a virtual channel and a data type. The dma context register is the thread filter. There are 32 dma context registers.

Figure 2 cadence stream to PSIL thread mapping

- Current tx does not support interleaved virtual channels. You must send the entire frame per channel before starting a new channel. TI logic will push back on thread id’s not of the current frame till frame is complete.

- TI logic will re-arbitrate after completion of thread to lowest available thread id ready for transmit, thread id 0 goes before thread id 1. This could mean that if you have multiple pending threads the higher order ones could be delayed indefinitely.

- DATA is assumed in a specific organization for tx to export data correctly. See memory organization section below.

Mmr(CNTLn L2L delay) control of line start to next line start. Also mmr (F2F delay) control of last line of frame to next start of frame first line start of next frame.

Buffering of PSIL data is 2kx128 ram.

- Color bar.

Supports YUV422 8bit format only. Color Param mmr’s are used to control height and width information. Width is divided by 8 for each color section. Colors are defined below. transfers  are done on stream interface in 32-bit packed 8-bit pixel format(required programming in cadence csi tx to understand). Virtual channel and data sel is configurable in color param mmr. Data sel inside tx controller maps data type information. Color Line to line mmrs can configure delays from enabled to start, line to line delay and Color Frame delay mmr can delay last line to first line of new frame. Once enabled it will continuously send out frames until disabled. Once disabled it will stop at end of current frame.

| Color | Y | Cb | cr |
| --- | --- | --- | --- |
| white | 235 | 128 | 128 |
| yellow | 162 | 44 | 142 |
| cyan | 131 | 156 | 44 |
| green | 112 | 72 | 58 |
| magenta | 84 | 184 | 198 |
| red | 65 | 100 | 212 |
| blue | 35 | 212 | 114 |
| black | 16 | 128 | 128 |

- Rx to tx path is a loopback path

*There is an issue with this as Cadence does not glue-lessly support this requirement. This  results in program limitations on the csirx and tx sides.

** CSITX  only supports single, non interleafed, virtual channels. Program restriction on rx side must limit this.

The tx will use stream 2 and 3 to support 2 virtual channels of rx data.  The csi rx can stream interleaved virtual channels of data. It is limited to programming the csi rx so that it only sends data on up to 2 static virtual channels. The vc0 and vc 1 mmr need to be programmed to match which virtual channels are streamed on the csi rx side so that the tx can stream them. csi tx can only stream a single virtual channel at any given time. Only RAW data formats can be used for this retransmit interface due to csi tx limitations.

.

*NOTE: It is a requirement that only color, dma or loopback streams are not active at the same point in time. The intended use model is not to have multiple combinations active at any given time. The main issue with this is that rams are not sized for concurrent operation and will likely overflow resulting in data loss. The 2 loopback streams can be active at the same time just not in addition to the color or dma streams.

## Memory organization of data.

Data must be in in memory based on below tables.

Figure 3 yuv422-8 memory data organization

Figure 4 yuv422-10 memory data organization

Figure 5 yuv420-8 memory data organization

Figure 6 yuv420-10 memory data organization

*User defined data types behave the same as RAW data types.

Figure 7 raw memory data organization(unpacked).

Figure 8 rgb memory data organization

Figure 9 raw12 -packed memory data organization

NOTE: Where data does not fill out to a byte boundary it is zero extended.

## Functional Operation

# Interrupt Conditions

## CPU Interrupts

| Name | Width | Type | Polarity | Category | Clock | Reset | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| csi_pulse_intr | 1 | pulse | high | func | vbus | mod_g_rst_n | TI error interrupts |
| csi_level_intr | 1 | level | high | func | vbus | mod_g_rst_n | TI error interrupts |
| csi_interrupt_intr | 1 | level | high | func | vbus | mod_g_rst_n | Cdns interrupt |
| csi_fatal_intr | 1 | level | high | Safety | vbus | mod_g_rst_n | Cdns interrupt |
| csi_nonfatal_intr | 1 | level | high | Safety | vbus | mod_g_rst_n | Cdns interrupt |
| uncorr_pulse_intr | 1 | Pulse | high | Safety | vbus | mod_g_rst_n | Ecc error in fifo |
| uncorr_level_intr | 1 | level | High | safety | vbus | mod_g_rst_n | Ecc error in fifo |
| corr_pulse_intr | 1 | Pulse | high | Safety | vbus | mod_g_rst_n | Ecc error in fifo |
| corr_level_intr | 1 | level | High | safety | vbus | mod_g_rst_n | Ecc error in fifo |
| cdns_ram_uncorr_pulse_intr | 1 | Pulse | high | Safety | vbus | mod_g_rst_n | Ecc error in cadence ram wrapper |
| cdns_ram_uncorr_level_intr | 1 | level | High | safety | vbus | mod_g_rst_n | Ecc error in Cadence ram wrapper |
| cdns_ram_corr_pulse_intr | 1 | Pulse | high | Safety | vbus | mod_g_rst_n | Ecc error in fifo. Can not be generated due to inject only config of ram wrapper. |
| cdns_ram_corr_level_intr | 1 | level | High | safety | vbus | mod_g_rst_n | Ecc error in fifo. Can not be generated due to inject only config of ram wrapper. |

## Debug Events

None.

## Interrupt Description

### csi_level_intr/csi_pulse_intr

These are error interrupts that are generated from TI logic. These are the conditions for these interrupts. More may be added later.

- Retransmit error on interface 2(packet error on cadence stream interface)

- Retransmit error on interface 3(packet error on cadence stream interface)

### csi_interrupt_intr

See cadence documentation, direct from cdns ip.

### csi_fatal_intr

See cadence documentation, direct from cdns ip.

### csi_nonfatal_intr

See cadence documentation, direct from cdns ip.

### uncorr_pulse_intr/uncorr_level_intr

### safety interrupts on internal fifo ram external to cadence ip

### corr_pulse_intr/uncorr_level_intr

safety interrupts on internal fifo ram external to cadence ip.

### cdns_ram_uncorr_pulse_intr/uncorr_level_intr

### safety interrupts for cadence ip ram wrappers.

### cdns_ram_corr_pulse_intr/uncorr_level_intr

safety interrupts for cadence ip ram wrappers. Due to ram wrapper being inject only, this interrupt can not be toggled.

# Memory Map

## Legend

The following sections describe the register map and register definitions for each region in the design. The state of each register after reset is shown. The key used to describe each register field is described in the following table.

| Type | Description |
| --- | --- |
| r | A read-only field. Writes have no effect. |
| w | A write-only field. Reads return zeroes. |
| rw | A readable and writable field. Writing any value to this field results in the corresponding bits written with value written. Reads do not alter the value of the field. |
| rw1tc | A write ones to clear field. Writing a one to any bit will clear the corresponding bits. Reads do not alter the value of the field. |
| rw1ts | A write ones to set bit. Writing a one to any bit will set the corresponding bit. Reads do not alter the value of the field. |
| rwi | A write to increment field. Writing a value to this field increment the field value by the value written. Reads do not alter the value of the field. |
| rwd | A write to decrement field. Writing a value to this field decrements the field value by the value written. Reads do not alter the value of the field. |
| w1tc | A write-only write ones to clear field. Writing a one to any bit will clear the corresponding bits. Reads return zeroes. |
| w1ts | A write-only write ones to set bit. Writing a one to any bit will set the corresponding bit. Reads return zeroes. |
| wi | A write-only write to increment field. Writing a value to this field increment the field value by the value written. Reads return zeroes. |
| wd | A write-only write to decrement field. Writing a value to this field decrements the field value by the value written. Reads return zeroes. |

## MMR Region Summary

The following table provides a summary of the bus interfaces and the MMR regions mapped to each interface in this module.

| Interface name | Region name | Addr size | Byte offset | Request ID |
| --- | --- | --- | --- | --- |
| vbusp | mmr__vbusp_mmr__csi2txif_regs | 12 | 0x0 | 1 |
| vbusp | vbus2apb_wrap__vbusp_apb__csi2tx_regs | 12 | 0x0 | 2 |
| vbusp | cp_intd__intd_cfg__intd_cfg | 12 | 0x0 | 3 |
| vbusp | ecc_aggr__cfg__regs | 10 | 0x0 | 0 |
| vbusp | ecc_aggr_byte__cfg__regs | 10 | 0x400 | 0 |

* Access to the ecc_aggr_byte space requires one to program the dphy tx so that the byte clocks are running. If this is not done then an error will occur on that access.

** ecc aggr and csi2tx_regs are only word accessible not byte.

## MMR Region: mmr__vbusp_mmr__csi2txif_regs

MMR in this region are for the wrapper portion of the CSI2_tx_IF.  CSI2TX MMR are in a separate region and comprise the majority of the CFG

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | CSITX_ID |  |
| 0x4 | COLOR_CNTL | color bar control register |
| 0x8 | COLOR_PARAM | color bar frame paramters |
| 0xc | COLOR_START_DELAY | delay from starting first line after enabling |
| 0x20 | COLOR_LINE_DELAY | last line start to next line start delay |
| 0x24 | COLOR_FRAME_DELAY | line start to next frame start delay |
| 0x28 | RETRANS_CNTL | retransmit control register |
| 0x2c | CNTL | control register for csi rx wrapper |
| 0x40+(a*0x4), a=0..15 | F2F_DELAY[a] | delay in clock cyles from start of last frame line to start of next frame first line |
| 0x100+(a*0x20), a=0..31 | CNTL[a]_DMACNTX | DMA Channel Context |
| 0x100+(a*0x20)+0x4, a=0..31 | CNTL[a]_L2L_DELAY | delay in clock cyles from start of one line to start of another line |

### Register Definitions

#### CSITX_ID (0x0)

nothing

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:30 | scheme | r | 0x1 | scheme |
| 29:28 | bu | r | 0x2 | bu |
| 27:16 | function | r | 0x478 | function |
| 15:11 | rtlver | r | 0x9 | rtl version |
| 10:8 | majrev | r | 0x1 | major revision |
| 7:6 | custom | r | 0x0 | custom revision |
| 5:0 | minrev | r | 0x0 | min revision |

#### COLOR_CNTL (color bar control register), (0x4)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:20 | reserved | r | 0x0 | Reserved |
| 19:16 | vchnl | rw | 0x0 | color bar virtual channel |
| 15:11 | reserved | r | 0x0 | Reserved |
| 10:08 | dtype | rw | 0x0 | color bar data type, data sel |
| 7:1 | reserved | r | 0x0 | Reserved |
| 0 | en | rw | 0x0 | 1: enable |

#### COLOR_PARAM (color bar frame parameters), (0x8)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:29 | reserved | r | 0x0 | Reserved |
| 28:16 | ih_cfg | rw | 0x0 | height in number of lines |
| 15:13 | reserved | r | 0x0 | Reserved |
| 12:00 | iw_cfg | rw | 0x0 | width in units of pixel pairs. A value of 1 means 2 pixels, 4 bytes of data. Must be >= 16. |

#### COLOR_START_DELAY (delay from starting first line after enabling), (0xc)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | line_delay | rw | 0x0 | delay in terms of main clock cycles before sending first line after enabling |

#### COLOR_LINE_DELAY (last line start to next line start delay), (0x20)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | line_delay | rw | 0x0 | delay in terms of main clock cycles from line start to next line start |

#### COLOR_FRAME_DELAY (line start to next frame start delay), (0x24)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | frame_delay | rw | 0x0 | delay in terms of main clock cycles from last line start to start of next frame |

#### RETRANS_CNTL (retransmit control register), (0x28)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:20 | reserved | r | 0x0 | Reserved |
| 19:16 | vc1 | rw | 0x1 | virtual channel of csi rx to send out stream 4 interface |
| 15:4 | reserved | r | 0x0 | Reserved |
| 3:0 | vc0 | rw | 0x0 | virtual channel of csi rx to send out stream 3 interface |

#### CNTL (control register for csi rx wrapper), (0x2c)

control register for csi rx wrapper

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:12 | reserved | r | 0x0 | Reserved |
| 11 | stream3_idle | r | 0x1 | indicates if stream interface is idle(1) or not(0) |
| 10 | stream2_idle | r | 0x1 | indicates if stream interface is idle(1) or not(0) |
| 9 | stream1_idle | r | 0x1 | indicates if stream interface is idle(1) or not(0) |
| 8 | stream0_idle | r | 0x1 | indicates if stream interface is idle(1) or not(0) |
| 7:1 | reserved | r | 0x0 | Reserved |
| 0 | pixel_reset | rw | 0x0 | reset for the pixel interface. 0=reset, 1 not in reset. this should be asserted till after you program the csi controller configuration registers |

#### F2F_DELAY[a] (delay in number of main clock cyles from start of last frame line to start of next frame first line), (0x40+(a*0x4)), a=0..15

last line start to next frame start.  Configuration for each of 16 virtual channels.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:00 | delay | rw | 0x0 | counter value delay of last line start of frame to first line of next frame |

#### CNTL[a]_DMACNTX (DMA Channel Context), (0x100+(a*0x20)), a=0..31

DMA Channel Context.  Configuration for each of 32 possible threads.  Illegal to program 2 threads with same virtual channel and data type values.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:29 | reserved | r | 0x0 | Reserved |
| 28 | yuv420_cfg | rw | 0x0 | yuv422 format enable 0: not yuv420 /n 1: yuv420 |
| 27:26 | yuv422_mode_cfg | rw | 0x3 | yuv422 mode 00:UYVY, 01:VYUY, 10:YUYV, 11:YVYU |
| 25 | yuv422_cfg | rw | 0x0 | yuv422 format enable 0: not yuv422 /n 1: yuv422 |
| 24 | reserved | r | 0x0 | Reserved |
| 23 | pack12_cfg | rw | 0x0 | pack12 format enable 0. used in conjunction with size for proper packing: not pack12 /n 1: pack12 |
| 22 | reserved | r | 0x0 | Reserved |
| 21:20 | size_cfg | rw | 0x2 | data size shift when unpacking, 00=8, 01=16, 10=32, 11=RSVD |
| 19:10 | reserved | r | 0x0 | Reserved |
| 9:06 | virtCh_cfg | rw | 0x0 | CSI virtual channel index. Supplied by MIPI CSI protocol to DPHY. For CSIver1.3 program 2MSb==0 |
| 5:00 | datsel_cfg | rw | 0x0 | CSI data type index. Supplied by MIPI CSI protocol to DPHY. This should be limited to 0-7. |

#### CNTL[a]_L2L_DELAY (delay in main clock cycles from start of one line to start of another line), (0x100+(a*0x20)+0x4), a=0..31

line to line delay.  Configuration for each of 32 possible channel contexts.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:00 | delay | rw | 0x0 | counter value delay line start to next line start within a frame |

## MMR Region: vbus2apb_wrap__vbusp_apb__csi2tx_regs

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | DEVICE_CONFIG |  |
| 0x4 | STATUS |  |
| 0x8 | IRQ |  |
| 0xc | IRQ_MASK |  |
| 0x10 | DPHY_IRQ |  |
| 0x14 | DPHY_IRQ_MASK |  |
| 0x20 | TX_CONF |  |
| 0x24 | WAIT_BURST_TIME |  |
| 0x28 | DPHY_CFG |  |
| 0x2c | DPHY_CLK_WAKEUP |  |
| 0x30 | DPHY_ULPS_WAKEUP |  |
| 0x40 | VC0_CFG |  |
| 0x44 | VC1_CFG |  |
| 0x48 | VC2_CFG |  |
| 0x4c | VC3_CFG |  |
| 0x50 | VC4_CFG |  |
| 0x54 | VC5_CFG |  |
| 0x58 | VC6_CFG |  |
| 0x5c | VC7_CFG |  |
| 0x60 | VC8_CFG |  |
| 0x64 | VC9_CFG |  |
| 0x68 | VC10_CFG |  |
| 0x6c | VC11_CFG |  |
| 0x70 | VC12_CFG |  |
| 0x74 | VC13_CFG |  |
| 0x78 | VC14_CFG |  |
| 0x7c | VC15_CFG |  |
| 0x80 | DT0_CFG |  |
| 0x84 | DT0_FORMAT |  |
| 0x88 | DT1_CFG |  |
| 0x8c | DT1_FORMAT |  |
| 0x90 | DT2_CFG |  |
| 0x94 | DT2_FORMAT |  |
| 0x98 | DT3_CFG |  |
| 0x9c | DT3_FORMAT |  |
| 0xa0 | DT4_CFG |  |
| 0xa4 | DT4_FORMAT |  |
| 0xa8 | DT5_CFG |  |
| 0xac | DT5_FORMAT |  |
| 0xb0 | DT6_CFG |  |
| 0xb4 | DT6_FORMAT |  |
| 0xb8 | DT7_CFG |  |
| 0xbc | DT7_FORMAT |  |
| 0x100 | STREAM_IF_0_CFG |  |
| 0x104 | STREAM_IF_1_CFG |  |
| 0x108 | STREAM_IF_2_CFG |  |
| 0x10c | STREAM_IF_3_CFG |  |
| 0x110 | DEBUG_CFG |  |
| 0x114 | DEBUG_LN_FSM |  |
| 0x118 | DEBUG_CLK_LN_FSM |  |
| 0x11c | DEBUG_DATA_LN_FSM |  |
| 0x120 | DEBUG_PROT0_FSM |  |
| 0x124 | DEBUG_PROT1_FSM |  |
| 0x128 | DEBUG_PROT2_FSM |  |
| 0x12c | DEBUG_PROT3_FSM |  |
| 0x130 | DPHY_STATUS |  |
| 0x134 | DPHY_CFG1 |  |
| 0x13c | GENERIC |  |
| 0x200 | ASF_INT_STATUS |  |
| 0x204 | ASF_INT_RAW_STATUS |  |
| 0x208 | ASF_INT_MASK |  |
| 0x20c | ASF_INT_TEST |  |
| 0x210 | ASF_FATAL_NONFATAL_SELECT |  |
| 0x220 | ASF_SRAM_CORR_FAULT_STATUS |  |
| 0x224 | ASF_SRAM_UNCORR_FAULT_STATUS |  |
| 0x228 | ASF_SRAM_FAULT_STATS |  |
| 0x230 | ASF_TRANS_TO_CTRL |  |
| 0x234 | ASF_TRANS_TO_FAULT_MASK |  |
| 0x238 | ASF_TRANS_TO_FAULT_STATUS |  |
| 0x240 | ASF_PROTOCOL_FAULT_MASK |  |
| 0x244 | ASF_PROTOCOL_FAULT_STATUS |  |
| 0xffc | ID_PROD_VER |  |

### Register Definitions

#### DEVICE_CONFIG (0x0)

CSI2 Transmitter Device Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:20 | reserved | r | 0x0 | Reserved |
| 19 | asf_present | r | 0x1 | Active Safety Features: 1=implemented |
| 18:14 | num_dts | r | 0x8 | Number of Datatypes DT_NUMBER=8 |
| 13:9 | num_vcs | r | 0x10 | Number of Virtual Channels VC_NUMBER=16 |
| 8:7 | datapath_size | r | 0x0 | Internal Datapath width 00 - 32 bit, 01 - 64bit, 10 - 16 bit, 11 - 8 Bits. |
| 6:4 | num_streams | r | 0x4 | Number of Stream interfaces [1-4] STREAM_IF_4 |
| 3 | cdns_phy_present | r | 0x1 | Cadence DPHY present 1 = Yes |
| 2:0 | max_lane_nb | r | 0x4 | Max Number of Lanes [1-4] |

#### STATUS (0x4)

CSI2 Transmitter Status Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | unused | r | 0x0 | Reserved |
| 6 | ulp_mode_active | r | 0x0 | Ultra Low Power Mode Active Flag When HIGH this bit indicates that ultra low power mode is active. |
| 5 | hs_mode_active | r | 0x0 | High Speed Mode Active Flag When HIGH this bit indicates that high speed mode is active. |
| 4 | frame_transmission_active | r | 0x0 | Frame Transmission Active Flag When HIGH this bit indicates that frame transmission is active. |
| 3 | Reserved_3 | r | 0x0 | Unused bits |
| 2 | configuration_active | r | 0x1 | Configuration Mode Active Flag When HIGH this bit indicates that the CSI2TX module is in the configuration mode. |
| 1 | soft_reset_active | r | 0x0 | Soft Reset Active Flag When HIGH this bit indicates that soft reset is active. |
| 0 | bypass_mode_active | r | 0x0 | Bypass Mode Active Flag When HIGH this bit indicates that bypass mode is active. |

#### IRQ (0x8)

CSI2 Transmitter Interrupt Register. Write '1' to clear

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | line_number_error3 | rw1tc | 0x0 | Pixel IF 3 Line Number Error Flag - When HIGH this bit indicates that a line number error occurred. |
| 30 | byte_count_mismatch_irq3 | rw1tc | 0x0 | Pixel IF 3 Byte Count Mismatch Flag - When HIGH this bit indicates that a byte count mismatch occurred. |
| 29 | data_flow_err_irq3 | rw1tc | 0x0 | Pixel IF 3 Data Flow Error Flag - When HIGH this bit indicates that data flow error has occurred, caused by the Frame/Line valid being asserted when the DPhy is not ready. |
| 28 | fifo_underflow_irq3 | rw1tc | 0x0 | Pixel IF 3 FIFO Underflow Flag - When HIGH this bit indicates that at least one internal FIFO underflow occurred. |
| 27 | line_end_irq3 | rw1tc | 0x0 | Pixel IF 3 Line End Flag - When HIGH this bit indicates that a line end occurred. |
| 26 | line_start_irq3 | rw1tc | 0x0 | Pixel IF 3 Line Start Flag - When HIGH this bit indicates that a line start occurred. |
| 25 | frame_end_irq3 | rw1tc | 0x0 | Pixel IF 3 Frame End Flag - When HIGH this bit indicates that a frame end occurred. |
| 24 | frame_start_irq3 | rw1tc | 0x0 | Pixel IF 3 Frame Start Flag - When HIGH this bit indicates that a frame start occurred. |
| 23 | line_number_error2 | rw1tc | 0x0 | Pixel IF 2 Line Number Error Flag - When HIGH this bit indicates that a line number error occurred. |
| 22 | byte_count_mismatch_irq2 | rw1tc | 0x0 | Pixel IF 2 Byte Count Mismatch Flag - When HIGH this bit indicates that a byte count mismatch occurred. |
| 21 | data_flow_err_irq2 | rw1tc | 0x0 | Pixel IF 2 Data Flow Error Flag - When HIGH this bit indicates that data flow error has occurred, caused by the Frame/Line valid being asserted when the DPhy is not ready. |
| 20 | fifo_underflow_irq2 | rw1tc | 0x0 | Pixel IF 2 FIFO Underflow Flag - When HIGH this bit indicates that at least one internal FIFO underflow occurred. |
| 19 | line_end_irq2 | rw1tc | 0x0 | Pixel IF 2 Line End Flag - When HIGH this bit indicates that a line end occurred. |
| 18 | line_start_irq2 | rw1tc | 0x0 | Pixel IF 2 Line Start Flag - When HIGH this bit indicates that a line start occurred. |
| 17 | frame_end_irq2 | rw1tc | 0x0 | Pixel IF 2 Frame End Flag - When HIGH this bit indicates that a frame end occurred. |
| 16 | frame_start_irq2 | rw1tc | 0x0 | Pixel IF 2 Frame Start Flag - When HIGH this bit indicates that a frame start occurred. |
| 15 | line_number_error1 | rw1tc | 0x0 | Pixel IF 1 Line Number Error Flag - When HIGH this bit indicates that a line number error occurred. |
| 14 | byte_count_mismatch_irq1 | rw1tc | 0x0 | Pixel IF 1 Byte Count Mismatch Flag - When HIGH this bit indicates that a byte count mismatch occurred. |
| 13 | data_flow_err_irq1 | rw1tc | 0x0 | Pixel IF 1 Data Flow Error Flag - When HIGH this bit indicates that data flow error has occurred, caused by the Frame/Line valid being asserted when the DPhy is not ready. |
| 12 | fifo_underflow_irq1 | rw1tc | 0x0 | Pixel IF 1 FIFO Underflow Flag - When HIGH this bit indicates that at least one internal FIFO underflow occurred. |
| 11 | line_end_irq1 | rw1tc | 0x0 | Pixel IF 1 Line End Flag - When HIGH this bit indicates that a line end occurred. |
| 10 | line_start_irq1 | rw1tc | 0x0 | Pixel IF 1 Line Start Flag - When HIGH this bit indicates that a line start occurred. |
| 9 | frame_end_irq1 | rw1tc | 0x0 | Pixel IF 1 Frame End Flag - When HIGH this bit indicates that a frame end occurred. |
| 8 | frame_start_irq1 | rw1tc | 0x0 | Pixel IF 1 Frame Start Flag - When HIGH this bit indicates that a frame start occurred. |
| 7 | line_number_error0 | rw1tc | 0x0 | Pixel IF 0 Line Number Error Flag - When HIGH this bit indicates that a line number error occurred. |
| 6 | byte_count_mismatch_irq0 | rw1tc | 0x0 | Pixel IF 0 Byte Count Mismatch Flag - When HIGH this bit indicates that a byte count mismatch occurred. |
| 5 | data_flow_err_irq0 | rw1tc | 0x0 | Pixel IF 0 Data Flow Error Flag - When HIGH this bit indicates that data flow error has occurred, caused by the Frame/Line valid being asserted when the DPhy is not ready. |
| 4 | fifo_underflow_irq0 | rw1tc | 0x0 | Pixel IF 0 FIFO Underflow Flag - When HIGH this bit indicates that at least one internal FIFO underflow occurred. |
| 3 | line_end_irq0 | rw1tc | 0x0 | Pixel IF 0 Line End Flag - When HIGH this bit indicates that a line end occurred. |
| 2 | line_start_irq0 | rw1tc | 0x0 | Pixel IF 0 Line Start Flag - When HIGH this bit indicates that a line start occurred. |
| 1 | frame_end_irq0 | rw1tc | 0x0 | Pixel IF 0 Frame End Flag - When HIGH this bit indicates that a frame end occurred. |
| 0 | frame_start_irq0 | rw1tc | 0x0 | Pixel IF 0 Frame Start Flag - When HIGH this bit indicates that a frame start occurred. |

#### IRQ_MASK (0xc)

CSI2 Transmitter Interrupt Mask Set Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | mask_line_number_error3 | rw | 0x0 | Pixel IF 3 Line Number Error Mask - Writing 1 to this bit enables interrupt generation from the line_number_error_irq bit. |
| 30 | mask_byte_count_mismatch_irq3 | rw | 0x0 | Pixel IF 3 Byte Count Mismatch Mask - Writing 1 to this bit enables interrupt generation from the byte_count_mismatch_irq bit. |
| 29 | mask_data_flow_err_irq3 | rw | 0x0 | Pixel IF 3 FIFO Overflow Mask - Writing 1 to this bit enables interrupt generation from the data_flow_err_irq bit. |
| 28 | mask_fifo_underflow_irq3 | rw | 0x0 | Pixel IF 3 FIFO Underflow Mask - Writing 1 to this bit enables interrupt generation from the fifo_underflow_irq bit. |
| 27 | mask_line_end_irq3 | rw | 0x0 | Pixel IF 3 Line End Mask - Writing 1 to this bit enables interrupt generation from the line_en_irq bit. |
| 26 | mask_line_start_irq3 | rw | 0x0 | Pixel IF 3 Line Start Mask - Writing 1 to this bit enables interrupt generation from the line_start_irq bit. |
| 25 | mask_frame_end_irq3 | rw | 0x0 | Pixel IF 3 Frame End Mask - Writing 1 to this bit enables interrupt generation from the frame_end_irq bit. |
| 24 | mask_frame_start_irq3 | rw | 0x0 | Pixel IF 3 Frame Start Mask - Writing 1 to this bit enables interrupt generation from the frame_start_irq bit. |
| 23 | mask_line_number_error2 | rw | 0x0 | Pixel IF 2 Line Number Error Mask - Writing 1 to this bit enables interrupt generation from the line_number_error_irq bit. |
| 22 | mask_byte_count_mismatch_irq2 | rw | 0x0 | Pixel IF 2 Byte Count Mismatch Mask - Writing 1 to this bit enables interrupt generation from the byte_count_mismatch_irq bit. |
| 21 | mask_data_flow_err_irq2 | rw | 0x0 | Pixel IF 2 FIFO Overflow Mask - Writing 1 to this bit enables interrupt generation from the data_flow_err_irq bit. |
| 20 | mask_fifo_underflow_irq2 | rw | 0x0 | Pixel IF 2 FIFO Underflow Mask - Writing 1 to this bit enables interrupt generation from the fifo_underflow_irq bit. |
| 19 | mask_line_end_irq2 | rw | 0x0 | Pixel IF 2 Line End Mask - Writing 1 to this bit enables interrupt generation from the line_en_irq bit. |
| 18 | mask_line_start_irq2 | rw | 0x0 | Pixel IF 2 Line Start Mask - Writing 1 to this bit enables interrupt generation from the line_start_irq bit. |
| 17 | mask_frame_end_irq2 | rw | 0x0 | Pixel IF 2 Frame End Mask - Writing 1 to this bit enables interrupt generation from the frame_end_irq bit. |
| 16 | mask_frame_start_irq2 | rw | 0x0 | Pixel IF 2 Frame Start Mask - Writing 1 to this bit enables interrupt generation from the frame_start_irq bit. |
| 15 | mask_line_number_error1 | rw | 0x0 | Pixel IF 1 Line Number Error Mask - Writing 1 to this bit enables interrupt generation from the line_number_error_irq bit. |
| 14 | mask_byte_count_mismatch_irq1 | rw | 0x0 | Pixel IF 1 Byte Count Mismatch Mask - Writing 1 to this bit enables interrupt generation from the byte_count_mismatch_irq bit. |
| 13 | mask_data_flow_err_irq1 | rw | 0x0 | Pixel IF 1 FIFO Overflow Mask - Writing 1 to this bit enables interrupt generation from the data_flow_err_irq bit. |
| 12 | mask_fifo_underflow_irq1 | rw | 0x0 | Pixel IF 1 FIFO Underflow Mask - Writing 1 to this bit enables interrupt generation from the fifo_underflow_irq bit. |
| 11 | mask_line_end_irq1 | rw | 0x0 | Pixel IF 1 Line End Mask - Writing 1 to this bit enables interrupt generation from the line_en_irq bit. |
| 10 | mask_line_start_irq1 | rw | 0x0 | Pixel IF 1 Line Start Mask - Writing 1 to this bit enables interrupt generation from the line_start_irq bit. |
| 9 | mask_frame_end_irq1 | rw | 0x0 | Pixel IF 1 Frame End Mask - Writing 1 to this bit enables interrupt generation from the frame_end_irq bit. |
| 8 | mask_frame_start_irq1 | rw | 0x0 | Pixel IF 1 Frame Start Mask - Writing 1 to this bit enables interrupt generation from the frame_start_irq bit. |
| 7 | mask_line_number_error0 | rw | 0x0 | Pixel IF 0 Line Number Error Mask - Writing 1 to this bit enables interrupt generation from the line_number_error_irq bit. |
| 6 | mask_byte_count_mismatch_irq0 | rw | 0x0 | Pixel IF 0 Byte Count Mismatch Mask - Writing 1 to this bit enables interrupt generation from the byte_count_mismatch_irq bit. |
| 5 | mask_data_flow_err_irq0 | rw | 0x0 | Pixel IF 0 FIFO Overflow Mask - Writing 1 to this bit enables interrupt generation from the data_flow_err_irq bit. |
| 4 | mask_fifo_underflow_irq0 | rw | 0x0 | Pixel IF 0 FIFO Underflow Mask - Writing 1 to this bit enables interrupt generation from the fifo_underflow_irq bit. |
| 3 | mask_line_end_irq0 | rw | 0x0 | Pixel IF 0 Line End Mask - Writing 1 to this bit enables interrupt generation from the line_en_irq bit. |
| 2 | mask_line_start_irq0 | rw | 0x0 | Pixel IF 0 Line Start Mask - Writing 1 to this bit enables interrupt generation from the line_start_irq bit. |
| 1 | mask_frame_end_irq0 | rw | 0x0 | Pixel IF 0 Frame End Mask - Writing 1 to this bit enables interrupt generation from the frame_end_irq bit. |
| 0 | mask_frame_start_irq0 | rw | 0x0 | Pixel IF 0 Frame Start Mask - Writing 1 to this bit enables interrupt generation from the frame_start_irq bit. |

#### DPHY_IRQ (0x10)

DPHY Transmitter Interrupt Status. Level sensitive interrupt bits driven by DPHY signal pins. Write 1 to clear

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | reserved | r | 0x0 | Reserved |
| 15 | err_ctrl_tx3_irq | rw1tc | 0x0 | DPHY Transmitter Lane 3 ERR_CONTROL_IRQ |
| 14 | err_ctrl_tx2_irq | rw1tc | 0x0 | DPHY Transmitter Lane 2 ERR_CONTROL_IRQ |
| 13 | err_ctrl_tx1_irq | rw1tc | 0x0 | DPHY Transmitter Lane 1 ERR_CONTROL_IRQ |
| 12 | err_ctrl_tx0_irq | rw1tc | 0x0 | DPHY Transmitter Lane 0 ERR_CONTROL_IRQ |
| 11 | err_esc_tx3_irq | rw1tc | 0x0 | DPHY Transmitter Lane 3 ERR_ESC_IRQ |
| 10 | err_esc_tx2_irq | rw1tc | 0x0 | DPHY Transmitter Lane 2 ERR_ESC_IRQ |
| 9 | err_esc_tx1_irq | rw1tc | 0x0 | DPHY Transmitter Lane 1 ERR_ESC_IRQ |
| 8 | err_esc_tx0_irq | rw1tc | 0x0 | DPHY Transmitter Lane 0 ERR_ESC_IRQ |
| 7 | err_sync_tx3_irq | rw1tc | 0x0 | DPHY Transmitter Lane 3 ERR_SYNC_ESC_IRQ |
| 6 | err_sync_tx2_irq | rw1tc | 0x0 | DPHY Transmitter Lane 2 ERR_SYNC_ESC_IRQ |
| 5 | err_sync_tx1_irq | rw1tc | 0x0 | DPHY Transmitter Lane 1 ERR_SYNC_ESC_IRQ |
| 4 | err_sync_tx0_irq | rw1tc | 0x0 | DPHY Transmitter Lane 0 ERR_SYNC_ESC_IRQ |
| 3:0 | reserved | r | 0x0 | Reserved |

#### DPHY_IRQ_MASK (0x14)

DPHY Transmitter Interrupt Mask. Writing '1' to any bit enables interrupt generation when signal irq is set and interrupts are enabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | reserved | r | 0x0 | Reserved |
| 15 | mask_err_ctrl_tx3_irq | rw | 0x0 | DPHY Transmitter Mask Lane 3 ERR_CONTROL_IRQ |
| 14 | mask_err_ctrl_tx2_irq | rw | 0x0 | DPHY Transmitter Mask Lane 2 ERR_CONTROL_IRQ |
| 13 | mask_err_ctrl_tx1_irq | rw | 0x0 | DPHY Transmitter Mask Lane 1 ERR_CONTROL_IRQ |
| 12 | mask_err_ctrl_tx0_irq | rw | 0x0 | DPHY Transmitter Mask Lane 0 ERR_CONTROL_IRQ |
| 11 | mask_err_esc_tx3_irq | rw | 0x0 | DPHY Transmitter Mask Lane 3 ERR_ESC_IRQ |
| 10 | mask_err_esc_tx2_irq | rw | 0x0 | DPHY Transmitter Mask Lane 2 ERR_ESC_IRQ |
| 9 | mask_err_esc_tx1_irq | rw | 0x0 | DPHY Transmitter Mask Lane 1 ERR_ESC_IRQ |
| 8 | mask_err_esc_tx0_irq | rw | 0x0 | DPHY Transmitter Mask Lane 0 ERR_ESC_IRQ |
| 7 | mask_err_sync_tx3_irq | rw | 0x0 | DPHY Transmitter Mask Lane 3 ERR_SYNC_ESC_IRQ |
| 6 | mask_err_sync_tx2_irq | rw | 0x0 | DPHY Transmitter Mask Lane 2 ERR_SYNC_ESC_IRQ |
| 5 | mask_err_sync_tx1_irq | rw | 0x0 | DPHY Transmitter Mask Lane 1 ERR_SYNC_ESC_IRQ |
| 4 | mask_err_sync_tx0_irq | rw | 0x0 | DPHY Transmitter Mask Lane 0 ERR_SYNC_ESC_IRQ |
| 3:0 | reserved | r | 0x0 | Reserved |

#### TX_CONF (0x20)

CSI2 Transmitter Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | irq_enable | rw | 0x0 | Interrupt Enable - Writing 1 to this bit enables interrupts. |
| 30:3 | reserved | r | 0x0 | Reserved |
| 2 | configuration_request | rw | 0x1 | Configuration Request - Writing 1 to this bit enables configuration mode. |
| 1 | soft_reset_request | rw | 0x0 | Soft Reset Request - Writing 1 to this bit enables soft reset. |
| 0 | bypass_mode_enable | rw | 0x0 | Bypass Mode - Enable Writing 1 to this bit enables bypass mode. |

#### WAIT_BURST_TIME (0x24)

CSI2 Transmitter DPHY Wait Time configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | reserved | r | 0x0 | Reserved |
| 23:16 | tx_clock_exit_time | rw | 0x5 | Tx clock exit time - Number of tx_byte_clk cycles corresponding to the HS clock exit time. |
| 15:8 | reserved | r | 0x0 | Reserved |
| 7:0 | wait_burst_time_cnt | rw | 0x5 | Wait Burst Time - Number of tx_byte_clk cycles corresponding to the inter HS burst gap. |

#### DPHY_CFG (0x28)

CSI2 Transmitter DPHY Lane Enable configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:17 | reserved | r | 0x0 | Reserved |
| 16 | dphy_clk_reset | rw | 0x0 | DPHY Clock Lane Reset - Active low reset for DPHY clock lane. |
| 15 | dphy_ln_3_reset | rw | 0x0 | DPHY Line 0 Reset - Active low reset for DPHY data lane 3. |
| 14 | dphy_ln_2_reset | rw | 0x0 | DPHY Line 0 Reset - Active low reset for DPHY data lane 2. |
| 13 | dphy_ln_1_reset | rw | 0x0 | DPHY Line 0 Reset - Active low reset for DPHY data lane 1. |
| 12 | dphy_ln_0_reset | rw | 0x0 | DPHY Line 0 Reset - Active low reset for DPHY data lane 0. |
| 11 | dphy_cal_enable | rw | 0x0 | DPHY Calibration Enable. Enables caliberation for DPHY speeds over 1.5G. |
| 10 | dphy_clock_mode | rw | 0x0 | DPHY Clock Mode. Select the DPHY Clock Lane mode as Continuous [0], Non Continuous [1]. |
| 9:8 | dphy_mode | rw | 0x2 | DPHY Mode. Select the DPHY clock mode during Non Continuous operation ultra low power [00], high speed [01] or low power stop state [10]. |
| 7:5 | reserved | r | 0x0 | Reserved |
| 4 | dphy_clk_enable | rw | 0x0 | DPHY Clock Lane - Active high enable for DPHY clock lane. |
| 3 | dphy_ln_3_enable | rw | 0x0 | DPHY Lane 3 Enable - Active high enable for DPHY data lane 0. |
| 2 | dphy_ln_2_enable | rw | 0x0 | DPHY Lane 2 Enable - Active high enable for DPHY data lane 0. |
| 1 | dphy_ln_1_enable | rw | 0x0 | DPHY Lane 1 Enable - Active high enable for DPHY data lane 0. |
| 0 | dphy_ln_0_enable | rw | 0x0 | DPHY Lane 0 Enable - Active high enable for DPHY data lane 0. |

#### DPHY_CLK_WAKEUP (0x2c)

CSI2 Transmitter DPHY Clock Lane wakeup time configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | reserved | r | 0x0 | Reserved |
| 15:0 | ulps_clk_lane_wakeup | rw | 0x4e20 | DPHY clock lane wakeup time in esc_clk cycles. |

#### DPHY_ULPS_WAKEUP (0x30)

CSI2 Transmitter DPHY Data Lane wakeup time configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | reserved | r | 0x0 | Reserved |
| 15:0 | ulps_data_lane_wakeup | rw | 0x4e20 | DPHY data lane wakeup time in in esc_clk cycles. |

#### VC0_CFG (0x40)

CSI2 Transmitter Virtual Channel 0 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_0_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 0. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_0_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 0. |

#### VC1_CFG (0x44)

CSI2 Transmitter Virtual Channel 1 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_1_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 1. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_1_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 1. |

#### VC2_CFG (0x48)

CSI2 Transmitter Virtual Channel 2 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_2_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 2. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_2_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 2. |

#### VC3_CFG (0x4c)

CSI2 Transmitter Virtual Channel 3 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_3_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 3. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_3_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 3. |

#### VC4_CFG (0x50)

CSI2 Transmitter Virtual Channel 4 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_4_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 4. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_4_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 4. |

#### VC5_CFG (0x54)

CSI2 Transmitter Virtual Channel 5 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_5_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 5. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_5_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 5. |

#### VC6_CFG (0x58)

CSI2 Transmitter Virtual Channel 6 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_6_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 6. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_6_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 6. |

#### VC7_CFG (0x5c)

CSI2 Transmitter Virtual Channel 7 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_7_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 7. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_7_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 7. |

#### VC8_CFG (0x60)

CSI2 Transmitter Virtual Channel 8 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_8_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 8. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_8_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 8. |

#### VC9_CFG (0x64)

CSI2 Transmitter Virtual Channel 9 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_9_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 9. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_9_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 9. |

#### VC10_CFG (0x68)

CSI2 Transmitter Virtual Channel 10 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_10_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 10. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_10_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 10. |

#### VC11_CFG (0x6c)

CSI2 Transmitter Virtual Channel 11 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_11_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 11. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_11_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 11. |

#### VC12_CFG (0x70)

CSI2 Transmitter Virtual Channel 12 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_12_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 12. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_12_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 12. |

#### VC13_CFG (0x74)

CSI2 Transmitter Virtual Channel 13 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_13_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 13. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_13_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 13. |

#### VC14_CFG (0x78)

CSI2 Transmitter Virtual Channel 14 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_14_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 14. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_14_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 14. |

#### VC15_CFG (0x7c)

CSI2 Transmitter Virtual Channel 15 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_15_max_frame_number | rw | 0x1 | Max Frame Number - Maximum number of frames on Virtual Channel 15. |
| 15:1 | reserved | r | 0x0 | Reserved |
| 0 | vc_15_frame_count_en | rw | 0x0 | Frame Count Enable - Writing 1 to this bit enables frame count on Virtual Channel 15. |

#### DT0_CFG (0x80)

CSI2 Transmitter Data Type 0 Configuration Register with pixel_dt_sel[:] = 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_0_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 0 with pixel_dt_sel[:] = 0. |
| 8 | dt_0_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 0 with pixel_dt_sel[:] = 0. |
| 7:2 | dt_0_data_type | rw | 0x12 | Data Type - Type of data on Data Type 0 with pixel_dt_sel[:] = 0. Default ED [0x12] |
| 1 | dt_0_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 0. |
| 0 | dt_0_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 0. |

#### DT0_FORMAT (0x84)

CSI2 Transmitter Data Type 0 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_0_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 0 with pixel_dt_sel[:] = 0. |
| 15:0 | dt_0_max_line_number | rw | 0x1 | Max Line Number - Maximum number of lines on Data Type 0 with pixel_dt_sel[:] = 0. |

#### DT1_CFG (0x88)

CSI2 Transmitter Data Type 1 Configuration Register with pixel_dt_sel[:] = 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_1_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 1 with pixel_dt_sel[:] = 1. |
| 8 | dt_1_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 1 with pixel_dt_sel[:] = 1. |
| 7:2 | dt_1_data_type | rw | 0x1e | Data Type - Type of data on Data Type 1 with pixel_dt_sel[:] = 1. Default YUV422 8bit [0x1E] |
| 1 | dt_1_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 1. |
| 0 | dt_1_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 1. |

#### DT1_FORMAT (0x8c)

CSI2 Transmitter Data Type 1 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_1_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 1 with pixel_dt_sel[:] = 1. |
| 15:0 | dt_1_max_line_number | rw | 0x0 | Max Line Number - Maximum number of lines on Data Type 1 with pixel_dt_sel[:] = 1. |

#### DT2_CFG (0x90)

CSI2 Transmitter Data Type 2 Configuration Register with pixel_dt_sel[:] = 2

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_2_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 2 with pixel_dt_sel[:] = 2. |
| 8 | dt_2_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 2 with pixel_dt_sel[:] = 2. |
| 7:2 | dt_2_data_type | rw | 0x2a | Data Type - Type of data on Data Type 2 with pixel_dt_sel[:] = 2. Default RAW8 [0x2A] |
| 1 | dt_2_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 2. |
| 0 | dt_2_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 2. |

#### DT2_FORMAT (0x94)

CSI2 Transmitter Data Type 2 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_2_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 2 with pixel_dt_sel[:] = 2. |
| 15:0 | dt_2_max_line_number | rw | 0x0 | Max Line Number - Maximum number of lines on Data Type 2 with pixel_dt_sel[:] = 2. |

#### DT3_CFG (0x98)

CSI2 Transmitter Data Type 3 Configuration Register with pixel_dt_sel[:] = 3

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_3_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 3 with pixel_dt_sel[:] = 3. |
| 8 | dt_3_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 3 with pixel_dt_sel[:] = 3. |
| 7:2 | dt_3_data_type | rw | 0x24 | Data Type - Type of data on Data Type 3 with pixel_dt_sel[:] = 3. Default RGB8 [0x24] |
| 1 | dt_3_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 3. |
| 0 | dt_3_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 3. |

#### DT3_FORMAT (0x9c)

CSI2 Transmitter Data Type 3 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_3_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 3 with pixel_dt_sel[:] = 3. |
| 15:0 | dt_3_max_line_number | rw | 0x0 | Max Line Number - Maximum number of lines on Data Type 3 with pixel_dt_sel[:] = 3. |

#### DT4_CFG (0xa0)

CSI2 Transmitter Data Type 4 Configuration Register with pixel_dt_sel[:] = 4

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_4_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 4 with pixel_dt_sel[:] = 4. |
| 8 | dt_4_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 4 with pixel_dt_sel[:] = 4. |
| 7:2 | dt_4_data_type | rw | 0x0 |  |
| 1 | dt_4_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 4. |
| 0 | dt_4_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 4. |

#### DT4_FORMAT (0xa4)

CSI2 Transmitter Data Type 4 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_4_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 4 with pixel_dt_sel[:] = 4. |
| 15:0 | dt_4_max_line_number | rw | 0x0 | Max Line Number - Maximum number of lines on Data Type 4 with pixel_dt_sel[:] = 4. |

#### DT5_CFG (0xa8)

CSI2 Transmitter Data Type 5 Configuration Register with pixel_dt_sel[:] = 5

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_5_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 5 with pixel_dt_sel[:] = 5. |
| 8 | dt_5_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 5 with pixel_dt_sel[:] = 5. |
| 7:2 | dt_5_data_type | rw | 0x0 |  |
| 1 | dt_5_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 5. |
| 0 | dt_5_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 5. |

#### DT5_FORMAT (0xac)

CSI2 Transmitter Data Type 5 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_5_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 5 with pixel_dt_sel[:] = 5. |
| 15:0 | dt_5_max_line_number | rw | 0x0 | Max Line Number - Maximum number of lines on Data Type 5 with pixel_dt_sel[:] = 5. |

#### DT6_CFG (0xb0)

CSI2 Transmitter Data Type 6 Configuration Register with pixel_dt_sel[:] = 6

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_6_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 6 with pixel_dt_sel[:] = 6. |
| 8 | dt_6_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 6 with pixel_dt_sel[:] = 6. |
| 7:2 | dt_6_data_type | rw | 0x0 |  |
| 1 | dt_6_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 6. |
| 0 | dt_6_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 6. |

#### DT6_FORMAT (0xb4)

CSI2 Transmitter Data Type 6 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_6_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 6 with pixel_dt_sel[:] = 6. |
| 15:0 | dt_6_max_line_number | rw | 0x0 | Max Line Number - Maximum number of lines on Data Type 6 with pixel_dt_sel[:] = 6. |

#### DT7_CFG (0xb8)

CSI2 Transmitter Data Type 7 Configuration Register with pixel_dt_sel[:] = 7

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | dt_7_packed_enable | rw | 0x0 | Packed Enable - data stream is prepacked in to 32bit words on the pixel interface, the data is sent with the defined Data Type 7 with pixel_dt_sel[:] = 7. |
| 8 | dt_7_extd_data_type | rw | 0x0 | Extended Data Type - Type of data on Data Type 7 with pixel_dt_sel[:] = 7. |
| 7:2 | dt_7_data_type | rw | 0x0 |  |
| 1 | dt_7_lsle_generation_en | rw | 0x0 | Line Start And Line End Short Packet Generation Enable - Writing 1 to this bit enables Line Start and Line End generation on Data Type 7. |
| 0 | dt_7_line_count_en | rw | 0x0 | Line Count Enable - Writing 1 to this bit enables line count on Data Type 7. |

#### DT7_FORMAT (0xbc)

CSI2 Transmitter Data Type 7 Format Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dt_7_bytes_line_number | rw | 0x64 | Bytes Per Line - Bytes per line on Data Type 7 with pixel_dt_sel[:] = 7. |
| 15:0 | dt_7_max_line_number | rw | 0x0 | Max Line Number - Maximum number of lines on Data Type 7 with pixel_dt_sel[:] = 7. |

#### STREAM_IF_0_CFG (0x100)

CSI2 Stream 0 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | reserved | r | 0x0 | Reserved |
| 15:0 | stream_if_0_fill_level | rw | 0x0 | Fill Level - Minimum number of packed 32 words loaded into the stream fifo before Tx will start for Stream if 0. |

#### STREAM_IF_1_CFG (0x104)

CSI2 Stream 1 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | reserved | r | 0x0 | Reserved |
| 23 | stream_if_1_slave_mode | rw | 0x0 | Stream Slave Mode suppresses frame start/end packets in stream 1. To be used when DT interleving is required in the same VC. NOTE: the stream must then be paired with stream O as the master on the StreamIF |
| 22:16 | reserved | r | 0x0 | Reserved |
| 15:0 | stream_if_1_fill_level | rw | 0x0 | Fill Level - Minimum number of packed 32 words loaded into the stream fifo before Tx will start for Stream if 1. |

#### STREAM_IF_2_CFG (0x108)

CSI2 Stream 2 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | reserved | r | 0x0 | Reserved |
| 23 | stream_if_2_slave_mode | rw | 0x0 | Stream Slave Mode suppresses frame start/end packets in stream 2. To be used when DT interleving is required in the same VC. NOTE: the stream must then be paired with stream O as the master on the StreamIF |
| 22:16 | reserved | r | 0x0 | Reserved |
| 15:0 | stream_if_2_fill_level | rw | 0x0 | Fill Level - Minimum number of packed 32 words loaded into the stream fifo before Tx will start for Stream if 2. |

#### STREAM_IF_3_CFG (0x10c)

CSI2 Stream 3 Configuration Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | reserved | r | 0x0 | Reserved |
| 23 | stream_if_3_slave_mode | rw | 0x0 | Stream Slave Mode suppresses frame start/end packets in stream 3. To be used when DT interleving is required in the same VC. NOTE: the stream must then be paired with stream O as the master on the StreamIF |
| 22:16 | reserved | r | 0x0 | Reserved |
| 15:0 | stream_if_3_fill_level | rw | 0x0 | Fill Level - Minimum number of packed 32 words loaded into the stream fifo before Tx will start for Stream if 3. |

#### DEBUG_CFG (0x110)

CSI2 Transmitter Debug Enable Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | dbg_en | rw | 0x0 | Debug Enable. This bit enables debug when high. If low then other DEBUG registers read 0. |

#### DEBUG_LN_FSM (0x114)

Debug Register for Lane FSM.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:10 | reserved | r | 0x0 | Reserved |
| 9 | new_burst_allowed | r | 0x0 | Lane Mangement FSM. New burst allowed - WAIT_BURST_TIME expired |
| 8 | packet_valid_r | r | 0x0 | Lane Mangement FSM. Valid packet in packet register |
| 7 | packet_valid_in | r | 0x0 | Lane Mangement FSM. Valid packet at input |
| 6 | end_of_burst | r | 0x0 | Lane Mangement FSM. End of High speed burst |
| 5 | trans_active | r | 0x0 | Lane Mangement FSM. HS transmission active |
| 4 | start_hs_trans | r | 0x0 | Lane Mangement FSM. HS transmission ready |
| 3 | reserved | r | 0x0 | Reserved |
| 2:0 | lane_mgr_fsm_st | r | 0x0 | Lane Mangement FSM. State of Lane Management FSM |

#### DEBUG_CLK_LN_FSM (0x118)

Debug Register for Clock Lane FSM

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:14 | reserved | r | 0x0 | Reserved |
| 13 | ulps_active_clk | r | 0x0 | Clock Lane FSM. ULPS active from DPHY |
| 12 | ulps_mode_active | r | 0x0 | Clock Lane FSM. ULPS mode active |
| 11 | ulps_exit_clk_ppi | r | 0x0 | Clock Lane FSM. ULPS EXIT Request to DPHY |
| 10 | ulps_request_clk_ppi | r | 0x0 | Clock Lane FSM. ULPS request to DPHY |
| 9 | hs_mode_active_clk | r | 0x0 | Clock Lane FSM. High speed mode active |
| 8 | request_hs_clk_ppi | r | 0x0 | Clock Lane FSM. High speed clock request |
| 7 | ready_hs_clk | r | 0x0 | Clock Lane FSM. High speed clock active from DPHY |
| 6 | ulps_wakeup_count_done_cl | r | 0x0 | Clock Lane FSM. Clock lane ULPS wakeup counter expired |
| 5 | ulps_request_clk | r | 0x0 | Clock Lane FSM. Request ULPS mode |
| 4 | hs_mode_req | r | 0x0 | Clock Lane FSM. Request High Speed mode |
| 3:2 | ulps_clk_fsm | r | 0x0 | Clock Lane FSM. State of ULPS control FSM. |
| 1:0 | hs_clk_fsm | r | 0x0 | Clock Lane FSM. State of High Speed Clock FSM. |

#### DEBUG_DATA_LN_FSM (0x11c)

Debug Register for Data Lane FSM.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:17 | reserved | r | 0x0 | Reserved |
| 16 | ulps_active | r | 0x0 | Data Lane FSM. Clock and data lane ULPS active |
| 15 | data_ulps_active | r | 0x0 | Debug Register for Data Lane FSM. Data lane ULPS active |
| 14 | tx_ulps_exit_esc | r | 0x0 | Data Lane FSM. Exit escape mode to DPHY |
| 13 | tx_ulps_esc | r | 0x0 | Data Lane FSM. ULPS escape mode to DPHY |
| 12 | tx_request_esc | r | 0x0 | Data Lane FSM. Request escape mode to DPHY |
| 11 | reserved | r | 0x0 | Reserved |
| 10 | ulps_wakeup_count_done_dl | r | 0x0 | Data Lane FSM. Data Lane ULPS wakeup counter expired |
| 9:6 | ulps_active_n | r | 0x0 | Data Lane FSM. Data Lane ULPS active from DPHY |
| 5 | hs_mode_req_sync | r | 0x0 | Data Lane FSM. HS Mode request |
| 4 | ulps_req_sync | r | 0x0 | Data Lane FSM. ULPS Request |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1:0 | ulps_data_lane_fsm | r | 0x0 | Data Lane FSM. State of Data Lane FSM FSM |

#### DEBUG_PROT0_FSM (0x120)

Debug Register for Pixel IF0 Protocol FSM.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:19 | reserved | r | 0x0 | Reserved |
| 18:15 | virtual_channel_if0 | r | 0x0 | Pixel IF0 Protocol FSM. State of the top level Virtual Channel select signals State of the Virtual Channel[3:2] select signals when VCX enabled, otherwise '00' and state of the Virtual Channel[1:0] |
| 14:12 | data_type_if0 | r | 0x0 | Pixel IF0 Protocol FSM. State of the Data Type select signals |
| 11 | frame_valid_if0 | r | 0x0 | Pixel IF0 Protocol FSM. State of the Frame Valid signal |
| 10 | line_valid_if0 | r | 0x0 | Pixel IF0 Protocol FSM. State of the Line Valid signal |
| 9 | reserved | r | 0x0 | Reserved |
| 8 | last_payload_data_if0 | r | 0x0 | Pixel IF0 Protocol FSM. Last payload data in long packet |
| 7 | payload_fifo_empty_if0 | r | 0x0 | Pixel IF0 Protocol FSM. Payload FIFO empty |
| 6:0 | prot_fsm_if0 | r | 0x0 | Pixel IF0 Protocol FSM. State of Protocol Control FSM |

#### DEBUG_PROT1_FSM (0x124)

Debug Register for Pixel IF1 Protocol FSM.  (Optional: Available when PIXEL_IF2 or 4)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:19 | reserved | r | 0x0 | Reserved |
| 18:15 | virtual_channel_if1 | r | 0x0 | Pixel IF1 Protocol FSM. State of the top level Virtual Channel select signals State of the Virtual Channel[3:2] select signals when VCX enabled, otherwise '00' and state of the Virtual Channel[1:0] |
| 14:12 | data_type_if1 | r | 0x0 | Pixel IF1 Protocol FSM. State of the Data Type select signals |
| 11 | frame_valid_if1 | r | 0x0 | Pixel IF1 Protocol FSM. State of the Frame Valid signal |
| 10 | line_valid_if1 | r | 0x0 | Pixel IF1 Protocol FSM. State of the Line Valid signal |
| 9 | reserved | r | 0x0 | Reserved |
| 8 | last_payload_data_if1 | r | 0x0 | Pixel IF1 Protocol FSM. Last payload data in long packet |
| 7 | payload_fifo_empty_if1 | r | 0x0 | Pixel IF1 Protocol FSM. Payload FIFO empty |
| 6:0 | prot_fsm_if1 | r | 0x0 | Pixel IF1 Protocol FSM. State of Protocol Control FSM |

#### DEBUG_PROT2_FSM (0x128)

Debug Register for Pixel IF2 Protocol FSM.  (Optional: Available when PIXEL_IF4)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:19 | reserved | r | 0x0 | Reserved |
| 18:15 | virtual_channel_if2 | r | 0x0 | Pixel IF2 Protocol FSM. State of the top level Virtual Channel select signals State of the Virtual Channel[3:2] select signals when VCX enabled, otherwise '00' and state of the Virtual Channel[1:0] |
| 14:12 | data_type_if2 | r | 0x0 | Pixel IF2 Protocol FSM. State of the Data Type select signals |
| 11 | frame_valid_if2 | r | 0x0 | Pixel IF2 Protocol FSM. State of the Frame Valid signal |
| 10 | line_valid_if2 | r | 0x0 | Pixel IF2 Protocol FSM. State of the Line Valid signal |
| 9 | reserved | r | 0x0 | Reserved |
| 8 | last_payload_data_if2 | r | 0x0 | Pixel IF2 Protocol FSM. Last payload data in long packet |
| 7 | payload_fifo_empty_if2 | r | 0x0 | Pixel IF2 Protocol FSM. Payload FIFO empty |
| 6:0 | prot_fsm_if2 | r | 0x0 | Pixel IF2 Protocol FSM. State of Protocol Control FSM |

#### DEBUG_PROT3_FSM (0x12c)

Debug Register for Pixel IF3 Protocol FSM.  (Optional: Available when PIXEL_IF4)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:19 | reserved | r | 0x0 | Reserved |
| 18:15 | virtual_channel_if3 | r | 0x0 | Pixel IF3 Protocol FSM. State of the top level Virtual Channel select signals State of the Virtual Channel[3:2] select signals when VCX enabled, otherwise '00' and state of the Virtual Channel[1:0] |
| 14:12 | data_type_if3 | r | 0x0 | Pixel IF3 Protocol FSM. State of the Data Type select signals |
| 11 | frame_valid_if3 | r | 0x0 | Pixel IF3 Protocol FSM. State of the Frame Valid signal |
| 10 | line_valid_if3 | r | 0x0 | Pixel IF3 Protocol FSM. State of the Line Valid signal |
| 9 | reserved | r | 0x0 | Reserved |
| 8 | last_payload_data_if3 | r | 0x0 | Pixel IF3 Protocol FSM. Last payload data in long packet |
| 7 | payload_fifo_empty_if3 | r | 0x0 | Pixel IF3 Protocol FSM. Payload FIFO empty |
| 6:0 | prot_fsm_if3 | r | 0x0 | Pixel IF3 Protocol FSM. State of Protocol Control FSM |

#### DPHY_STATUS (0x130)

DPHY Transmitter Status.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:13 | reserved | r | 0x0 | Reserved |
| 12 | dphy_ulps_active_n_clk | r | 0x1 | DPHY Transmitter ULPS_ACTIVE_N Clock Lane Status. |
| 11:8 | dphy_ulps_active_n_dl | r | 0xf | DPHY Transmitter ULPS_ACTIVE_N Data Lane [3:0] Status. |
| 7:5 | reserved | r | 0x0 | Reserved |
| 4 | dphy_stopstate_clk | r | 0x1 | DPHY Transmitter STOP_STATE Clock Lane Status. |
| 3:0 | dphy_stopstate_dl | r | 0xf | DPHY Transmitter STOP_STATE Data Lane [3:0] Status. |

#### DPHY_CFG1 (0x134)

DPHY Transmitter Configuration.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:28 | reserved | r | 0x0 | Reserved |
| 27:25 | gen_powerdown | rw | 0x0 | DPHY Transmitter Power Down Common Module |
| 24 | c_powerdown | rw | 0x0 | DPHY Transmitter Power Down TX Clock |
| 23:20 | d_powerdown | rw | 0x0 | DPHY Transmitter Power Down TX Data Lanes |
| 19:9 | reserved | r | 0x0 | Reserved |
| 8 | swap_dp_dn_ctx | rw | 0x0 | DPHY Transmitter Swap DP_DN on Clock Lane |
| 7:4 | swap_dp_dn_tx0 | rw | 0x0 | DPHY Transmitter Swap DP_DN on TX Data Lanes |
| 3:0 | force_stop_mode_tx0 | rw | 0x0 | DPHY Transmitter FORCE_STOP_MODE TX Data Lanes |

#### GENERIC (0x13c)

CSI2 Transmitter Test Register.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | test_generic_status | r | 0x0 | CSI2 Transmitter Test Generic Status signals. |
| 15:0 | test_generic_ctrl | rw | 0x0 | CSI2 Transmitter Test Generic Control signals. |

#### ASF_INT_STATUS (0x200)

ASF Interrupt Status Register. This register indicates the source of ASF interrupts. The corresponding bit in the mask register must be clear for a bit to be set. If any bit is set in this register the asf_int_fatal or asf_int_nonfatal signal will be asserted. Writing to either raw or masked status registers, clear both registers. For test purposes, trigger signal interrupt event by writing to the ASF interrupt status test register.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err | rw1tc | 0x0 | Integrity error interrupt |
| 5 | asf_protocol_err | rw1tc | 0x0 | Protocol error interrupt |
| 4 | asf_trans_to_err | rw1tc | 0x0 | Transaction timeouts error interrupt |
| 3 | asf_csr_err | rw1tc | 0x0 | Configuration and status registers error interrupt |
| 2 | asf_dap_err | rw1tc | 0x0 | Data and address paths parity error interrupt |
| 1 | asf_sram_uncorr_err | rw1tc | 0x0 | SRAM uncorrectable error interrupt |
| 0 | asf_sram_corr_err | rw1tc | 0x0 | SRAM correctable error interrupt |

#### ASF_INT_RAW_STATUS (0x204)

ASF Interrupt Raw Status Register. A bit set in this raw register indicates a source of ASF fault in the corresponding feature. Writing to either raw or masked status registers, clear both registers. For test purposes, trigger signal interrupt event by writing to the ASF interrupt status test register.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err | rw1tc | 0x0 | Integrity error interrupt |
| 5 | asf_protocol_err | rw1tc | 0x0 | Protocol error interrupt |
| 4 | asf_trans_to_err | rw1tc | 0x0 | Transaction timeouts error interrupt |
| 3 | asf_csr_err | rw1tc | 0x0 | Configuration and status registers error interrupt |
| 2 | asf_dap_err | rw1tc | 0x0 | Data and address paths parity error interrupt |
| 1 | asf_sram_uncorr_err | rw1tc | 0x0 | SRAM uncorrectable error interrupt |
| 0 | asf_sram_corr_err | rw1tc | 0x0 | SRAM correctable error interrupt |

#### ASF_INT_MASK (0x208)

The ASF interrupt mask register indicating which interrupt bits in the ASF interrupt status register are masked. All bits are set at reset. Clear the individual bit to enable the corresponding interrupt.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err_mask | rw | 0x1 | Mask bit for integrity error interrupt |
| 5 | asf_protocol_err_mask | rw | 0x1 | Mask bit for protocol error interrupt. |
| 4 | asf_trans_to_err_mask | rw | 0x1 | Mask bit for transaction timeouts error interrupt. |
| 3 | asf_csr_err_mask | rw | 0x1 | Mask bit for configuration and status registers error interrupt. |
| 2 | asf_dap_err_mask | rw | 0x1 | Mask bit for data and address paths parity error interrupt. |
| 1 | asf_sram_uncorr_err_mask | rw | 0x1 | Mask bit for SRAM uncorrectable error interrupt. |
| 0 | asf_sram_corr_err_mask | rw | 0x1 | Mask bit for SRAM correctable error interrupt. |

#### ASF_INT_TEST (0x20c)

The ASF interrupt test register emulate hardware even. Write one to individual bit to trigger single event in (masked and raw) status registers according to mask and will generate interrupt accordingly.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err_test | w | 0x0 | Test bit for integrity error interrupt |
| 5 | asf_protocol_err_test | w | 0x0 | Test bit for protocol error interrupt. |
| 4 | asf_trans_to_err_test | w | 0x0 | Test bit for transaction timeouts error interrupt. |
| 3 | asf_csr_err_test | w | 0x0 | Test bit for configuration and status registers error interrupt. |
| 2 | asf_dap_err_test | w | 0x0 | Test bit for data and address paths parity error interrupt. |
| 1 | asf_sram_uncorr_err_test | w | 0x0 | Test bit for SRAM uncorrectable error interrupt. |
| 0 | asf_sram_corr_err_test | w | 0x0 | Test bit for SRAM correctable error interrupt. |

#### ASF_FATAL_NONFATAL_SELECT (0x210)

The fatal or non-fatal interrupt register selects whether a fatal (asf_int_fatal) or non-fatal (asf_int_nonfatal) interrupt is triggered. If the bit of the event will be set to one then fatal interrupt (asf_int_fatal) will be triggered. Otherwise the non-fatal interrupt (asf_int_nonfatal) will be triggered.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err | rw | 0x1 | Enable integrity error interrupt as fatal |
| 5 | asf_protocol_err | rw | 0x1 | Enable protocol error interrupt as fatal. |
| 4 | asf_trans_to_err | rw | 0x1 | Enable transaction timeouts error interrupt as fatal. |
| 3 | asf_csr_err | rw | 0x1 | Enable configuration and status registers error interrupt as fatal. |
| 2 | asf_dap_err | rw | 0x1 | Enable data and address paths parity error interrupt as fatal. |
| 1 | asf_sram_uncorr_err | rw | 0x1 | Enable SRAM uncorrectable error interrupt as fatal. |
| 0 | asf_sram_corr_err | rw | 0x1 | Enable SRAM correctable error interrupt as fatal. |

#### ASF_SRAM_CORR_FAULT_STATUS (0x220)

Status register for SRAM correctable fault. These fields are updated whenever asf_sram_corr_fault input is active.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | asf_sram_corr_fault_inst | r | 0x0 | Last SRAM instance that generated fault. |
| 23:0 | asf_sram_corr_fault_addr | r | 0x0 | Last SRAM address that generated fault. |

#### ASF_SRAM_UNCORR_FAULT_STATUS (0x224)

Status register for SRAM uncorrectable fault. These fields are updated whenever asf_sram_uncorr_fault input is active.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | asf_sram_uncorr_fault_inst | r | 0x0 | Last SRAM instance that generated fault. |
| 23:0 | asf_sram_uncorr_fault_addr | r | 0x0 | Last SRAM address that generated fault. |

#### ASF_SRAM_FAULT_STATS (0x228)

Statistics register for SRAM faults. Note that this register clears when software writes to any field.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | reserved_31_16 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 15:0 | asf_sram_fault_corr_stats | rw1tc | 0x0 | Count of number of correctable errors if implemented. Count value will saturate at 0xffff. |

#### ASF_TRANS_TO_CTRL (0x230)

Control register to configure the ASF transaction timeout monitors.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | asf_trans_to_en | rw | 0x0 | Enable transaction timeout monitoring. |
| 30:16 | reserved | r | 0x0 | Reserved |
| 15:0 | asf_trans_to_ctrl | rw | 0x0 | Timer value to use for transaction timeout monitor. |

#### ASF_TRANS_TO_FAULT_MASK (0x234)

Control register to mask out ASF transaction timeout faults from triggering interrupts. On reset, all bits are set to mask out all sources. Clear the corresponding bit to enable the interrupt source. The width of this field is parameterisable and the bit definitions are implementation specific.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | reserved | r | 0x0 | Reserved |
| 4 | asf_trans_to_fault_4_mask | rw | 0x1 | Mask register for each ASF transaction timeout fault source. |
| 3 | asf_trans_to_fault_3_mask | rw | 0x1 | Mask register for each ASF transaction timeout fault source. |
| 2 | asf_trans_to_fault_2_mask | rw | 0x1 | Mask register for each ASF transaction timeout fault source. |
| 1 | asf_trans_to_fault_1_mask | rw | 0x1 | Mask register for each ASF transaction timeout fault source. |
| 0 | asf_trans_to_fault_0_mask | rw | 0x1 | Mask register for each ASF transaction timeout fault source. |

#### ASF_TRANS_TO_FAULT_STATUS (0x238)

Status register for transaction timeouts fault. If a fault occurs the revelant status bit will be set to 1. Each bit can be cleared by software writing 1 to each bit.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | reserved | r | 0x0 | Reserved |
| 4 | asf_trans_to_fault_4_status | rw1tc | 0x0 | Status bits for transaction timeouts faults. |
| 3 | asf_trans_to_fault_3_status | rw1tc | 0x0 | Status bits for transaction timeouts faults. |
| 2 | asf_trans_to_fault_2_status | rw1tc | 0x0 | Status bits for transaction timeouts faults. |
| 1 | asf_trans_to_fault_1_status | rw1tc | 0x0 | Status bits for transaction timeouts faults. |
| 0 | asf_trans_to_fault_0_status | rw1tc | 0x0 | Status bits for transaction timeouts faults. |

#### ASF_PROTOCOL_FAULT_MASK (0x240)

Control register to mask out ASF Protocol faults from triggering interrupts. On reset, all bits are set to mask out all sources. Clear the corresponding bit to enable the interrupt source. The width of this field is parameterisable and the bit definitions are implementation specific.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:17 | reserved | r | 0x0 | Reserved |
| 16 | asf_protocol_fault_16_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 15 | asf_protocol_fault_15_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 14 | asf_protocol_fault_14_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 13 | asf_protocol_fault_13_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 12 | asf_protocol_fault_12_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 11 | asf_protocol_fault_11_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 10 | asf_protocol_fault_10_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 9 | asf_protocol_fault_9_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 8 | asf_protocol_fault_8_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 7 | asf_protocol_fault_7_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 6 | asf_protocol_fault_6_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 5 | asf_protocol_fault_5_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 4 | asf_protocol_fault_4_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 3 | asf_protocol_fault_3_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 2 | asf_protocol_fault_2_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 1 | asf_protocol_fault_1_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 0 | asf_protocol_fault_0_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |

#### ASF_PROTOCOL_FAULT_STATUS (0x244)

Status register for protocol faults. If a fault occurs the revelant status bit will be set to 1. Each bit can be cleared by software writing 1 to each bit

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:17 | reserved | r | 0x0 | Reserved |
| 16 | asf_protocol_fault_16_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 15 | asf_protocol_fault_15_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 14 | asf_protocol_fault_14_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 13 | asf_protocol_fault_13_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 12 | asf_protocol_fault_12_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 11 | asf_protocol_fault_11_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 10 | asf_protocol_fault_10_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 9 | asf_protocol_fault_9_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 8 | asf_protocol_fault_8_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 7 | asf_protocol_fault_7_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 6 | asf_protocol_fault_6_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 5 | asf_protocol_fault_5_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 4 | asf_protocol_fault_4_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 3 | asf_protocol_fault_3_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 2 | asf_protocol_fault_2_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 1 | asf_protocol_fault_1_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 0 | asf_protocol_fault_0_status | rw1tc | 0x0 | Status bits for protocol faults. |

#### ID_PROD_VER (0xffc)

CSI2 Transmitter Product ID and Version Register.

This register is hard-coded in order to allow software to identify

the product and its release version.  The product ID will be fixed

for all versions, while the version will be updated as new releases

for the same product are made.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | product_id | r | 0x5023 | CSI2 Transmitter Product Identification Number. |
| 15:0 | version_id | r | 0x200 | CSI2 Transmitter Product Version Number. |

## MMR Region: cp_intd__intd_cfg__intd_cfg

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | REVISION |  |
| 0x4 | CONTROL |  |
| 0x10 | EOI_REG |  |
| 0x14 | INTR_VECTOR_REG |  |
| 0x100 | ENABLE_REG_LEVEL_0 |  |
| 0x104 | ENABLE_REG_PULSE_0 |  |
| 0x300 | ENABLE_CLR_REG_LEVEL_0 |  |
| 0x304 | ENABLE_CLR_REG_PULSE_0 |  |
| 0x500 | STATUS_REG_LEVEL_0 |  |
| 0x504 | STATUS_REG_PULSE_0 |  |
| 0x700 | STATUS_CLR_REG_LEVEL_0 |  |
| 0x704 | STATUS_CLR_REG_PULSE_0 |  |
| 0xa80 | INTR_VECTOR_REG_LEVEL |  |
| 0xa84 | INTR_VECTOR_REG_PULSE |  |

### Register Definitions

#### REVISION (0x0)

Revision Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:30 | SCHEME | r | 0x1 | Scheme |
| 29:28 | BU | r | 0x2 | BU |
| 27:16 | FUNCTION | r | 0x690 | Module ID |
| 15:11 | RTLVER | r | 0x12 | RTL revisions |
| 10:8 | MAJREV | r | 0x2 | Major revision |
| 7:6 | CUSTOM | r | 0x0 | Custom revision |
| 5:0 | MINREV | r | 0x0 | Minor revision |

#### CONTROL (0x4)

control Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | reserved | r | 0x0 | Reserved |

#### EOI_REG (0x10)

End of Interrupt Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:8 | reserved | r | 0x0 | Reserved |
| 7:0 | eoi_vector | rw | 0x0 | End of Interrupt Vector |

#### INTR_VECTOR_REG (0x14)

Interrupt Vector Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | intr_vector | r | 0x0 | Interrupt Vector Register |

#### ENABLE_REG_LEVEL_0 (0x100)

Enable Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | enable_level_en_temp_int1 | rw1ts | 0x0 | Enable Set for level_en_temp_int1 |
| 0 | enable_level_en_temp_int0 | rw1ts | 0x0 | Enable Set for level_en_temp_int0 |

#### ENABLE_REG_PULSE_0 (0x104)

Enable Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | enable_pulse_en_temp_int1 | rw1ts | 0x0 | Enable Set for pulse_en_temp_int1 |
| 0 | enable_pulse_en_temp_int0 | rw1ts | 0x0 | Enable Set for pulse_en_temp_int0 |

#### ENABLE_CLR_REG_LEVEL_0 (0x300)

Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | enable_level_en_temp_int1_clr | rw1tc | 0x0 | Enable Clear for level_en_temp_int1 |
| 0 | enable_level_en_temp_int0_clr | rw1tc | 0x0 | Enable Clear for level_en_temp_int0 |

#### ENABLE_CLR_REG_PULSE_0 (0x304)

Enable Clear Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | enable_pulse_en_temp_int1_clr | rw1tc | 0x0 | Enable Clear for pulse_en_temp_int1 |
| 0 | enable_pulse_en_temp_int0_clr | rw1tc | 0x0 | Enable Clear for pulse_en_temp_int0 |

#### STATUS_REG_LEVEL_0 (0x500)

Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | status_level_temp_int1 | rw1ts | 0x0 | Status ,write 1 to set, for level_en_temp_int1 |
| 0 | status_level_temp_int0 | rw1ts | 0x0 | Status ,write 1 to set, for level_en_temp_int0 |

#### STATUS_REG_PULSE_0 (0x504)

Status Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | status_pulse_temp_int1 | rw1ts | 0x0 | Status ,write 1 to set, for pulse_en_temp_int1 |
| 0 | status_pulse_temp_int0 | rw1ts | 0x0 | Status ,write 1 to set, for pulse_en_temp_int0 |

#### STATUS_CLR_REG_LEVEL_0 (0x700)

Status Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | status_level_temp_int1_clr | rw1tc | 0x0 | Status ,write 1 to clear, for level_en_temp_int1 |
| 0 | status_level_temp_int0_clr | rw1tc | 0x0 | Status ,write 1 to clear, for level_en_temp_int0 |

#### STATUS_CLR_REG_PULSE_0 (0x704)

Status Clear Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | status_pulse_temp_int1_clr | rw1tc | 0x0 | Status ,write 1 to clear, for pulse_en_temp_int1 |
| 0 | status_pulse_temp_int0_clr | rw1tc | 0x0 | Status ,write 1 to clear, for pulse_en_temp_int0 |

#### INTR_VECTOR_REG_LEVEL (0xa80)

Interrupt Vector for level

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | intr_vector_level | r | 0x0 | Interrupt Vector |

#### INTR_VECTOR_REG_PULSE (0xa84)

Interrupt Vector for pulse

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | intr_vector_pulse | r | 0x0 | Interrupt Vector |

## MMR Region: ecc_aggr__cfg__regs

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | AGGR_REVISION | Aggregator Revision Register |
| 0x8 | ECC_VECTOR | ECC Vector Register |
| 0xc | MISC_STATUS | Misc Status |
| 0x3c | SEC_EOI_REG | EOI Register |
| 0x40 | SEC_STATUS_REG0 | Interrupt Status Register 0 |
| 0x80 | SEC_ENABLE_SET_REG0 | Interrupt Enable Set Register 0 |
| 0xc0 | SEC_ENABLE_CLR_REG0 | Interrupt Enable Clear Register 0 |
| 0x13c | DED_EOI_REG | EOI Register |
| 0x140 | DED_STATUS_REG0 | Interrupt Status Register 0 |
| 0x180 | DED_ENABLE_SET_REG0 | Interrupt Enable Set Register 0 |
| 0x1c0 | DED_ENABLE_CLR_REG0 | Interrupt Enable Clear Register 0 |
| 0x200 | AGGR_ENABLE_SET | AGGR interrupt enable set Register |
| 0x204 | AGGR_ENABLE_CLR | AGGR interrupt enable clear Register |
| 0x208 | AGGR_STATUS_SET | AGGR interrupt status set Register |
| 0x20c | AGGR_STATUS_CLR | AGGR interrupt status clear Register |

### Register Definitions

#### AGGR_REVISION (Aggregator Revision Register), (0x0)

Revision parameters

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:30 | scheme | r | 0x1 | Scheme |
| 29:28 | bu | r | 0x2 | bu |
| 27:16 | module_id | r | 0x6a0 | Module ID |
| 15:11 | revrtl | r | 0x1c | RTL version |
| 10:8 | revmaj | r | 0x2 | Major version |
| 7:6 | custom | r | 0x0 | Custom version |
| 5:0 | revmin | r | 0x0 | Minor version |

#### ECC_VECTOR (ECC Vector Register), (0x8)

ECC Vector Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:25 | reserved | r | 0x0 | Reserved |
| 24 | rd_svbus_done | rw1tc | 0x0 | Status to indicate if read on serial VBUS is complete, write of any value will clear this bit. |
| 23:16 | rd_svbus_address | rw | 0x0 | Read address |
| 15 | rd_svbus | rw1ts | 0x0 | Write 1 to trigger a read on the serial VBUS |
| 14:11 | reserved | r | 0x0 | Reserved |
| 10:0 | ecc_vector | rw | 0x0 | Value written to select the corresponding ECC RAM for control or status |

#### MISC_STATUS (Misc Status), (0xc)

Misc Status

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:11 | reserved | r | 0x0 | Reserved |
| 10:0 | num_rams | r | 0x7 | Indicates the number of RAMS serviced by the ECC aggregator |

#### SEC_EOI_REG (EOI Register), (0x3c)

EOI Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | eoi_wr | rw1ts | 0x0 | EOI Register |

#### SEC_STATUS_REG0 (Interrupt Status Register 0), (0x40)

Interrupt Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | fifo_ramecc_pend | rw1ts | 0x0 | Interrupt Pending Status for fifo_ramecc_pend |
| 0 | busecc_pend | rw1ts | 0x0 | Interrupt Pending Status for busecc_pend |

#### SEC_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x80)

Interrupt Enable Set Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | fifo_ramecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for fifo_ramecc_pend |
| 0 | busecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for busecc_pend |

#### SEC_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0xc0)

Interrupt Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | fifo_ramecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for fifo_ramecc_pend |
| 0 | busecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for busecc_pend |

#### DED_EOI_REG (EOI Register), (0x13c)

EOI Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | eoi_wr | rw1ts | 0x0 | EOI Register |

#### DED_STATUS_REG0 (Interrupt Status Register 0), (0x140)

Interrupt Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | fifo_ramecc_pend | rw1ts | 0x0 | Interrupt Pending Status for fifo_ramecc_pend |
| 0 | busecc_pend | rw1ts | 0x0 | Interrupt Pending Status for busecc_pend |

#### DED_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x180)

Interrupt Enable Set Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | fifo_ramecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for fifo_ramecc_pend |
| 0 | busecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for busecc_pend |

#### DED_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x1c0)

Interrupt Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | fifo_ramecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for fifo_ramecc_pend |
| 0 | busecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for busecc_pend |

#### AGGR_ENABLE_SET (AGGR interrupt enable set Register), (0x200)

AGGR interrupt enable set Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | timeout | rw1ts | 0x0 | interrupt enable set for svbus timeout errors |
| 0 | parity | rw1ts | 0x0 | interrupt enable set for parity errors |

#### AGGR_ENABLE_CLR (AGGR interrupt enable clear Register), (0x204)

AGGR interrupt enable clear Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | timeout | rw1tc | 0x0 | interrupt enable clear for svbus timeout errors |
| 0 | parity | rw1tc | 0x0 | interrupt enable clear for parity errors |

#### AGGR_STATUS_SET (AGGR interrupt status set Register), (0x208)

AGGR interrupt status set Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3:2 | timeout | rwi | 0x0 | interrupt status set for svbus timeout errors |
| 1:0 | parity | rwi | 0x0 | interrupt status set for parity errors |

#### AGGR_STATUS_CLR (AGGR interrupt status clear Register), (0x20c)

AGGR interrupt status clear Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3:2 | timeout | rwd | 0x0 | interrupt status clear for svbus timeout errors |
| 1:0 | parity | rwd | 0x0 | interrupt status clear for parity errors |

## MMR Region: ecc_aggr_byte__cfg__regs

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x400 | AGGR_REVISION | Aggregator Revision Register |
| 0x408 | ECC_VECTOR | ECC Vector Register |
| 0x40c | MISC_STATUS | Misc Status |
| 0x410+(a*0x4), a=0..7 | RESERVED_SVBUS[a] | Reserved Area for Serial VBUS Registers |
| 0x43c | SEC_EOI_REG | EOI Register |
| 0x440 | SEC_STATUS_REG0 | Interrupt Status Register 0 |
| 0x480 | SEC_ENABLE_SET_REG0 | Interrupt Enable Set Register 0 |
| 0x4c0 | SEC_ENABLE_CLR_REG0 | Interrupt Enable Clear Register 0 |
| 0x53c | DED_EOI_REG | EOI Register |
| 0x540 | DED_STATUS_REG0 | Interrupt Status Register 0 |
| 0x580 | DED_ENABLE_SET_REG0 | Interrupt Enable Set Register 0 |
| 0x5c0 | DED_ENABLE_CLR_REG0 | Interrupt Enable Clear Register 0 |
| 0x600 | AGGR_ENABLE_SET | AGGR interrupt enable set Register |
| 0x604 | AGGR_ENABLE_CLR | AGGR interrupt enable clear Register |
| 0x608 | AGGR_STATUS_SET | AGGR interrupt status set Register |
| 0x60c | AGGR_STATUS_CLR | AGGR interrupt status clear Register |

### Register Definitions

#### AGGR_REVISION (Aggregator Revision Register), (0x400)

Revision parameters

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:30 | scheme | r | 0x1 | Scheme |
| 29:28 | bu | r | 0x2 | bu |
| 27:16 | module_id | r | 0x6a0 | Module ID |
| 15:11 | revrtl | r | 0x1d | RTL version |
| 10:8 | revmaj | r | 0x2 | Major version |
| 7:6 | custom | r | 0x0 | Custom version |
| 5:0 | revmin | r | 0x0 | Minor version |

#### ECC_VECTOR (ECC Vector Register), (0x408)

ECC Vector Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:25 | reserved | r | 0x0 | Reserved |
| 24 | rd_svbus_done | rw1tc | 0x0 | Status to indicate if read on serial VBUS is complete, write of any value will clear this bit. |
| 23:16 | rd_svbus_address | rw | 0x0 | Read address |
| 15 | rd_svbus | rw1ts | 0x0 | Write 1 to trigger a read on the serial VBUS |
| 14:11 | reserved | r | 0x0 | Reserved |
| 10:0 | ecc_vector | rw | 0x0 | Value written to select the corresponding ECC RAM for control or status |

#### MISC_STATUS (Misc Status), (0x40c)

Misc Status

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:11 | reserved | r | 0x0 | Reserved |
| 10:0 | num_rams | r | 0x4 | Indicates the number of RAMS serviced by the ECC aggregator |

#### RESERVED_SVBUS[a] (Reserved Area for Serial VBUS Registers), (0x410+(a*0x4)), a=0..7

Reference other documents that contain the ECC RAM wrapper and EDC controller serial vbus register sets.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | data | rw | 0x0 | Serial VBUS register data |

#### SEC_EOI_REG (EOI Register), (0x43c)

EOI Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | eoi_wr | rw1ts | 0x0 | EOI Register |

#### SEC_STATUS_REG0 (Interrupt Status Register 0), (0x440)

Interrupt Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3 | ram_ramecc3_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc3_pend |
| 2 | ram_ramecc2_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc2_pend |
| 1 | ram_ramecc1_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc1_pend |
| 0 | ram_ramecc0_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc0_pend |

#### SEC_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x480)

Interrupt Enable Set Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3 | ram_ramecc3_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc3_pend |
| 2 | ram_ramecc2_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc2_pend |
| 1 | ram_ramecc1_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc1_pend |
| 0 | ram_ramecc0_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc0_pend |

#### SEC_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x4c0)

Interrupt Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3 | ram_ramecc3_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc3_pend |
| 2 | ram_ramecc2_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc2_pend |
| 1 | ram_ramecc1_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc1_pend |
| 0 | ram_ramecc0_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc0_pend |

#### DED_EOI_REG (EOI Register), (0x53c)

EOI Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | eoi_wr | rw1ts | 0x0 | EOI Register |

#### DED_STATUS_REG0 (Interrupt Status Register 0), (0x540)

Interrupt Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3 | ram_ramecc3_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc3_pend |
| 2 | ram_ramecc2_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc2_pend |
| 1 | ram_ramecc1_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc1_pend |
| 0 | ram_ramecc0_pend | rw1ts | 0x0 | Interrupt Pending Status for ram_ramecc0_pend |

#### DED_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x580)

Interrupt Enable Set Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3 | ram_ramecc3_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc3_pend |
| 2 | ram_ramecc2_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc2_pend |
| 1 | ram_ramecc1_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc1_pend |
| 0 | ram_ramecc0_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for ram_ramecc0_pend |

#### DED_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x5c0)

Interrupt Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3 | ram_ramecc3_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc3_pend |
| 2 | ram_ramecc2_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc2_pend |
| 1 | ram_ramecc1_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc1_pend |
| 0 | ram_ramecc0_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for ram_ramecc0_pend |

#### AGGR_ENABLE_SET (AGGR interrupt enable set Register), (0x600)

AGGR interrupt enable set Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | timeout | rw1ts | 0x0 | interrupt enable set for svbus timeout errors |
| 0 | parity | rw1ts | 0x0 | interrupt enable set for parity errors |

#### AGGR_ENABLE_CLR (AGGR interrupt enable clear Register), (0x604)

AGGR interrupt enable clear Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | timeout | rw1tc | 0x0 | interrupt enable clear for svbus timeout errors |
| 0 | parity | rw1tc | 0x0 | interrupt enable clear for parity errors |

#### AGGR_STATUS_SET (AGGR interrupt status set Register), (0x608)

AGGR interrupt status set Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3:2 | timeout | rwi | 0x0 | interrupt status set for svbus timeout errors |
| 1:0 | parity | rwi | 0x0 | interrupt status set for parity errors |

#### AGGR_STATUS_CLR (AGGR interrupt status clear Register), (0x60c)

AGGR interrupt status clear Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3:2 | timeout | rwd | 0x0 | interrupt status clear for svbus timeout errors |
| 1:0 | parity | rwd | 0x0 | interrupt status clear for parity errors |

SECTION II: INTEGRATION DETAILS

# Submodule information

# Integration Overview

## Parameterization Requirements

N/A  No parameterization

## Memories Embedded in the Peripheral

| CSI_TX_IF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Qty | RAM | RAM Name | GS80 Area (mm2) | GS80 Total (mm2) |
| DMA | 2 |  | saculs0c4s2p2048x64m4b2w0c0p1d1r1s2z1rw10. |  |  |
|  | 5 |  | sadrls0c4s2p1024x44m4b8w0c0p1d1r1s2z1rw10 |  |  |
|  | 4 |  | sadrls0c4l2p256x64m4b1w1c0p1d1r1s2z1rw10 |  |  |
|  | 1 |  | saculs0c4s2p2048x24m4b2w0c0p1d1r1s2z1rw10. |  |  |
|  | 1 |  | sadrls0c4l2p256x16m4b1w1c0p1d1r1s2z1rw10 |  |  |
| Total |  |  |  |  |  |

Table 4 : RAM

*rams associated with cadence ip have parity. All other rams have full ecc using ti ecc wrappers.

### Error Correcting Code (ECC) Support

ECC is a mechanism for providing increased system reliability (via reduction of memory soft errors) by allowing single bit errors to be detected and corrected and double bit errors to be detected.

One or more memories within this subsystem are ECC protected using an ECC Memory Wrapper which implements Single Error Correction and Double Error Detection (SECDED). Refer to the “ECC Wrapper Functional Specification” for complete details on the features and functions of the ECC Memory Wrapper. Refer to section 1.1.2 in this document for specific information on the ECC protected memories within this subsystem.

The ECC wrapper provides Single Error Detection and Correction (SED/SEC). This logic detects and corrects a single bit error (1 bit error per ECC word or per ECC data segment). For memories that contain critical and/or persistent data, automatic (immediate or delayed) write-back of the corrected data to the corresponding memory address is supported. In addition, the ECC wrapper also supports multiple options for partial word writes, such as read-modify-write or multiple ECC code segments per word.

The ECC wrapper also provides Double Error Detection (DED). This logic only detects (does not correct) double errors (2 bit errors per ECC word or per ECC data segment).

An ECC Aggregator at the IP subsystem level consolidates the ECC configuration and status bits for all the ECC supported memories in the subsystem. It provides a single EOI-handshake based interrupt to the host (for both single and double error detections) and a standard 32-bit VBUSP interface for configuring and querying the various ECC wrappers via their ECC register set. Refer to the “ECC Aggregator Functional Specification” for complete details on the features and functions of the ECC Aggregator.

### ECC Memories

The following table lists the memories within this subsystem that are ECC protected, along with ECC attributes for each memory.

Table 5 : ECC Memory Table

| RAM Wrapper | Type | ECC width (bits) | Fast_Q mode | Write-back (Immediate, Delayed, or None) | Partial- word writes |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | rmw | Data splitting |
| csi_tx_if_tx_shim_dma_psil_fifo_tpram_2048x152_sww_sr | tpram | 9 | N | None | N | N |
| csi_tx_if_tx_shim_ksdma_psil_endpt_ipcfifo_f0_tpram_256x167_sbw_sr_tpram_256x272_sbw_sr | tpram | 5 | N | None | N | Y |

## PLL Requirements

There are no special PLL requirements for this module.   All clocks generation, gating, distribution is performs by parent modules.

## Overall Design For Test

## Target Processes

The initial target process is 16nm finFET.  While developing RTL, 28nm RAM will be used to DV.

## Area Requirements

.8mm^2 in 16ffc

## Power Requirements

## Performance Requirements

#### MAIN CLK

Most logic runs off main clock. It needs to be same frequency as cs_rx_if main clock. 500MHz 16nm.

When CLK is operating lower than 312.5MHz, then the clock is essentially limiting the clock rate of the DPHY.  Said another way, CLK must be at least the Byte_clk rate else FIFOs will overflow, crashing the module.

#### VBUSP_CLK

The VBUSP_CLK is the same domain as main_clk.  This free clock runs at 250MHz or /2 of main clock in 16ff.

#### Byte Clock

The Byte clock is the clock supplied by the DPHY.  DPHY is spec’d for max of 10gbps.  This translates to a max byte clock of 312.5MHz.

#### ESC Clock

Escape clock runs at 20MHz.  Esc interface is hardly used by CSI, only to clock in some low speed control signals from DPHY.  The ESC interface is a low speed DPHY comm link to the camera/sensor.

# I/O Description

## Module I/O

The following table provides a summary of the interfaces on this module:

| Interface Name | Description | Clock | Reset |
| --- | --- | --- | --- |
| main_rstn | synchronous Reset Interface For csi | main_clk_clk |  |
| main_clk | MAIN_CLK free running clock for csi_tx_if |  |  |
| vbus_clk | MAIN_CLK free running clock for csi_tx_if |  |  |
| esc_clk | ESC_CLK low speed clock for LP low power mode domm |  |  |
| pwr_stop | Clock Power Stop | async | async |
| pwr_clken | Power Clock Enable for csi | async | async |
| dft_esc_local | DFT_CLK clock for escape Clock Domain during Text... muxed in for DFT |  |  |
| dft_byte_local | DFT_CLK clock for dphy_tx_byte_clk Clock Domain during Text... muxed in for DFT |  |  |
| dft | DFT Interface |  |  |
| csi_pulse | Interrupt servicing all CSI TI generated RTL aggregated by INTD | vbus_clk_clk | main_rstn_mod_g_rst_n |
| csi_level | Interrupt servicing all CSI TI generated RTL aggregated by INTD | vbus_clk_clk | main_rstn_mod_g_rst_n |
| csi_interrupt | CSI interupt from CSI2TX submodule | main_clk_clk | main_rstn_mod_g_rst_n |
| csi_fatal | CSI fatal interrupt from CSI2TX submodule | main_clk_clk | main_rstn_mod_g_rst_n |
| csi_nonfatal | CSI non fatal interrupt from CSI2TX submodule | main_clk_clk | main_rstn_mod_g_rst_n |
| vbusp | Vbusp configuration port | vbus_clk_clk | main_rstn_mod_g_rst_n |
| retran | RETRAN retransmission stream interface from csi_rx_if to csi_tx_if | main_clk_clk | main_rstn_mod_g_rst_n |
| dphy | DPHY PPI interface between csi_tx_if and DPHY | main_clk_clk | main_rstn_mod_g_rst_n |
| strm | PSI-L Streaming Interface | main_clk_clk | main_rstn_mod_g_rst_n |
| fdram_ramdft |  | main_clk_clk | main_rstn_mod_g_rst_n |
| fifo_ramdft | mem gpio interface | main_clk_clk |  |
| uncorr_pulse | CPU 2 uncorrectable error, pulse | main_clk_clk | main_rstn_mod_g_rst_n |
| uncorr_level | CPU 2 uncorrectable error, level | main_clk_clk | main_rstn_mod_g_rst_n |

The following sections provide pin level detail for each of the interfaces on this module:

### synchronous Reset Interface For csi

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| main_rstn_mod_g_rst_n | In | 1'd1 | Module level main reset |

### MAIN_CLK free running clock for csi_tx_if

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| main_clk_clk | In | 1'd0 | Single ended clock |
| main_clk_cs_func_clk_en | In | 1'd0 | catscan enable on kp_ti_ipg_clk_l1 |

### MAIN_CLK free running clock for csi_tx_if

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| vbus_clk_clk | In | 1'd0 | Single ended clock |

### ESC_CLK low speed clock for LP low power mode domm

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| esc_clk_clk | In | 1'd0 | Single ended clock |

### Clock Power Stop

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_clkstop_req | In | 1'd0 | Clock stop request |
| csi_clkstop_ack | Out | pwr_stop_clkstop_req | Clock stop acknowledge |

### Power Clock Enable for csi

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_clk_en | In | 1'd1 | Clock-gate enable to IP |
| csi_clk_en_ack | Out | pwr_clken_clk_en | Clock-gate enable acknowledge from IP |

### DFT_CLK clock for escape Clock Domain during Text... muxed in for DFT

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| dft_esc_local_clk | In | 1'd0 | Single ended clock |

### DFT_CLK clock for dphy_tx_byte_clk Clock Domain during Text... muxed in for DFT

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| dft_byte_local_clk | In | 1'd0 | Single ended clock |

### DFT Interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| dft_clk_force_en | In | 1'd0 | dft_clk_force_en |
| dft_partition_en | In | {1{1'b1}} | dft_partition_en |
| dft_scan_en | In | 1'd0 | dft_scan_en |
| dft_async_rst_sel | In | 1'd0 | dft_async_rst_sel |
| dft_test_async_rst_n | In | 1'd1 | dft_test_async_rst_n |
| dft_tft_mcp_dis | In | 1'd0 | Assert High to block MCP path |
| dft_local_clk_en | In | 1'd0 | dft_local_clk_en |

### DFT Interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| dft_phy_l1_clk_en | In | 1'd0 | DFT local clk enable contribution to kp_ti_ipg_clk_phy_l1 |
| dft_tft_phy_icg_clk_en | In | 1'd1 | DFT local clk gate enable contribution to kp_ti_ipg_clk_phy_l1 |

### Interrupt servicing all CSI TI generated RTL aggregated by INTD

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_pulse_intr | Out | 1'd0 | Interrupt signal |

### Interrupt servicing all CSI TI generated RTL aggregated by INTD

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_level_intr | Out | 1'd0 | Interrupt signal |

### CSI interupt from CSI2TX submodule

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_interrupt_intr | Out | 1'd0 | Interrupt signal |

### CSI fatal interrupt from CSI2TX submodule

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_fatal_intr | Out | 1'd0 | Interrupt signal |

### CSI non fatal interrupt from CSI2TX submodule

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_nonfatal_intr | Out | 1'd0 | Interrupt signal |

### Vbusp configuration port

This is the slave MMR configuration interface with 4Byte addressable range

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| vbusp_req | In | 1'b0 | Request |
| vbusp_rsel[3:0] | In | 4'd0 | Region Select |
| vbusp_dir | In | 1'b0 | Direction |
| vbusp_address[13:0] | In | 14'd0 | Address |
| vbusp_xcnt[2:0] | In | 3'd4 | Good Byte Count |
| vbusp_byten[3:0] | In | {4{1'b1}} | Byte Enables |
| vbusp_wdata[31:0] | In | 32'd0 | Write Data |
| vbusp_wready | Out | 1'b1 | Write Ready |
| vbusp_rdatap[31:0] | Out | 32'd0 | Read Data (Pipelined) |
| vbusp_rready | Out | 1'b1 | Read Ready |

### RETRAN retransmission stream interface from csi_rx_if to csi_tx_if

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| retran_pixel_ready_if | Out | 1'd0 | CSI retransmit Data enable |
| retran_vsync_valid_if[15:0] | In | 16'd0 | CSI retransmit Vertical sync |
| retran_hsync_valid_if | In | 1'd0 | CSI retransmit Horizontal sync |
| retran_pixel_da_vld_if[3:0] | In | 4'd0 | CSI retransmit Data enable |
| retran_pixel_byte_vld_if[3:0] | In | 4'd0 | CSI retransmit Byte enable |
| retran_pixel_data_if[31:0] | In | 32'd0 | CSI retransmit Pixel data |
| retran_virtual_channel_if[3:0] | In | 4'd0 | CSI retransmit Virtual Channel |
| retran_pixel_datatype_if[5:0] | In | 6'd0 | CSI retransmit Data Type |
| retran_pixel_wordcount_if[15:0] | In | 16'd0 | CSI retransmit Word Count |
| retran_packet_error_if | In | 1'd0 | CSI retransmit Packet Error |
| retran_stream_idle_if | In | 1'd0 | CSI retransmit Stream Idle |

### DPHY PPI interface between csi_tx_if and DPHY

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| dphy_TxRequestHSClk_cl_l | Out | 1'd0 |  |
| dphy_TxReadyHSClk_cl_l | In | 1'd0 |  |
| dphy_TxULPSClk_cl_l | Out | 1'd0 |  |
| dphy_TxULPSExitClk_cl_l | Out | 1'd0 |  |
| dphy_TxEnableClk_cl_l | Out | 1'd0 |  |
| dphy_M_Clk_SwapDpDn_cl_l | Out | 1'd0 |  |
| dphy_TxULPSActiveNotClk_cl_l | In | 1'd0 |  |
| dphy_TxStopStateClk_cl_l | In | 1'd0 |  |
| dphy_lane_rstb_cl_l | Out | 1'd0 |  |
| dphy_TxRequestHS_dl_l_0 | Out | 1'd0 |  |
| dphy_TxDataHS_dl_l_0[7:0] | Out | 8'd0 |  |
| dphy_TxReadyHS_dl_l_0 | In | 1'd0 |  |
| dphy_TxSkewCalHS_dl_l_0 | Out | 1'd0 |  |
| dphy_M_TxRequestEsc_dl_l_0 | Out | 1'd0 |  |
| dphy_M_TxLPDTEsc_dl_l_0 | Out | 1'd0 |  |
| dphy_M_TxDataEsc_dl_l_0[7:0] | Out | 8'd0 |  |
| dphy_M_TxValidEsc_dl_l_0 | Out | 1'd0 |  |
| dphy_M_TxULPSEsc_dl_l_0 | Out | 1'd0 |  |
| dphy_M_TxULPSExitEsc_dl_l_0 | Out | 1'd0 |  |
| dphy_ForceTxStopMode_dl_l_0 | Out | 1'd0 |  |
| dphy_M_TxTriggerEsc_dl_l_0[3:0] | Out | 4'd0 |  |
| dphy_M_TxReadyEsc_dl_l_0 | In | 1'd0 |  |
| dphy_M_RxLPDTEsc_dl_l_0 | In | 1'd0 |  |
| dphy_M_RxDataEsc_dl_l_0[7:0] | In | 8'd0 |  |
| dphy_M_RxValidEsc_dl_l_0 | In | 1'd0 |  |
| dphy_M_RxULPSEsc_dl_l_0 | In | 1'd0 |  |
| dphy_M_RxTriggerEsc_dl_l_0[3:0] | In | 4'd0 |  |
| dphy_M_Enable_dl_l_0 | Out | 1'd0 |  |
| dphy_M_TurnRequest_dl_l_0 | Out | 1'd0 |  |
| dphy_M_Data_SwapDpDn_dl_l_0 | Out | 1'd0 |  |
| dphy_M_Direction_dl_l_0 | In | 1'd0 |  |
| dphy_M_ULPSActiveNot_dl_l_0 | In | 1'd0 |  |
| dphy_M_StopState_dl_l_0 | In | 1'd0 |  |
| dphy_M_ErrControl_dl_l_0 | In | 1'd0 |  |
| dphy_M_ErrEsc_dl_l_0 | In | 1'd0 |  |
| dphy_M_ErrSyncEsc_dl_l_0 | In | 1'd0 |  |
| dphy_M_ErrContentionLP0_dl_l_0 | In | 1'd0 |  |
| dphy_M_ErrContentionLP1_dl_l_0 | In | 1'd0 |  |
| dphy_lane_rstb_dl_l_0 | Out | 1'd0 |  |
| dphy_TxRequestHS_dl_l_1 | Out | 1'd0 |  |
| dphy_TxDataHS_dl_l_1[7:0] | Out | 8'd0 |  |
| dphy_TxReadyHS_dl_l_1 | In | 1'd0 |  |
| dphy_TxSkewCalHS_dl_l_1 | Out | 1'd0 |  |
| dphy_M_TxRequestEsc_dl_l_1 | Out | 1'd0 |  |
| dphy_M_TxLPDTEsc_dl_l_1 | Out | 1'd0 |  |
| dphy_M_TxDataEsc_dl_l_1[7:0] | Out | 8'd0 |  |
| dphy_M_TxValidEsc_dl_l_1 | Out | 1'd0 |  |
| dphy_M_TxULPSEsc_dl_l_1 | Out | 1'd0 |  |
| dphy_M_TxULPSExitEsc_dl_l_1 | Out | 1'd0 |  |
| dphy_ForceTxStopMode_dl_l_1 | Out | 1'd0 |  |
| dphy_M_TxTriggerEsc_dl_l_1[3:0] | Out | 4'd0 |  |
| dphy_M_TxReadyEsc_dl_l_1 | In | 1'd0 |  |
| dphy_M_RxLPDTEsc_dl_l_1 | In | 1'd0 |  |
| dphy_M_RxDataEsc_dl_l_1[7:0] | In | 8'd0 |  |
| dphy_M_RxValidEsc_dl_l_1 | In | 1'd0 |  |
| dphy_M_RxULPSEsc_dl_l_1 | In | 1'd0 |  |
| dphy_M_RxTriggerEsc_dl_l_1[3:0] | In | 4'd0 |  |
| dphy_M_Enable_dl_l_1 | Out | 1'd0 |  |
| dphy_M_TurnRequest_dl_l_1 | Out | 1'd0 |  |
| dphy_M_Data_SwapDpDn_dl_l_1 | Out | 1'd0 |  |
| dphy_M_Direction_dl_l_1 | In | 1'd0 |  |
| dphy_M_ULPSActiveNot_dl_l_1 | In | 1'd0 |  |
| dphy_M_StopState_dl_l_1 | In | 1'd0 |  |
| dphy_M_ErrControl_dl_l_1 | In | 1'd0 |  |
| dphy_M_ErrEsc_dl_l_1 | In | 1'd0 |  |
| dphy_M_ErrSyncEsc_dl_l_1 | In | 1'd0 |  |
| dphy_M_ErrContentionLP0_dl_l_1 | In | 1'd0 |  |
| dphy_M_ErrContentionLP1_dl_l_1 | In | 1'd0 |  |
| dphy_lane_rstb_dl_l_1 | Out | 1'd0 |  |
| dphy_TxRequestHS_dl_l_2 | Out | 1'd0 |  |
| dphy_TxDataHS_dl_l_2[7:0] | Out | 8'd0 |  |
| dphy_TxReadyHS_dl_l_2 | In | 1'd0 |  |
| dphy_TxSkewCalHS_dl_l_2 | Out | 1'd0 |  |
| dphy_M_TxRequestEsc_dl_l_2 | Out | 1'd0 |  |
| dphy_M_TxLPDTEsc_dl_l_2 | Out | 1'd0 |  |
| dphy_M_TxDataEsc_dl_l_2[7:0] | Out | 8'd0 |  |
| dphy_M_TxValidEsc_dl_l_2 | Out | 1'd0 |  |
| dphy_M_TxULPSEsc_dl_l_2 | Out | 1'd0 |  |
| dphy_M_TxULPSExitEsc_dl_l_2 | Out | 1'd0 |  |
| dphy_ForceTxStopMode_dl_l_2 | Out | 1'd0 |  |
| dphy_M_TxTriggerEsc_dl_l_2[3:0] | Out | 4'd0 |  |
| dphy_M_TxReadyEsc_dl_l_2 | In | 1'd0 |  |
| dphy_M_RxLPDTEsc_dl_l_2 | In | 1'd0 |  |
| dphy_M_RxDataEsc_dl_l_2[7:0] | In | 8'd0 |  |
| dphy_M_RxValidEsc_dl_l_2 | In | 1'd0 |  |
| dphy_M_RxULPSEsc_dl_l_2 | In | 1'd0 |  |
| dphy_M_RxTriggerEsc_dl_l_2[3:0] | In | 4'd0 |  |
| dphy_M_Enable_dl_l_2 | Out | 1'd0 |  |
| dphy_M_TurnRequest_dl_l_2 | Out | 1'd0 |  |
| dphy_M_Data_SwapDpDn_dl_l_2 | Out | 1'd0 |  |
| dphy_M_Direction_dl_l_2 | In | 1'd0 |  |
| dphy_M_ULPSActiveNot_dl_l_2 | In | 1'd0 |  |
| dphy_M_StopState_dl_l_2 | In | 1'd0 |  |
| dphy_M_ErrControl_dl_l_2 | In | 1'd0 |  |
| dphy_M_ErrEsc_dl_l_2 | In | 1'd0 |  |
| dphy_M_ErrSyncEsc_dl_l_2 | In | 1'd0 |  |
| dphy_M_ErrContentionLP0_dl_l_2 | In | 1'd0 |  |
| dphy_M_ErrContentionLP1_dl_l_2 | In | 1'd0 |  |
| dphy_lane_rstb_dl_l_2 | Out | 1'd0 |  |
| dphy_TxRequestHS_dl_l_3 | Out | 1'd0 |  |
| dphy_TxDataHS_dl_l_3[7:0] | Out | 8'd0 |  |
| dphy_TxReadyHS_dl_l_3 | In | 1'd0 |  |
| dphy_TxSkewCalHS_dl_l_3 | Out | 1'd0 |  |
| dphy_M_TxRequestEsc_dl_l_3 | Out | 1'd0 |  |
| dphy_M_TxLPDTEsc_dl_l_3 | Out | 1'd0 |  |
| dphy_M_TxDataEsc_dl_l_3[7:0] | Out | 8'd0 |  |
| dphy_M_TxValidEsc_dl_l_3 | Out | 1'd0 |  |
| dphy_M_TxULPSEsc_dl_l_3 | Out | 1'd0 |  |
| dphy_M_TxULPSExitEsc_dl_l_3 | Out | 1'd0 |  |
| dphy_ForceTxStopMode_dl_l_3 | Out | 1'd0 |  |
| dphy_M_TxTriggerEsc_dl_l_3[3:0] | Out | 4'd0 |  |
| dphy_M_TxReadyEsc_dl_l_3 | In | 1'd0 |  |
| dphy_M_RxLPDTEsc_dl_l_3 | In | 1'd0 |  |
| dphy_M_RxDataEsc_dl_l_3[7:0] | In | 8'd0 |  |
| dphy_M_RxValidEsc_dl_l_3 | In | 1'd0 |  |
| dphy_M_RxULPSEsc_dl_l_3 | In | 1'd0 |  |
| dphy_M_RxTriggerEsc_dl_l_3[3:0] | In | 4'd0 |  |
| dphy_M_Enable_dl_l_3 | Out | 1'd0 |  |
| dphy_M_TurnRequest_dl_l_3 | Out | 1'd0 |  |
| dphy_M_Data_SwapDpDn_dl_l_3 | Out | 1'd0 |  |
| dphy_M_Direction_dl_l_3 | In | 1'd0 |  |
| dphy_M_ULPSActiveNot_dl_l_3 | In | 1'd0 |  |
| dphy_M_StopState_dl_l_3 | In | 1'd0 |  |
| dphy_M_ErrControl_dl_l_3 | In | 1'd0 |  |
| dphy_M_ErrEsc_dl_l_3 | In | 1'd0 |  |
| dphy_M_ErrSyncEsc_dl_l_3 | In | 1'd0 |  |
| dphy_M_ErrContentionLP0_dl_l_3 | In | 1'd0 |  |
| dphy_M_ErrContentionLP1_dl_l_3 | In | 1'd0 |  |
| dphy_lane_rstb_dl_l_3 | Out | 1'd0 |  |

### PSI-L Streaming Interface

This interface is a PSI-L endpoint and is used to send and receive data phases from the PSI-L interconnect.

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| strm_i_link | Out | 1'd0 | Link alive indicator |
| strm_i_req | In | 1'd0 | Request |
| strm_i_ready | Out | 1'd0 | Ready |
| strm_i_thread_id[15:0] | In | 16'd0 | Destination Thread ID |
| strm_i_src_id[15:0] | In | 16'd0 | Source Thread ID |
| strm_i_data_type[7:0] | In | 8'd0 | Data type |
| strm_i_wnum[4:0] | In | 5'd0 | Word Number |
| strm_i_lastw | In | 1'd0 | Last Word Indicator |
| strm_i_xcnt[4:0] | In | ALL1S | Valid Data Byte Count |
| strm_i_worden[3:0] | In | ALL1S | Word enables |
| strm_i_data[127:0] | In | 128'd0 | Data |
| strm_i_sop | In | 1'd0 | Start of Packet Indicator |
| strm_i_eop | In | 1'd0 | End of Packet Indicator |
| strm_i_sol | In | 1'd0 | Start of Line Indicator |
| strm_i_eol | In | 1'd0 | End of Line Indicator |
| strm_i_tdown | In | 1'd0 | Channel Teardown Indicator |
| strm_i_interest | In | 1'b0 | Debug marker |
| strm_i_sreq | Out | 1'b0 | Response Request |
| strm_i_sready | In | 1'b1 | Response Ready |
| strm_i_ssrc_id[15:0] | Out | 16'd0 | Response Source Thread ID |
| strm_i_sthread_id[15:0] | Out | 16'd0 | Reaponse Destination Thread ID |
| strm_i_sccnt[7:0] | Out | 8'd0 | Response data phase credit count return |
| strm_o_link | In | 1'd0 | Link alive indicator |
| strm_o_req | Out | 1'd0 | Request |
| strm_o_ready | In | 1'd0 | Ready |
| strm_o_thread_id[15:0] | Out | 16'd0 | Destination Thread ID |
| strm_o_src_id[15:0] | Out | 16'd0 | Source Thread ID |
| strm_o_data_type[7:0] | Out | 8'd0 | Data type |
| strm_o_wnum[4:0] | Out | 5'd0 | Word Number |
| strm_o_lastw | Out | 1'd0 | Last Word Indicator |
| strm_o_xcnt[4:0] | Out | ALL1S | Valid Data Byte Count |
| strm_o_worden[3:0] | Out | ALL1S | Word enables |
| strm_o_data[127:0] | Out | 128'd0 | Data |
| strm_o_sop | Out | 1'd0 | Start of Packet Indicator |
| strm_o_eop | Out | 1'd0 | End of Packet Indicator |
| strm_o_sol | Out | 1'd0 | Start of Line Indicator |
| strm_o_eol | Out | 1'd0 | End of Line Indicator |
| strm_o_tdown | Out | 1'd0 | Channel Teardown Indicator |
| strm_o_interest | Out | 1'b0 | Debug marker |
| strm_o_sreq | In | 1'b0 | Response Request |
| strm_o_sready | Out | 1'b1 | Response Ready |
| strm_o_ssrc_id[15:0] | In | 16'd0 | Response Source Thread ID |
| strm_o_sthread_id[15:0] | In | 16'd0 | Reaponse Destination Thread ID |
| strm_o_sccnt[7:0] | In | 8'd0 | Response data phase credit count return |

### Tx FIFO RAM 0 DFT Interface

This is a RAM Design for Test interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| fdram_ramdft_gpi[1023:0] | In | 1024'd0 | BIST GPI |
| fdram_ramdft_gpo[1023:0] | Out | 1024'd0 | BIST GPO |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| fifo_ramdft_gpi[2999:0] | In | 3000'd0 | BIST GPI |
| fifo_ramdft_gpo[2999:0] | Out | 3000'd0 | BIST GPO |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| ram0_ramdft_gpi[1999:0] | In | 2000'd0 | BIST GPI |
| ram0_ramdft_gpo[1999:0] | Out | 2000'd0 | BIST GPO |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| ram1_ramdft_gpi[1999:0] | In | 2000'd0 | BIST GPI |
| ram1_ramdft_gpo[1999:0] | Out | 2000'd0 | BIST GPO |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| ram2_ramdft_gpi[1999:0] | In | 2000'd0 | BIST GPI |
| ram2_ramdft_gpo[1999:0] | Out | 2000'd0 | BIST GPO |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| ram3_ramdft_gpi[1999:0] | In | 2000'd0 | BIST GPI |
| ram3_ramdft_gpo[1999:0] | Out | 2000'd0 | BIST GPO |

### CPU 4 uncorrectable error, pulse

CPU 4 uncorrectable error, pulse

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| cdns_ram_uncorr_pulse_intr | Out | 1'd0 | Interrupt signal |

### CPU 4 uncorrectable error, level

CPU 4 uncorrectable error, level

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| cdns_ram_uncorr_level_intr | Out | 1'd0 | Interrupt signal |

### CPU 4 correctable error, pulse

CPU 4 correctable error, pulse

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| cdns_ram_corr_pulse_intr | Out | 1'd0 | Interrupt signal |

### CPU 4 correctable error, level

CPU 4 correctable error, level

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| cdns_ram_corr_level_intr | Out | 1'd0 | Interrupt signal |

### CPU 3 uncorrectable error, pulse

CPU 3 uncorrectable error, pulse

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| uncorr_pulse_intr | Out | 1'd0 | Interrupt signal |

### CPU 3 uncorrectable error, level

CPU 3 uncorrectable error, level

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| uncorr_level_intr | Out | 1'd0 | Interrupt signal |

### CPU 3 correctable error, pulse

CPU 3 correctable error, pulse

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| corr_pulse_intr | Out | 1'd0 | Interrupt signal |

### CPU 3 correctable error, level

CPU 3 correctable error, level

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| corr_level_intr | Out | 1'd0 | Interrupt signal |

## External Pin IO

None

## Data Flow Diagram

(Please See Figure 1: Block Diagram)

Figure 11 : Data Flow Diagram

CSI_TX_IF Data Flow

- Output

  - Data is supplied by CSI_TX_IF to DPHY via the PPI interface using CSI2 protocol

  - CSI2 Protocol serially transfers 1 context at a time with minimum granularity of 1 row per burst transfer prior to switching context.

- 4 Input paths (Stream0/1/2/3)

  - TI implementation supports only 1 active input port.  If multiple input ports/paths are used simultaneously, it is possible to overflow FIFOs, causing errors in the system..

    - DMA/PSIL –OR-

    - 2 retransmit interfaces –OR-

    - Color Bar

  - Stream0 High BW DMA

    - Up to 32 contexts(thread id’s)

    - TI adds FIFO elasticity buffering of 1 context, 1 full line

    - NO VC interleaving due to cadecneip limitation

  - Stream1 Color Bar

    - 1 single VC channel context

  - Stream2/3 Retransmit path RX-to-TX

    - 16 VC, not interleaved.

    - Only 1 VC per stream interface so max of 2 VC’s active at any given time.

    - Minimal buffering

    - (Any other active input ports will overflow retransmit buffer)

    - Glue logic required for non compatibility issues between cadence ip, as a result only supporting RAW formats.

# Physical Implementation

## Clocking

There are Three clock domain.

- main_clk_clk

  - main module clock

  - stream retransmit interface

  - dma interface

- VBUS clock

  - Config interface

  - /2 of main clock

- dphy_tx_byte_clk

  - PHY clock domain

  - Supplied by DPHY PLL, divided down to byte clock

  - Max Freq 312.5MHz

  - Inactive clk when PHY is not in HS operation

  - TI implements DFT Mux to take over clock domain for DFT operation

- Esc clock

  - 20MHz max clock for low speed data transmission and some control signals.

  - Has separate dft clock.

Table 6 : Inter-clock dependency table

|  | Main_clk_clk | Vbus_clk | Dphy_rx_byte_clk | Esc_Clk |
| --- | --- | --- | --- | --- |
| Min. freq | Dphy_rx_byte_clk |  | N/A | N/A |
| Max. freq | 500MHz | 250(/2 main) | 312.5MHz | 20MHz |

- Csi rx/tx main clock should be the same clock.

## Clock Diagram

CSI TX Clocking and Reset Diagram

## Reset

Vendor circuits require async resets.  (Please See Figure Above)

## Latches

There are no latches in the module.

## Placement and Hierarchy Information

TBD

## Hard Macro Information

- CSI_TX_IF: contains no hard macros (with the exception of RAM)

## Power / Voltage

TBD

## Interrupt

- Nothing special.

# DFT

## Scan Implementation

Scan implementation details

## IDDQ Modes

There are no specific IDDQ modes or sequences for IDDQ testing.

## Memory BIST

Memory BIST is not included in this module, but can be facilitated by the inclusion of the RAM_GPI and RAM_GPO I/Os.

## I/O DFT

SECTION III: SOFTWARE USAGE GUIDE

# Programmer’s guide

- You must keep the reset mmr in the asserted state in the control register until all csi tx controller mmr’s are programmed. Only after all csi tx controller programming can you de-assert control reset mmr. If you have to reprogram any csi tx controller registers you MUST have the control mmr reset asserted.

## Mmr configuration guide for PSIL

- You must program these before enabling csi tx controller and sending psil data.

| format | Pixel transmit mode | size | Comments |
| --- | --- | --- | --- |
| Yuv420-8 | single | 0 | Must set yuv420 mode |
| Yuv420-10 | single | 1 | Must set yuv420 mode |
| Yuv422-8 | single | 0 | Must set yuv422 mode |
| Yuv422-10 | single | 1 | Must set yuv422 mode |
| Yuv422-8 | packed | 2 | Must set yuv422 mode |
| Rgb888, rgb666 | single | 2 |  |
| Rgb565, rbg555, rgb444 | single | 1 |  |
| Raw 6-8 | single | 0 |  |
| Raw8 | dual | 1 |  |
| Raw8 | quad | 2 |  |
| Raw10 | single | 1 |  |
| Raw10 | dual | 2 |  |
| Raw12 | single | 1 | No packing |
| Raw12 | dual | 2 | No packing |
| Raw12(packed) | single | 0 | Must set pack12=1 |
| Raw12(packed) | dual | 1 | Must set pack12=1 |
| Raw14 | single | 1 |  |
| Raw14 | dual | 2 |  |
| Raw16 | single | 1 |  |
| Raw20 | single | 2 |  |
| User defined 8 | single | 0 | Same as raw8 |
| User defined 16 | single | 1 | Same as raw16 |

## MMR configuration for retransmit

- You must program retransmit registers before enabling csi rx to send data.

- In csirx to csi tx loop back  need to have the same mode on both csi rx and csi tx  controllers(single, dual, quad). Can not mix dual on csi rx and single on csi tx.

- Update retrains_cntl for virtual channel desired to be transmitted on stream 2/3.

- Enable csi rx to send out data.

- Only supports raw formats

- Data type select(0-7) for tx are driven by bottom 3 bits of datatype from rx. Rx has the true data type. you will need to program the appropriate config registers based on this.

| Datasel(tx) | Datatype(rx) | description |
| --- | --- | --- |
| 0 | 0x28 | RAW6 |
| 1 | 0x29 | RAW7 |
| 2 | 0x2A | RAW8 |
| 3 | 0x2B | RAW10 |
| 4 | 0x2C | RAW12 |
| 5 | 0x2D | RAW14 |
| 6 | 0x2E | RAW16 |
| 7 | 0x2F | RAW20 |

## MMR configuration for COLOR BAR

- Color bar interface must be set to packed yuv422-8 format, this is an extended datatype in tx controller

- You must program csi tx controller before enabling the color bar.

- Program COLOR line delay, frame delay, start delay, param registers with appropriate configurations. Update COLOR CNTL with virtual cahnnel and data type desired and enable.

## Initialization sequence

See cadence csi tx controller spec.

Only after programming the csi tx configuration registers should you de-assert the pixel reset register in the CNTL mmr. After de-asserting the pixel reset you can now enable the csi tx ip.

## Real-time operating requirements

See cadence csi tx controller spec.

## Error recovery

When an underflow error occurs you must reset the entire ip by soc psc reset and restart over..

## Power up/down sequence

- You must ensure no more psil traffic is directed toward the csi tx module.

- You must then go through a tear down process on psil gasket inside csi tx wrapper. Refer to psil spec for tear down process.

- You must then check the stream idle values so that they are idle in the control register.

- You must then stop all traffic inside the csi tx controller and wait till it is idle.  See cadence csi tx controller spec.

- After that you can then proceed to do psc power down routine.