Camera Streaming Interface

CSI_RX_IF

MODULE SPECIFICATION

Recipient agrees that it will not knowingly export or re-export, directly or indirectly, any product or technical data (as defined by the U.S. Export Administration Regulations) including software, or any controlled product (including software) restricted by other applicable national regulations, received from Disclosing party under this Agreement, or any direct product of such technical data, to any destination to which such export or re-export is restricted or prohibited by U.S. or other applicable law, without obtaining prior authorization from U.S. Department of Commerce and other competent government authorities to the extent required by those laws. This provision shall survive termination or expiration of this Agreement. The technology or goods covered by the present agreement are classified according to the following Export Control Code Number (ECCN) in conformance with the US department of Commerce regulation and the European Export Control regulation:

-US ECCN: EAR99

and may require export or re-export license for shipping it in compliance with certain countries regulations.

| TI-Ref | CSI |
| --- | --- |
| Document version | 1.0.37 |
| Release date | Aug 06, 2021 |
| Department | Embedded Processing – Processor |
| Owner | Tony Lell |

Revision History

| Revision | Author | Date | Comments |
| --- | --- | --- | --- |
| x.y.s |  |  |  |
| 0.0.1 | Michael Lachmayr | 2016 DEC20 | Draft |
| 0.1.0 | Michael Lachmayr | 2017 Mar29 | CSI_RX_IF 0.1 release… |
| 0.1.1 | Michael Lachmayr |  | Skipped rev |
| 0.1.2 | Michael Lachmayr | 2017 April 12 | Added some missing topics from RAMP review |
| 0.1.3 | Michael Lachmayr | 2017 June 07 | Add more detail to both VP and DMA portions of the spec. Checkin of latest spec prior to CSI mothball and switch to NSF debug… |
| 0.1.4 | Tony Lell | August 21, 2017 | More details and alignment with latest cadence specs |
| 0.1.5 | Tony Lell | 9/6/2017 | Minor clarifications |
| 0.1.6 | Tony Lell | 9/26/2017 | Updates after implementation |
| 0.1.7 | Tony Lell | 9/27/2017 | Implementation final details |
| 0.1.8 | Tony Lell | 9/28/2017 | clarifications |
| 0.1.9 | Tony Lell | 9/28/2017 | Removed yuv420-8 legacy support |
| 1.0.0 | Tony Lell | 10/16/2017 | Cleanup for sign off version. |
| 1.0.1 | Tony Lell | 10/172017 | Added s/w programming table for using different data types and modes |
| 1.0.2 | Tony Lell | 10/18/2017 | More updates for s/w restrictions in section 10. |
| 1.0.3 | Tony Lell | 10/25/2017 | Updated mmr info from new cadence handoff |
| 1..0.4 | Tony Lell | 11/2/2017 | Updated clock frequencies or main clock to max 500. |
| 1.0.5 | Tony Lell | 11/14/2017 | Added hard ware restrictions for programming vp0/1 registers. |
| 1.0.6 | Tony Lell | 11/15/2017 | Added programming restrictions for vp ports. |
| 1.0.7 | Tony Lell | 12/5/2017 | Updated mmr for pixel reset. |
| 1.0.8 | Tony Lell | 12/6/2017 | Added comments on ram ecc/parity for safety |
| 1.0.9 | Tony Lell | 12/8/2017 | Added safety requirement |
| 1.0.10 | Tony Lell | 12/11/2017 | Ec aggr spec updates |
| 1.0.11 | Tony Lell | 12/13/2017 | Spec feedback |
| 1.0.12 | Tony Lell | 12/282017 | Updated safety info |
| 1.0.13 | Tony Lell | 1/19/2018 | Updated software restrictions to not use full line buffer mode. |
| 1.0.14 | Tony Lell | 1/19/2018 | Updates to remove datatype mmrs. Now derived from interface. |
| 1.0.15 | Tony Lell | 1/22/2018 | Updated r/w rsv mmrs to r only. Updated programming reference for raw8-dual mode. |
| 1.0.16 | Tony Lell | 1/23/2018 | Remove more mmr config and hardware derived. |
| 1.0.17 | Tony Lell | 1/29/2018 | Slight change in wording for height and width definitions for video port mmrs. |
| 1.0.18 | Tony Lell | 1/30/2018 | Updated to note that ecc aggr access requires you to program dphy byte clock to be enabled and running otherwise error on vbus access. |
| 1.0.19 | Tony Lell | 2/15/2018 | Added ecc ram info |
| 1.0.20 | Tony Lell | 2/20/2018 | Added programming sequence for wdog en and vp enable |
| 1.0.21 | Tony Lell | 3/15/2018 | Updated vp error recovery, removed watchdog feature on vp. |
| 1.0.22 | Tony Lell | 5/7/2018 | Added yuv420 limitation |
| 1.0.23 | Tony Lell | 5/18/2018 | Corrected cadence ip mmr reset values. Added retrains limitation |
| 1.0.24 | Tony Lell | 5/24/2018 | Made ready reset state X so you do not have to follow any rules when bringing up the ip |
| 1.0.25 | Tony Lell | 6/14/2018 | Updated clock frequencies. |
| 1.0.26 | Tony Lell | 8/10/2018 | Updated vp0/1 interrupt behavior. |
| 1.0.27 | Tony Lell | 8/13/2018 | Added missing raw 20 memory format. |
| 1.0.28 | Tony Lell | 8/23/2018 | Removed specifics on lanes as that is in cadence spec. |
| 1.0.29 | Tony Lell | 9/7/2018 | Misc clarifications |
| 1.0.30 | Tony Lell | 9/19/2018 | Pid updated and updated vp error conditions |
| 1.0.31 | Tony Lell | 9/21/2018 | Updates to vp error handeling. |
| 1.0.32 | Tony Lell | 11/27/2018 | Updated mmr descriptions due ot xml2vi bug |
| 1.0.33 | Tony Lell | 12/13/2018 | Masked mmr reset value from cadence ip. |
| 1.0.34 | Tony Lell | 2/14/2019 | Added programing issue to be aware of on EOL blanking. |
| 1.0.35 | Tony Lell | 3/19/2018 | Added blanking requirements to programming guide. |
| 1.0.36 | Abiel Orrego | 05/24/2021 | Added note about stream0_fifo_overflow. |
| 1.0.37 | Abiel Orrego | 08/06/2021 | Updated interrupt table. |

Contents

1.	Overview and Feature List	15

1.1	Standards	15

1.2	Features Supported	15

1.3	Features Not Supported	16

1.4	Safety	17

1.4.1	memories	17

Ram instance	17

safety	17

coments	17

Csi tx ram 0	17

Inject only	17

Cadence ip does parity	17

Csi tx ram 1	17

Inject only	17

Cadence ip does parity	17

Csi tx ram 2	17

Inject only	17

Cadence ip does parity	17

Csi tx ram 3	17

Inject only	17

Cadence ip does parity	17

Dma fifo	17

Ecc with No writeback	17

Read only once	17

Video port ram 0	17

Ecc with No writeback	17

Read only once	17

Video port ram 1	17

Ecc with No writeback	17

Read only once	17

1.4.2	cadence interfaces	17

asf_pixel_data_if0_par	17

asf_pixel_data_if1_par	17

asf_pixel_data_if2_par	17

asf_pixel_data_if3_par	17

asf_paddr_par	17

asf_wdata_par	17

Functional Description	18

1.5	Memory organization of data.	19

1.6	Functional Operation	23

2. Interrupt Conditions	23

2.1	CPU Interrupts	23

2.2	Debug Events	23

2.3	Interrupt Description	23

2.3.1	csi_level_intr/csi_pulse_intr	23

2.3.2	csi_irq_intr	23

2.3.3	csi_err_irq_intr	23

2.3.4	csi_fatal_intr	24

2.3.5	csi_nonfatal_intr	24

2.3.6	uncorr_pulse_intr/uncorr_level_intr	24

3. Memory Map	24

3.1	Legend	24

3.2	MMR Region Summary	24

3.3	MMR Region: mmr__vbusp_mmr__csi2rxif_regs	25

3.3.1	Register Map	25

3.3.2	Register Definitions	25

3.3.2.1	CSIRX_ID (0x0)	25

3.3.2.2	VP0 (Video Port 0 config), (0x8)	25

3.3.2.3	VP1 (Video Port 1 config), (0xc)	26

3.3.2.4	CNTL (control register for csi rx wrapper), (0x10)	26

3.3.2.5	CNTX_CNTL[a]_DMACNTX (DMA Channel Context), (0x20+(a*0x20)), a=0..31	27

3.3.2.6	CNTX_CNTL[a]_PSI_CFG0 (psi configuration register0), (0x20+(a*0x20)+0x4), a=0..31	27

3.3.2.7	CNTX_CNTL[a]_PSI_CFG1 (psi configuration register1), (0x20+(a*0x20)+0x8), a=0..31	27

3.4	MMR Region: vbus2apb_wrap__vbusp_apb__csi2rx_regs	28

3.4.1	Register Map	29

3.4.2	Register Definitions	30

3.4.2.1	DEVICE_CONFIG (0x0)	30

3.4.2.2	SOFT_RESET (0x4)	32

3.4.2.3	STATIC_CFG (0x8)	32

3.4.2.4	ERROR_BYPASS_CFG (0x10)	33

3.4.2.5	MONITOR_IRQS (0x18)	34

3.4.2.6	MONITOR_IRQS_MASK_CFG (0x1c)	35

3.4.2.7	INFO_IRQS (0x20)	37

3.4.2.8	INFO_IRQS_MASK_CFG (0x24)	38

3.4.2.9	ERROR_IRQS (0x28)	39

3.4.2.10	ERROR_IRQS_MASK_CFG (0x2c)	40

3.4.2.11	DPHY_LANE_CONTROL (0x40)	41

3.4.2.12	DPHY_STATUS (0x48)	42

3.4.2.13	DPHY_ERR_STATUS_IRQ (0x4c)	42

3.4.2.14	DPHY_ERR_IRQ_MASK_CFG (0x50)	43

3.4.2.15	INTEGRATION_DEBUG (0x60)	43

3.4.2.16	ERROR_DEBUG (0x74)	44

3.4.2.17	TEST_GENERIC (0x80)	44

3.4.2.18	STREAM0_CTRL (0x100)	44

3.4.2.19	STREAM0_STATUS (0x104)	45

3.4.2.20	STREAM0_DATA_CFG (0x108)	46

3.4.2.21	STREAM0_CFG (0x10c)	47

3.4.2.22	STREAM0_MONITOR_CTRL (0x110)	49

3.4.2.23	STREAM0_MONITOR_FRAME (0x114)	50

3.4.2.24	STREAM0_MONITOR_LB (0x118)	50

3.4.2.25	STREAM0_TIMER (0x11c)	51

3.4.2.26	STREAM0_FCC_CFG (0x120)	51

3.4.2.27	STREAM0_FCC_CTRL (0x124)	51

3.4.2.28	STREAM0_FIFO_FILL_LVL (0x128)	52

3.4.2.29	STREAM1_CTRL (0x200)	52

3.4.2.30	STREAM1_STATUS (0x204)	53

3.4.2.31	STREAM1_DATA_CFG (0x208)	54

3.4.2.32	STREAM1_CFG (0x20c)	55

3.4.2.33	STREAM1_MONITOR_CTRL (0x210)	57

3.4.2.34	STREAM1_MONITOR_FRAME (0x214)	58

3.4.2.35	STREAM1_MONITOR_LB (0x218)	58

3.4.2.36	STREAM1_TIMER (0x21c)	59

3.4.2.37	STREAM1_FCC_CFG (0x220)	59

3.4.2.38	STREAM1_FCC_CTRL (0x224)	59

3.4.2.39	STREAM1_FIFO_FILL_LVL (0x228)	60

3.4.2.40	STREAM2_CTRL (0x300)	60

3.4.2.41	STREAM2_STATUS (0x304)	61

3.4.2.42	STREAM2_DATA_CFG (0x308)	62

3.4.2.43	STREAM2_CFG (0x30c)	63

3.4.2.44	STREAM2_MONITOR_CTRL (0x310)	65

3.4.2.45	STREAM2_MONITOR_FRAME (0x314)	66

3.4.2.46	STREAM2_MONITOR_LB (0x318)	66

3.4.2.47	STREAM2_TIMER (0x31c)	67

3.4.2.48	STREAM2_FCC_CFG (0x320)	67

3.4.2.49	STREAM2_FCC_CTRL (0x324)	67

3.4.2.50	STREAM2_FIFO_FILL_LVL (0x328)	68

3.4.2.51	STREAM3_CTRL (0x400)	68

3.4.2.52	STREAM3_STATUS (0x404)	69

3.4.2.53	STREAM3_DATA_CFG (0x408)	70

3.4.2.54	STREAM3_CFG (0x40c)	71

3.4.2.55	STREAM3_MONITOR_CTRL (0x410)	73

3.4.2.56	STREAM3_MONITOR_FRAME (0x414)	74

3.4.2.57	STREAM3_MONITOR_LB (0x418)	74

3.4.2.58	STREAM3_TIMER (0x41c)	75

3.4.2.59	STREAM3_FCC_CFG (0x420)	75

3.4.2.60	STREAM3_FCC_CTRL (0x424)	75

3.4.2.61	STREAM3_FIFO_FILL_LVL (0x428)	76

3.4.2.62	ASF_INT_STATUS (0x900)	76

3.4.2.63	ASF_INT_RAW_STATUS (0x904)	76

3.4.2.64	ASF_INT_MASK (0x908)	77

3.4.2.65	ASF_INT_TEST (0x90c)	77

3.4.2.66	ASF_FATAL_NONFATAL_SELECT (0x910)	78

3.4.2.67	ASF_SRAM_CORR_FAULT_STATUS (0x920)	78

3.4.2.68	ASF_SRAM_UNCORR_FAULT_STATUS (0x924)	78

3.4.2.69	ASF_SRAM_FAULT_STATS (0x928)	78

3.4.2.70	ASF_TRANS_TO_CTRL (0x930)	79

3.4.2.71	ASF_TRANS_TO_FAULT_MASK (0x934)	79

3.4.2.72	ASF_TRANS_TO_FAULT_STATUS (0x938)	79

3.4.2.73	ASF_PROTOCOL_FAULT_MASK (0x940)	79

3.4.2.74	ASF_PROTOCOL_FAULT_STATUS (0x944)	80

3.4.2.75	ID_PROD_VER (0xffc)	81

3.5	MMR Region: cp_intd__intd_cfg__intd_cfg	81

3.5.1	Register Map	81

3.5.2	Register Definitions	82

3.5.2.1	REVISION (0x0)	82

3.5.2.2	EOI_REG (0x10)	82

3.5.2.3	INTR_VECTOR_REG (0x14)	82

3.5.2.4	ENABLE_REG_LEVEL_0 (0x100)	82

3.5.2.5	ENABLE_REG_PULSE_0 (0x104)	82

3.5.2.6	ENABLE_CLR_REG_LEVEL_0 (0x300)	83

3.5.2.7	ENABLE_CLR_REG_PULSE_0 (0x304)	83

3.5.2.8	STATUS_REG_LEVEL_0 (0x500)	83

3.5.2.9	STATUS_REG_PULSE_0 (0x504)	83

3.5.2.10	STATUS_CLR_REG_LEVEL_0 (0x700)	83

3.5.2.11	STATUS_CLR_REG_PULSE_0 (0x704)	84

3.5.2.12	INTR_VECTOR_REG_LEVEL (0xa80)	84

3.5.2.13	INTR_VECTOR_REG_PULSE (0xa84)	84

3.6	MMR Region: ecc_aggr__cfg__regs	85

3.6.1	Register Map	85

3.6.2	Register Definitions	85

3.6.2.1	AGGR_REVISION (Aggregator Revision Register), (0x0)	85

3.6.2.2	ECC_VECTOR (ECC Vector Register), (0x8)	85

3.6.2.3	MISC_STATUS (Misc Status), (0xc)	86

3.6.2.4	SEC_EOI_REG (EOI Register), (0x3c)	86

3.6.2.5	SEC_STATUS_REG0 (Interrupt Status Register 0), (0x40)	86

3.6.2.6	SEC_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x80)	86

3.6.2.7	SEC_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0xc0)	86

3.6.2.8	DED_EOI_REG (EOI Register), (0x13c)	86

3.6.2.9	DED_STATUS_REG0 (Interrupt Status Register 0), (0x140)	87

3.6.2.10	DED_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x180)	87

3.6.2.11	DED_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x1c0)	87

3.6.2.12	AGGR_ENABLE_SET (AGGR interrupt enable set Register), (0x200)	87

3.6.2.13	AGGR_ENABLE_CLR (AGGR interrupt enable clear Register), (0x204)	87

3.6.2.14	AGGR_STATUS_SET (AGGR interrupt status set Register), (0x208)	88

3.6.2.15	AGGR_STATUS_CLR (AGGR interrupt status clear Register), (0x20c)	88

4. Submodule Detail	88

4.1	CSI Protocol Details	88

4.1.1	CSI Statistical probability of bit errors	89

4.1.2	CSI dual bit error conditions	89

4.2	DMA (PSI/PSIL) Interface	90

4.2.1	DMA framing	90

4.2.2	DMA Error handling due to fifo overflow	91

4.2.3	DMA safety	91

4.3	VPORT Interface	92

4.3.1	VPORT Error Handling	92

4.3.2	Configuration parameters for CSI RX controller	92

4.4	DV Tap Points	94

5. Submodule details	95

6. Integration Overview	95

6.1	Parameterization Requirements	95

6.2	Memories Embedded in the Peripheral	95

6.2.1	Error Correcting Code (ECC) Support	95

6.2.2	ECC Memories	96

6.3	PLL Requirements	96

6.4	Overall Design For Test	96

6.5	Target Processes	96

6.6	Area Requirements	96

6.7	Power Requirements	96

6.8	Performance Requirements	96

6.8.1.1	MAIN CLK	96

6.8.1.2	VBUS_CLK	97

1.1.1.1	VP_CLK	97

1.1.1.2	RX Byte Clock	97

7. I/O Description	99

7.1	Module I/O	99

7.1.1	synchronous Reset Interface For csi	100

7.1.2	free running clock for csi_rx_if	100

7.1.3	free running clock for csi_rx_if	100

7.1.4	free running clock for video port interface. can be async to main clock. restrictions apply	100

7.1.5	Clock Power Stop	100

7.1.6	Power Clock Enable for csi	100

7.1.7	DFT clock for dphy_rx_byte_clk Clock Domain during Test... muxed in for DFT	100

7.1.8	DFT Interface	101

7.1.9	master interface for control and data interface for VP output	101

7.1.10	master  interface for control and data interface for VP output	101

7.1.11	Interrupt servicing all CSI TI generated RTL aggregated by INTD	101

7.1.12	Interrupt servicing all CSI TI generated RTL aggregated by INTD	101

7.1.13	CSI irq from CSI2RX submodule	102

7.1.14	CSI err_irq from CSI2RX submodule  ERROR Interrupts	102

7.1.15	CSI fatal interrupt from CSI2RX submodule	102

7.1.16	CSI non fatal interrupt from CSI2RX submodule	102

7.1.17	Vbusp configuration port	102

7.1.18	retran	103

7.1.19	ppi	104

7.1.20	PSI-L Streaming Interface	104

7.1.21	mem gpio interface	106

7.1.22	CPU 2 uncorrectable error, pulse	106

7.1.23	CPU 2 uncorrectable error, level	106

7.1.24	mem gpio interface	106

7.1.25	mem gpio interface	106

7.2	External Pin IO	106

7.3	Data Flow Diagram	106

8. Physical Implementation	108

8.1	Clocking	108

8.2	Clock Diagram	109

8.3	Reset	110

8.4	Latches	110

8.5	Placement and Hierarchy Information	110

8.6	Hard Macro Information	110

8.7	Power / Voltage	110

8.8	Interrupt	110

9. DFT	111

9.1	Scan Implementation	111

9.2	IDDQ Modes	111

9.3	Memory BIST	111

9.4	I/O DFT	111

10. Programmer’s guide	113

10.1.1	csirx programming details	113

10.2	Initialization sequence	114

10.3	Real-time operating requirements	114

10.4	Power up/down sequence	114

List of Figures

Figure 1 Functional block diagram	18

Figure 2 cadence stream to PSIL thread mapping	19

Figure 3 yuv422-8 memory data organization	19

Figure 4 yuv422-10 memory data organization	20

Figure 5 yuv420-8 memory data organization	20

Figure 6 yuv420-10 memory data organization	21

Figure 7 raw memory data organization(unpacked)	21

Figure 8 rgb memory data organization	22

Figure 9 raw12 -packed memory data organization	22

Figure 10 : Data Flow Diagram	106

List of Tables

Table 1 : Interrupts	23

Table 2 : Debug events	23

Table 3 : RAM	95

Table 4 : ECC Memory Table	96

Table 5 : Inter-clock dependency table	108

Table 6 Programming requirements for pixel modes and data sizes	113

# Reference documents

| Sl |  | Specification | Revision |  |  |
| --- | --- | --- | --- | --- | --- |

# Acronyms Abbreviations and Definitions

| Term; Definition. EOL; End of Line. SOL; Start of Line. EOF; End of frame. EOL; End of line. EOP; End of packet(EOF). Stream; Cadence streaming interface. psil; TI streaming interface. CSI; Camera streaming interface(MIPI STANDARD). Short packet; 1 set of data that contains meta data and control information. Line; A set of data, usually a single horizontal slice of an image.. Frame; 1 short packet or multiple lines of image data. Data type(DT); Type of data, RAW, YUV, RGB, ETC…. thread; PSIL context. Combination of virtual channel and data type.. |  |
| --- | --- |

SECTION I: FUNCTIONAL OVERVIEW

# Overview and Feature List

## Standards

| Document | Link | Description |
| --- | --- | --- |
| MIPI CSI2 ver1.3 |  | MIPI Spec |
| MIPI CSI2 ver2.0 |  | MIPI Spec |
| MIPI D-PHY ver1.2 |  | MIPI Spec |

## Features Supported

- CSI Version

  - CSI rev 1.3+, CSI 2.0(16 virtual channels)

- DPHY Interface

    - Up to 4 Lanes

- StreamIF0: PSI_L DMA

    - 128bit wide, byte packed(32x4) format, elastic buffer mode

    - Max rate 1 data cycle every 4 main clocks.

    - ByteValid per byte in Last Data Phase

    - 32 thread id’s supported (virtual channel & data type combinations)

    - Flexible number of threads (32 Max)

    - Internal FF based FIFO (Minimal size to cross clock domain)

    - External (TI) Buffer, RAM based(2kx128).

    - Virtual channels and data types are mapped, via mmr, to psil thread id’s.

    - Support for interleaving virtual channels.

      - YUV420 has a limitation that you cannot interleave. You must send the entire frame.

    - Data unpacking for correct memory storage

- StreamIF1: re-transmission to CSI2TX

    - Internal FF based FIFO

    - 32bit Width

    - No External Buffer

    - Csitx and csi rx main clocks must be running at the same frequency and synchronous.

    - Only support RAW formats only.

    - CURRENT ISSUE WTH CADENCE TX IP IS THAT IT DOES NOT SUPPORT Virtual Channel (VC) INTERLEAVING. Will support only 2 VC interleaving for J7 through 2 separate stream interfaces in csi tx. Need cadence ip upgraded for future devices.

    - Clocking rates on PPI interfaces are the same between rx and tx in order to not have underflow or overflow on re-transmit.

- StreamIF2/3: VP0/1/

    - 1 virtual channel and data type per port

    - Raw format only, 8-16 bits.

    - 32bit, 2 Pixels Wide, elastic buffer mode

      - Data[15:0]: Pixel n (MSB zero padding)

      - Data[31:0]: Pixel n+1 (MSB zero padding)

    - Internal FF based FIFO (2048x32) in cadence ip

    - Clock VP async clock to csirx main clock. Crossing done inside cadence ip

Submodule information.

- CDNS stream 2 vidPort conversion

  - Framing conversion

    - SOF/SOL,

      - CDNC vsync/hsync low2high (can happen prior to data)

      - Saves cadence strobe and send with first data

EOF/EOL

      - CDNC vsync/hsync high2low (can happen after data)

      - Uses mmr hiegth and width to send EOL/EOF. If does not later match cadence eol/sol then generate an interrupt.

  - Clock enable based transfers

    - Clock enable low when no data available or stalled from vp.

- TI PSI_L Bridge(shim)

  - Single FIFO exporting cadence ip data to TI PSIL format.

  - PSI_L protocol specifics

    - CSI2RX: SOF/SOL

      - SOF/SOL from cadence IP supplied prior to arrival of first data

      - PSI_L bridge will store SOF/SOL to supply with first data

    - CSI2RX: EOL/EOF

      - Cadence EOL, EOF is supplied after last data

      - PSI_L bridge cannot always apply EOL/EOF with last data

        - Will often see a zero byte cycle over PSI_L with EOL and /or EOP. This could be 2 separate zero length packets.

    - CSI2RX: Cadence packet error. The Cadence csi rx stream interface will send an error packet indication. This will be sent along with the PSIL interface and an erroir.

    - CSI2RX CSI PhyPacket too short

      - A very rare event.  Most likely a programming error or complete failure of PHY.

      - Whether or not CSI2RX creates EOF/EOL, PSI_L bridge will guarantee correct protocol

      - UDMA will react to EOL/EOF and act correctly

    - CSI2RX: PsiPacket too long

      - This is a rare and unlikely event.  Most likely reason would be programming error.  The only known cause of this via CSI injection would be in LineMode, if both the EOF & SOF packets when missing, then the Frame would be double length

      - UDMA should hit max length and truncate

      - Depending on Buffer organization, SW must come in and cleanup DMA, resync to beginning of Frame structure.

        - If the destination buffer is larger than a frame, Then DMA cannot resync since it only has a new frame indicator from CSI PHY.

## Features Not Supported

- CSI2 ver2.0 scrambling

- CSI2 ver2.0 optional no LP (low power state) between packets

- Cropping

- Pixel processing

- Virtualization

- Yuv420-8 legacy not supported

## Safety

- Will target Asil B for safety.

### memories

| # Ram instance | # safety | # coments |
| --- | --- | --- |
| # Csi tx ram 0 | # Inject only | # Cadence ip does parity |
| # Csi tx ram 1 | # Inject only | # Cadence ip does parity |
| # Csi tx ram 2 | # Inject only | # Cadence ip does parity |
| # Csi tx ram 3 | # Inject only | # Cadence ip does parity |
| # Dma fifo | # Ecc with No writeback | # Read only once |
| # Video port ram 0 | # Ecc with No writeback | # Read only once |
| # Video port ram 1 | # Ecc with No writeback | # Read only once |

### cadence interfaces

Cadence as a number of interfaces that have parity inputs along with the data. these are hooked up to edc_ctrl through a small piece of logic that can insert errors.

| # asf_pixel_data_if0_par |  |
| --- | --- |
| # asf_pixel_data_if1_par |  |
| # asf_pixel_data_if2_par |  |
| # asf_pixel_data_if3_par |  |
| # asf_paddr_par |  |
| # asf_wdata_par |  |

# Functional Description

Figure 1 Functional block diagram

The shim module converts cadence csirx controller stream interface to TI buses and proper memory storage. There are 2 bus types that require conversion.

- Video port communication to vpac(viss)(s2vp)

video port conversion requires single virtual channel and data type configuration within the cadence ip. Multiple virtual channels or data types are not supported. Supports only raw data format with 8-16 bit data. 2 pixels per cycle aligned to 16 bit boundary. See VP spec for more details. The VP interface is clock enable based and does not export a clock. The VP interface runs off an async clock compared to csirx controller. This clock must be the same rate or faster than the csirx functional clock. There are 2 VP interfaces, both operate independent from each other with exception of sharing the same clock.

- Psil(shim)

The cadence ip is 32-bits wide in pixel format  based on csi standards.  These 32-bits are re-formatted to TI requirements and sent as pixel format into 128 bits as PSIL data.

There are 32 sets of mmrs that map virtual channels and data type to psil threads. data is sent out PSIL in FIFO order as it is received. There is no priority. Data re formatting is done to end up with correct data organization in memory so that other ip can use the data. The data type must be configured in mmrs to ensure proper reformatting. See memory organization and programming reference for details.

Figure 2 cadence stream to PSIL thread mapping

Safety:

See cadence spec for what the internal csirx controller supports.

The large buffer for psil data(2048x128) has ecc protection..

## Memory organization of data.

Figure 3 yuv422-8 memory data organization

Figure 4 yuv422-10 memory data organization

Figure 5 yuv420-8 memory data organization

Figure 6 yuv420-10 memory data organization

Figure 7 raw memory data organization(unpacked)

Figure 8 rgb memory data organization

Figure 9 raw12 -packed memory data organization

NOTE: Where data does not fill out to a byte boundary it is zero extended.

## Functional Operation

# Interrupt Conditions

## CPU Interrupts

The following interrupts are generated by the module:

Table 1 : Interrupts

| Interrupt | Description |
| --- | --- |
| csi_level_intr | interrupt for error conditions in TI logic(level) |
| csi_pulse_intr | interrupt for error conditions in TI logic(pulse) |
| csi_irq_intr | Maps to irq interrupt from cadence ip(level) |
| csi_err_irq_intr | Maps to irq_err from cadence ip. (level) |
| csi_fatal_intr | Csi rx controller safety interrupt(level) |
| csi_nonfatal_intr | Csi rx controller safety interrupt(level) |
| corr_pulse_intr | Csi rx controller interface party interrupts |
| corr_level_intr | Csi rx controller interface party interrupts |
| uncorr_pulse_intr | Csi rx controller interface party interrupts |
| uncorr_level_intr | Csi rx controller interface party interrupts |

## Debug Events

Table 2 : Debug events

No debug events.

## Interrupt Description

There are a mixture of TI generated interrupts and cadence ip generated interrupts.

### csi_level_intr/csi_pulse_intr

These are error interrupts that are generated from TI logic. There 7 conditions for these interrupts.

- Psil fifo overflow

- VP0 frame/line mismatch. At a minimum an error interrupt will occur at the end of frame but may be issued within the frame as well.

- VP1 frame/line mismatch. At a minimum an error interrupt will occur at the end of frame but may be issued within the frame as well.

### csi_irq_intr

See cadence documentation

### csi_err_irq_intr

See cadence documentation

### csi_fatal_intr

See cadence documentation

### csi_nonfatal_intr

See cadence documentation

### uncorr_pulse_intr/uncorr_level_intr

These interrupts are for checking the interface signals of the csi controller for parity.

# Memory Map

## Legend

The following sections describe the register map and register definitions for each region in the design. The state of each register after reset is shown. The key used to describe each register field is described in the following table.

| Type | Description |
| --- | --- |
| r | A read-only field. Writes have no effect. |
| w | A write-only field. Reads return zeroes. |
| rw | A readable and writable field. Writing any value to this field results in the corresponding bits written with value written. Reads do not alter the value of the field. |
| rw1tc | A write ones to clear field. Writing a one to any bit will clear the corresponding bits. Reads do not alter the value of the field. |
| rw1ts | A write ones to set bit. Writing a one to any bit will set the corresponding bit. Reads do not alter the value of the field. |
| rwi | A write to increment field. Writing a value to this field increment the field value by the value written. Reads do not alter the value of the field. |
| rwd | A write to decrement field. Writing a value to this field decrements the field value by the value written. Reads do not alter the value of the field. |
| w1tc | A write-only write ones to clear field. Writing a one to any bit will clear the corresponding bits. Reads return zeroes. |
| w1ts | A write-only write ones to set bit. Writing a one to any bit will set the corresponding bit. Reads return zeroes. |
| wi | A write-only write to increment field. Writing a value to this field increment the field value by the value written. Reads return zeroes. |
| wd | A write-only write to decrement field. Writing a value to this field decrements the field value by the value written. Reads return zeroes. |

## MMR Region Summary

The following table provides a summary of the bus interfaces and the MMR regions mapped to each interface in this module.

| Interface name | Region name | Addr size | Byte offset | Request ID |
| --- | --- | --- | --- | --- |
| vbusp | mmr__vbusp_mmr__csi2rxif_regs | 12 | 0x0 | 1 |
| vbusp | vbus2apb_wrap__vbusp_apb__csi2rx_regs | 12 | 0x0 | 2 |
| vbusp | cp_intd__intd_cfg__intd_cfg | 12 | 0x0 | 3 |
| vbusp | ecc_aggr__cfg__regs* | 10 | 0x0 | 0 |

- ecc aggr and csi2rx_regs are only word accessible not byte.

## MMR Region: mmr__vbusp_mmr__csi2rxif_regs

MMR in this region are for the wrapper portion of the CSI2_RX_IF.  CSI2RX MMR are in a separate region and comprise the majority of the CFG

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | CSIRX_ID |  |
| 0x4 | VP | Video Port config, all vp |
| 0x8 | VP0 | Video Port 0 config |
| 0xc | VP1 | Video Port 1 config |
| 0x10 | CNTL | control register for csi rx wrapper |
| 0x20+(a*0x20), a=0..31 | CNTX_CNTL[a]_DMACNTX | DMA Channel Context |
| 0x20+(a*0x20)+0x4, a=0..31 | CNTX_CNTL[a]_PSI_CFG0 | psi configuration register0 |
| 0x20+(a*0x20)+0x8, a=0..31 | CNTX_CNTL[a]_PSI_CFG1 | psi configuration register1 |

### Register Definitions

#### CSIRX_ID (0x0)

nothing

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:30 | scheme | r | 0x1 | scheme |
| 29:28 | bu | r | 0x2 | bu |
| 27:16 | function | r | 0x470 | function |
| 15:11 | rtlver | r | 0xc | rtl version |
| 10:8 | majrev | r | 0x1 | major revision |
| 7:6 | custom | r | 0x0 | custom revision |
| 5:0 | minrev | r | 0x0 | min revision |

#### VP0 (Video Port 0 config), (0x8)

Video Port 0 configuration

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | en_cfg | rw | 0x0 | Video Port enable. Disable:drops pixel data Enable: start on VS, captures and sends frame data. Will force ih and iw size by zero pad or trunc. When 1 prevents writing rest of fields in this register. You should read this value first and if set to 0 then you should write the height/width values. |
| 30:29 | reserved | r | 0x0 | Reserved |
| 28:16 | ih_cfg | rw | 0x0 | input height in units of lines. Only writable when vp0_en_cfg=0. writes blockes when vp0_en_cfg=1 |
| 15:13 | reserved | r | 0x0 | Reserved |
| 12:00 | iw_cfg | rw | 0x0 | input width in units of RAW data samples. Max usable value determined by populated line buffer RAM size. Only writable when vp0_en_cfg=0. writes blovkes when vp0_en_cfg=1 |

#### VP1 (Video Port 1 config), (0xc)

Video Port 1 configuration

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | en_cfg | rw | 0x0 | Video Port enable. Disable:drops pixel data Enable: start on VS, captures and sends frame data. Will force ih and iw size by zero pad or trunc. When 1 prevents writing rest of fields in this register . You should read this value first and if set to 0 then you should write the height/width values. |
| 30:29 | reserved | r | 0x0 | Reserved |
| 28:16 | ih_cfg | rw | 0x0 | input height in units of lines. Only writable when vp0_en_cfg=0. writes blockes when vp1_en_cfg=1 |
| 15:13 | reserved | r | 0x0 | Reserved |
| 12:00 | iw_cfg | rw | 0x0 | input width in units of RAW data samples. Max usable value determined by populated line buffer RAM size. Only writable when vp0_en_cfg=0. writes blovkes when vp1_en_cfg=1 |

#### CNTL (control register for csi rx wrapper), (0x10)

control register for csi rx wrapper

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | pixel_reset | rw | 0x0 | reset for the pixel interface. 0=reset, 1 not in reset. this should be asserted till after you program the csi controller configuration registers |

#### CNTX_CNTL[a]_DMACNTX (DMA Channel Context), (0x20+(a*0x20)), a=0..31

DMA Channel Context.  Configuration for each of 32 possible channel contexts.  Illegal to program 2 chanCntx with same extraction values.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | en_cfg | rw | 0x0 | DMA context is enabled. Will extract channel if input matches dataType and VirtualChan |
| 30 | reserved | r | 0x0 | Reserved |
| 29 | rsv0 | r | 0x0 | reserved |
| 28 | rsv1 | r | 0x0 | reserved |
| 27:26 | yuv422_mode_cfg | rw | 0x3 | yuv422 mode 00:UYVY, 01:VYUY, 10:YUYV, 11:YVYU |
| 25 | rsv2 | r | 0x0 | reserved |
| 24 | dual_pck_cfg | rw | 0x0 | dual packed format extraction for 8 bits or less. See CSIRX programming details on section 10.1.1, and also figures 4 through 9 for more details. See stream_cfg register details on CDNS CSI Spec. |
| 23:22 | reserved | r | 0x0 | Reserved |
| 21:20 | size_cfg | rw | 0x0 | data size shift when unpacking, 00=8, 01=16, 10=32, 11=RSVD |
| 19 | reserved | R | 0x0 | Reserved |
| 18 | pck12_cfg | Rw | 0x0 | 12-bit packing enable. See Figure 9 “raw12 -packed memory data organization” for details. |
| 17:10 | reserved | R | 0x0 | Reserved |
| 9:06 | virtCh_cfg | rw | 0x0 | CSI virtual channel index. Supplied by MIPI CSI protocol to DPHY. For CSIver1.3 program 2MSb==0 |
| 5:00 | datTyp_cfg | rw | 0x0 | CSI data type index. Supplied by MIPI CSI protocol to DPHY |

#### CNTX_CNTL[a]_PSI_CFG0 (psi configuration register0), (0x20+(a*0x20)+0x4), a=0..31

psi configuration register0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | dst_tag | rw | 0x0 | psi dst tag |
| 15:0 | src_tag | rw | 0x0 | psi source tag |

#### CNTX_CNTL[a]_PSI_CFG1 (psi configuration register1), (0x20+(a*0x20)+0x8), a=0..31

psi configuration register1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:12 | reserved | r | 0x0 | Reserved |
| 11:8 | ps_flags | rw | 0x0 | ps flags |
| 7:5 | reserved | r | 0x0 | Reserved |
| 4:0 | pkt_type | rw | 0x0 | psi packet type |

## MMR Region: vbus2apb_wrap__vbusp_apb__csi2rx_regs

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | DEVICE_CONFIG |  |
| 0x4 | SOFT_RESET |  |
| 0x8 | STATIC_CFG |  |
| 0x10 | ERROR_BYPASS_CFG |  |
| 0x18 | MONITOR_IRQS |  |
| 0x1c | MONITOR_IRQS_MASK_CFG |  |
| 0x20 | INFO_IRQS |  |
| 0x24 | INFO_IRQS_MASK_CFG |  |
| 0x28 | ERROR_IRQS |  |
| 0x2c | ERROR_IRQS_MASK_CFG |  |
| 0x40 | DPHY_LANE_CONTROL |  |
| 0x48 | DPHY_STATUS |  |
| 0x4c | DPHY_ERR_STATUS_IRQ |  |
| 0x50 | DPHY_ERR_IRQ_MASK_CFG |  |
| 0x60 | INTEGRATION_DEBUG |  |
| 0x74 | ERROR_DEBUG |  |
| 0x80 | TEST_GENERIC |  |
| 0x100 | STREAM0_CTRL |  |
| 0x104 | STREAM0_STATUS |  |
| 0x108 | STREAM0_DATA_CFG |  |
| 0x10c | STREAM0_CFG |  |
| 0x110 | STREAM0_MONITOR_CTRL |  |
| 0x114 | STREAM0_MONITOR_FRAME |  |
| 0x118 | STREAM0_MONITOR_LB |  |
| 0x11c | STREAM0_TIMER |  |
| 0x120 | STREAM0_FCC_CFG |  |
| 0x124 | STREAM0_FCC_CTRL |  |
| 0x128 | STREAM0_FIFO_FILL_LVL |  |
| 0x200 | STREAM1_CTRL |  |
| 0x204 | STREAM1_STATUS |  |
| 0x208 | STREAM1_DATA_CFG |  |
| 0x20c | STREAM1_CFG |  |
| 0x210 | STREAM1_MONITOR_CTRL |  |
| 0x214 | STREAM1_MONITOR_FRAME |  |
| 0x218 | STREAM1_MONITOR_LB |  |
| 0x21c | STREAM1_TIMER |  |
| 0x220 | STREAM1_FCC_CFG |  |
| 0x224 | STREAM1_FCC_CTRL |  |
| 0x228 | STREAM1_FIFO_FILL_LVL |  |
| 0x300 | STREAM2_CTRL |  |
| 0x304 | STREAM2_STATUS |  |
| 0x308 | STREAM2_DATA_CFG |  |
| 0x30c | STREAM2_CFG |  |
| 0x310 | STREAM2_MONITOR_CTRL |  |
| 0x314 | STREAM2_MONITOR_FRAME |  |
| 0x318 | STREAM2_MONITOR_LB |  |
| 0x31c | STREAM2_TIMER |  |
| 0x320 | STREAM2_FCC_CFG |  |
| 0x324 | STREAM2_FCC_CTRL |  |
| 0x328 | STREAM2_FIFO_FILL_LVL |  |
| 0x400 | STREAM3_CTRL |  |
| 0x404 | STREAM3_STATUS |  |
| 0x408 | STREAM3_DATA_CFG |  |
| 0x40c | STREAM3_CFG |  |
| 0x410 | STREAM3_MONITOR_CTRL |  |
| 0x414 | STREAM3_MONITOR_FRAME |  |
| 0x418 | STREAM3_MONITOR_LB |  |
| 0x41c | STREAM3_TIMER |  |
| 0x420 | STREAM3_FCC_CFG |  |
| 0x424 | STREAM3_FCC_CTRL |  |
| 0x428 | STREAM3_FIFO_FILL_LVL |  |
| 0x900 | ASF_INT_STATUS |  |
| 0x904 | ASF_INT_RAW_STATUS |  |
| 0x908 | ASF_INT_MASK |  |
| 0x90c | ASF_INT_TEST |  |
| 0x910 | ASF_FATAL_NONFATAL_SELECT |  |
| 0x920 | ASF_SRAM_CORR_FAULT_STATUS |  |
| 0x924 | ASF_SRAM_UNCORR_FAULT_STATUS |  |
| 0x928 | ASF_SRAM_FAULT_STATS |  |
| 0x930 | ASF_TRANS_TO_CTRL |  |
| 0x934 | ASF_TRANS_TO_FAULT_MASK |  |
| 0x938 | ASF_TRANS_TO_FAULT_STATUS |  |
| 0x940 | ASF_PROTOCOL_FAULT_MASK |  |
| 0x944 | ASF_PROTOCOL_FAULT_STATUS |  |
| 0xffc | ID_PROD_VER |  |

### Register Definitions

#### DEVICE_CONFIG (0x0)

This register provides information related to the current configuration.

This should be read by FW to determine the number of streams available and other associated

parameters that will influence how the device should be controlled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | stream3_monitor_present | r | 0x1 | Pixel stream 3 Monitor present 1 = implemented |
| 30:29 | stream3_num_pixels | r | 0x0 | The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 28:27 | stream3_fifo_mode | r | 0x0 | Stream 3 FIFO Mode. |
| 26 | stream2_monitor_present | r | 0x1 | Pixel stream 2 Monitor present 1 = implemented |
| 25:24 | stream2_num_pixels | r | 0x0 | The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 23:22 | stream2_fifo_mode | r | 0x0 | Stream 2 FIFO Mode. |
| 21 | stream1_monitor_present | r | 0x1 | Pixel stream 1 Monitor present 1 = implemented |
| 20:19 | stream1_num_pixels | r | 0x0 | The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 18:17 | stream1_fifo_mode | r | 0x0 | Stream 1 FIFO Mode. |
| 16 | stream0_monitor_present | r | 0x1 | Pixel stream 0 Monitor present 1 = implemented |
| 15:14 | stream0_num_pixels | r | 0x0 | The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 13:12 | stream0_fifo_mode | r | 0x0 | Stream 0 FIFO Mode. |
| 11 | reserved | r | 0x0 | Reserved |
| 10 | asf_config | r | 0x1 | Additional Safety Features [ASF] Configuration: 0 = None; 1 = Full ASF. |
| 9 | vcx_config | r | 0x1 | Extended Virtual Channel [VCX] Configuration: 0 = 4 VCs; 1 = 16 VCs |
| 8:7 | datapath_size | r | 0x0 | Internal Datapath width 00 - 32 bit, 01 - 64bit, 10 - 16 bit, 11 - 8 Bits. |
| 6:4 | num_streams | r | 0x4 | Number of Stream interfaces [1-4] |
| 3 | cdns_phy_present | r | 0x1 | Cadence DPDHY present 1 = Yes |
| 2:0 | max_lane_nb | r | 0x4 | Max Number of Lanes [1-4] |

#### SOFT_RESET (0x4)

CSI2 Slave Controller Individual Soft Reset for Front and Protocol blocks.

Writing to these registers will cause a single cycle pulse to be applied to the

soft reset signals.

Soft reset must only be applied when the associated clocks are running.

These are used to recover from error conditions and should not be required during

normal operation.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | protocol | rw1tc | 0x0 | writing 1'b1 will apply a synchronous soft reset to the protocol module |
| 0 | front | rw1tc | 0x0 | writing 1'b1 will apply a synchronous soft reset to the Front module |

#### STATIC_CFG (0x8)

Configuration register to set the physical/logical DPHY lane mapping,

the number of lanes being used, external DPHY selection and ECC support for CSI2RX v2.0.

This register should be set prior to enabling the streams and must not be updated

when the stream is running.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | reserved | r | 0x0 | Reserved |
| 30:28 | dl3_map | rw | 0x4 | physical mapping of logical data lane 3 |
| 27 | reserved | r | 0x0 | Reserved |
| 26:24 | dl2_map | rw | 0x3 | physical mapping of logical data lane 2. |
| 23 | reserved | r | 0x0 | Reserved |
| 22:20 | dl1_map | rw | 0x2 | physical mapping of logical data lane 1. |
| 19 | reserved | r | 0x0 | Reserved |
| 18:16 | dl0_map | rw | 0x1 | physical mapping of logical data lane 0. |
| 15:11 | reserved | r | 0x0 | Reserved |
| 10:8 | lane_nb | rw | 0x1 | The number of lanes |
| 7:5 | reserved | r | 0x0 | Reserved |
| 4 | v2p0_support_enable | rw | 0x0 | Support extended VC, up to 16 virtual channels [4-bits] and RAW16/20. as per CSI2RX v2.0. Default is up to 4 virtual channels [3-bits] as per CSI2RX v1.3 |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1:0 | sel | rw | 0x0 | selection of DPHY used as input of CSI2RX module |

#### ERROR_BYPASS_CFG (0x10)

Error detection event flag configuration.

This allows various error conditions to be masked that would normally

prevent data being applied to the pixel interface.

This applies to ALL streams that are enabled.

This register should only be modified while the datapath is disabled.

In case this register is modified while the datapath is enabled,

the behavior on the current frame is unpredictable.

Hard reset value is 0x00000000, meaning all error masking is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | data_id | rw | 0x0 | Enables Data ID error bypass for stream outputs. When enabled, Data ID errors in packets are signalled as interrupt events, however the data is still passed to the pixel/packed data outputs. The system must decide to mask of use the pixel data. |
| 1 | ecc | rw | 0x0 | Enables ECC error bypass for stream outputs. When enabled, CRC errors in packets are signalled as interrupt events, however the data is still passed to the pixel/packed data outputs. The system must decide to mask of use the pixel data. NOTE:Currently not fully supported. |
| 0 | crc | rw | 0x0 | Enables CRC error bypass for stream outputs. When enabled, CRC errors in packets are signalled as interrupt events, however the data is still passed to the pixel/packed data outputs. The system must decide to mask of use the pixel data. |

#### MONITOR_IRQS (0x18)

Information type Interrupt status (non-error conditions)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | stream3_line_cnt_error_irq | rw1tc | 0x0 | Stream 3 Line count error interrupt |
| 30 | stream3_frame_mismatch_irq | rw1tc | 0x0 | Stream 3 Frame mismatch error interrupt |
| 29 | stream3_frame_cnt_error_irq | rw1tc | 0x0 | Stream 3 Frame count error interrupt |
| 28 | stream3_fcc_stop_irq | rw1tc | 0x0 | Stream 3 FCC stop interrupt |
| 27 | stream3_fcc_start_irq | rw1tc | 0x0 | Stream 3 FCC start interrupt |
| 26 | stream3_frame_irq | rw1tc | 0x0 | Stream 3 Frame interrupt |
| 25 | stream3_lb_irq | rw1tc | 0x0 | Stream 3 Line/byte interrupt |
| 24 | stream3_timer_irq | rw1tc | 0x0 | Stream 3 Timer interrupt |
| 23 | stream2_line_cnt_error_irq | rw1tc | 0x0 | Stream 2 Line count error interrupt |
| 22 | stream2_frame_mismatch_irq | rw1tc | 0x0 | Stream 2 Frame mismatch error interrupt |
| 21 | stream2_frame_cnt_error_irq | rw1tc | 0x0 | Stream 2 Frame count error interrupt |
| 20 | stream2_fcc_stop_irq | rw1tc | 0x0 | Stream 2 FCC stop interrupt |
| 19 | stream2_fcc_start_irq | rw1tc | 0x0 | Stream 2 FCC start interrupt |
| 18 | stream2_frame_irq | rw1tc | 0x0 | Stream 2 Frame interrupt |
| 17 | stream2_lb_irq | rw1tc | 0x0 | Stream 2 Line/byte interrupt |
| 16 | stream2_timer_irq | rw1tc | 0x0 | Stream 2 Timer interrupt |
| 15 | stream1_line_cnt_error_irq | rw1tc | 0x0 | Stream 1 Line count error interrupt |
| 14 | stream1_frame_mismatch_irq | rw1tc | 0x0 | Stream 1 Frame mismatch error interrupt |
| 13 | stream1_frame_cnt_error_irq | rw1tc | 0x0 | Stream 1 Frame count error interrupt |
| 12 | stream1_fcc_stop_irq | rw1tc | 0x0 | Stream 1 FCC stop interrupt |
| 11 | stream1_fcc_start_irq | rw1tc | 0x0 | Stream 1 FCC start interrupt |
| 10 | stream1_frame_irq | rw1tc | 0x0 | Stream 1 Frame interrupt |
| 9 | stream1_lb_irq | rw1tc | 0x0 | Stream 1 Line/byte interrupt |
| 8 | stream1_timer_irq | rw1tc | 0x0 | Stream 1 Timer interrupt |
| 7 | stream0_line_cnt_error_irq | rw1tc | 0x0 | Stream 0 Line count error interrupt |
| 6 | stream0_frame_mismatch_irq | rw1tc | 0x0 | Stream 0 Frame mismatch error interrupt |
| 5 | stream0_frame_cnt_error_irq | rw1tc | 0x0 | Stream 0 Frame count error interrupt |
| 4 | stream0_fcc_stop_irq | rw1tc | 0x0 | Stream 0 FCC stop interrupt |
| 3 | stream0_fcc_start_irq | rw1tc | 0x0 | Stream 0 FCC start interrupt |
| 2 | stream0_frame_irq | rw1tc | 0x0 | Stream 0 Frame interrupt |
| 1 | stream0_lb_irq | rw1tc | 0x0 | Stream 0 Line/byte interrupt |
| 0 | stream0_timer_irq | rw1tc | 0x0 | Stream 0 Timer interrupt |

#### MONITOR_IRQS_MASK_CFG (0x1c)

Monitor interrupt mask. Bit addressable mask register in order to independently enable

each event to trigger the monitor_irq line. Only events whose corresponding bit

is set to 1 can trigger the interruption line.

Hard reset is 0x00000000, i.e. interrupt line disabled

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | stream3_line_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 3 Line count error. |
| 30 | stream3_frame_mismatch_irqm | rw | 0x0 | Interrupt mask for stream 3 Frame mismatch error. |
| 29 | stream3_frame_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 3 Frame count error. |
| 28 | stream3_fcc_stop_irqm | rw | 0x0 | Interrupt mask for stream 3 FCC stop. |
| 27 | stream3_fcc_start_irqm | rw | 0x0 | Interrupt mask for stream 3 FCC start. |
| 26 | stream3_frame_irqm | rw | 0x0 | Interrupt mask for stream 3 Frame. |
| 25 | stream3_lb_irqm | rw | 0x0 | Interrupt mask for stream 3 Line/byte. |
| 24 | stream3_timer_irqm | rw | 0x0 | Interrupt mask stream 3 Timer |
| 23 | stream2_line_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 2 Line count error. |
| 22 | stream2_frame_mismatch_irqm | rw | 0x0 | Interrupt mask for stream 2 Frame mismatch error. |
| 21 | stream2_frame_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 2 Frame count error. |
| 20 | stream2_fcc_stop_irqm | rw | 0x0 | Interrupt mask for stream 2 FCC stop. |
| 19 | stream2_fcc_start_irqm | rw | 0x0 | Interrupt mask for stream 2 FCC start. |
| 18 | stream2_frame_irqm | rw | 0x0 | Interrupt mask for stream 2 Frame. |
| 17 | stream2_lb_irqm | rw | 0x0 | Interrupt mask for stream 2 Line/byte. |
| 16 | stream2_timer_irqm | rw | 0x0 | Interrupt mask stream 2 Timer |
| 15 | stream1_line_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 1 Line count error. |
| 14 | stream1_frame_mismatch_irqm | rw | 0x0 | Interrupt mask for stream 1 Frame mismatch error. |
| 13 | stream1_frame_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 1 Frame count error. |
| 12 | stream1_fcc_stop_irqm | rw | 0x0 | Interrupt mask for stream 1 FCC stop. |
| 11 | stream1_fcc_start_irqm | rw | 0x0 | Interrupt mask for stream 1 FCC start. |
| 10 | stream1_frame_irqm | rw | 0x0 | Interrupt mask for stream 1 Frame. |
| 9 | stream1_lb_irqm | rw | 0x0 | Interrupt mask for stream 1 Line/byte. |
| 8 | stream1_timer_irqm | rw | 0x0 | Interrupt mask stream 1 Timer |
| 7 | stream0_line_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 0 Line count error. |
| 6 | stream0_frame_mismatch_irqm | rw | 0x0 | Interrupt mask for stream 0 Frame mismatch error. |
| 5 | stream0_frame_cnt_error_irqm | rw | 0x0 | Interrupt mask for stream 0 Frame count error. |
| 4 | stream0_fcc_stop_irqm | rw | 0x0 | Interrupt mask for stream 0 FCC stop. |
| 3 | stream0_fcc_start_irqm | rw | 0x0 | Interrupt mask for stream 0 FCC start. |
| 2 | stream0_frame_irqm | rw | 0x0 | Interrupt mask for stream 0 Frame. |
| 1 | stream0_lb_irqm | rw | 0x0 | Interrupt mask for stream 0 Line/byte. |
| 0 | stream0_timer_irqm | rw | 0x0 | Interrupt mask stream 0 Timer |

#### INFO_IRQS (0x20)

Information type Interrupt status (non-error conditions)

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:15 | reserved | r | 0x0 | Reserved |
| 14 | stream3_abort_irq | rw1tc | 0x0 | Stream 3 Abort process complete. Apply a Soft reset before re-enabling the stream. |
| 13 | stream3_stop_irq | rw1tc | 0x0 | Stream 3 Stop process complete. Apply a Soft reset before re-enabling the stream. |
| 12 | stream2_abort_irq | rw1tc | 0x0 | Stream 2 Abort process complete. Apply a Soft reset before re-enabling the stream. |
| 11 | stream2_stop_irq | rw1tc | 0x0 | Stream 2 Stop process complete. Apply a Soft reset before re-enabling the stream. |
| 10 | stream1_abort_irq | rw1tc | 0x0 | Stream 1 Abort process complete. Apply a Soft reset before re-enabling the stream. |
| 9 | stream1_stop_irq | rw1tc | 0x0 | Stream 1 Stop process complete. Apply a Soft reset before re-enabling the stream. |
| 8 | stream0_abort_irq | rw1tc | 0x0 | Stream 0 Abort process complete. Apply a Soft reset before re-enabling the stream. |
| 7 | stream0_stop_irq | rw1tc | 0x0 | Stream 0 Stop process complete. Apply a Soft reset before re-enabling the stream. |
| 6 | sp_generic_rcvd_irq | rw1tc | 0x0 | A generic short packet has been received. |
| 5 | deskew_entry_irq | rw1tc | 0x0 | Either clock or any datalane has entered deskew |
| 4 | ecc_spares_nonzero_irq | rw1tc | 0x0 | Bits 7:6 of the ECC byte are non-zero. Indicates non compliance with the MIPI specification although the core will continue to operate as normal. |
| 3 | wakeup_irq | rw1tc | 0x0 | Wake-up interrupt. |
| 2 | sleep_irq | rw1tc | 0x0 | Sleep interrupt. |
| 1 | lp_rcvd_irq | rw1tc | 0x0 | Long Packet received by the protocol module |
| 0 | sp_rcvd_irq | rw1tc | 0x0 | Short Packet received by the protocol module |

#### INFO_IRQS_MASK_CFG (0x24)

Information interrupt mask. Bit addressable mask register in order to independently enable

each event to trigger the info_irq line. Only events whose corresponding bit

is set to 1 can trigger the interruption line.

Hard reset is 0x00000000, i.e. interrupt line disabled

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:15 | reserved | r | 0x0 | Reserved |
| 14 | stream3_abort_irqm | rw | 0x0 | Interrupt mask for stream 3 Abort process. |
| 13 | stream3_stop_irqm | rw | 0x0 | Interrupt mask for Stream 3 Stop process complete. |
| 12 | stream2_abort_irqm | rw | 0x0 | Interrupt mask for stream 2 Abort process. |
| 11 | stream2_stop_irqm | rw | 0x0 | Interrupt mask for Stream 2 Stop process complete. |
| 10 | stream1_abort_irqm | rw | 0x0 | Interrupt mask for stream 1 Abort process. |
| 9 | stream1_stop_irqm | rw | 0x0 | Interrupt mask for Stream 1 Stop process complete. |
| 8 | stream0_abort_irqm | rw | 0x0 | Interrupt mask for stream 0 Abort process. |
| 7 | stream0_stop_irqm | rw | 0x0 | Interrupt mask for Stream 0 Stop process complete. |
| 6 | sp_generic_rcvd_irqm | rw | 0x0 | Interrupt mask for Generic Short Packet received |
| 5 | deskew_entry_irqm | rw | 0x0 | Interrupt mask for Deskew entry check |
| 4 | ecc_spares_nonzero_irqm | rw | 0x0 | Interrupt mask for ECC spares check |
| 3 | wakeup_irqm | rw | 0x0 | Interrupt mask for Wake-up interrupt. |
| 2 | sleep_irqm | rw | 0x0 | Interrupt mask for Sleep interrupt. |
| 1 | lp_rcvd_irqm | rw | 0x0 | Interrupt mask for Long Packet received flag |
| 0 | sp_rcvd_irqm | rw | 0x0 | Interrupt mask for Short Packet received |

#### ERROR_IRQS (0x28)

Datapath error interrupt status.

Provides information about data path errors.

The host processor can read the interrupt status register to identify the root cause of the event,

typically after that the csi2rx_err_irq interrupt line is raised.

NOTE: stream0_fifo_overflow_irq will not fire during normal operating conditions. Overflow will be reported via PSIL.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:20 | reserved | r | 0x0 | Reserved |
| 19 | stream3_fifo_overflow_irq | rw1tc | 0x0 | Overflow of the Stream FIFO detected: stream_fifo_overflow[19] -> Stream 3 overflow |
| 18 | stream2_fifo_overflow_irq | rw1tc | 0x0 | Overflow of the Stream FIFO detected: stream_fifo_overflow[18] -> Stream 2 overflow |
| 17 | stream1_fifo_overflow_irq | rw1tc | 0x0 | Overflow of the Stream FIFO detected: stream_fifo_overflow[17] -> Stream 1 overflow |
| 16 | stream0_fifo_overflow_irq | rw1tc | 0x0 | Overflow of the Stream FIFO detected: stream_fifo_overflow[16] -> Stream 0 overflow |
| 15:13 | reserved | r | 0x0 | Reserved |
| 12 | front_trunc_hdr_irq | rw1tc | 0x0 | A truncated header [short or Long] has been received |
| 11 | prot_truncated_packet_irq | rw1tc | 0x0 | A truncated Long packet has been received. Too few/many bytes |
| 10 | front_lp_no_payload_irq | rw1tc | 0x0 | A truncated Long packet has been received. No payload |
| 9 | sp_invalid_rcvd_irq | rw1tc | 0x0 | A reserved or invalid short packet has been received |
| 8 | invalid_access_irq | rw1tc | 0x0 | Invalid access to the configuration register space. |
| 7 | data_id_irq | rw1tc | 0x0 | Data ID error has been detected in the header packet |
| 6 | header_corrected_ecc_irq | rw1tc | 0x0 | ECC error has been detected and corrected. |
| 5 | header_ecc_irq | rw1tc | 0x0 | Unrecoverable ECC error has been detected. |
| 4 | payload_crc_irq | rw1tc | 0x0 | CRC error has been detected. |
| 3:1 | reserved | r | 0x0 | Reserved |
| 0 | front_fifo_overflow_irq | rw1tc | 0x0 | Overflow detected in resynchronization FIFO between DPHY Lane Management and Protocol blocks. This will occur if sys_clk is not fast enough and should be increased since the byte clock frequency is fixed |

#### ERROR_IRQS_MASK_CFG (0x2c)

Datapath error interrupt enable Bits. This register is used to independently enable each event

to trigger the err_irq interrupt line. Only events whose corresponding enable bit is set to 1

can trigger the interrupt line.

Hard reset is 0x0000000, i.e. all interrupt lines are disabled

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:20 | reserved | r | 0x0 | Reserved |
| 19 | stream3_fifo_overflow_irqm | rw | 0x0 | Interrupt enable bit for: stream_fifo_overflow[19] -> Stream 3 overflow |
| 18 | stream2_fifo_overflow_irqm | rw | 0x0 | Interrupt enable bit for: stream_fifo_overflow[18] -> Stream 2 overflow |
| 17 | stream1_fifo_overflow_irqm | rw | 0x0 | Interrupt enable bit for: stream_fifo_overflow[17] -> Stream 1 overflow |
| 16 | stream0_fifo_overflow_irqm | rw | 0x0 | Interrupt enable bit for: stream_fifo_overflow[16] -> Stream 0 overflow |
| 15:13 | reserved | r | 0x0 | Reserved |
| 12 | front_trunc_hdr_irqm | rw | 0x0 | Interrupt enable bit for truncated hdr. |
| 11 | prot_truncated_packet_irqm | rw | 0x0 | Interrupt enable bit for long packet payload with too many/few bytes |
| 10 | front_lp_no_payload_irqm | rw | 0x0 | Interrupt enable bit for long packet header received with no payload |
| 9 | sp_invalid_rcvd_irqm | rw | 0x0 | Interrupt enable bit for invalid short packet |
| 8 | invalid_access_irqm | rw | 0x0 | Interrupt enable bit for error_irqs_invalid_access. |
| 7 | data_id_irqm | rw | 0x0 | Interrupt enable bit for error_irqs_data_id |
| 6 | header_corrected_ecc_irqm | rw | 0x0 | Interrupt enable bit for error_irqs_header_corrected_ecc |
| 5 | header_ecc_irqm | rw | 0x0 | Interrupt enable bit for error_irqs_header_ecc |
| 4 | payload_crc_irqm | rw | 0x0 | Interrupt enable bit for error_irqs_payload_crc |
| 3:1 | reserved | r | 0x0 | Reserved |
| 0 | front_fifo_overflow_irqm | rw | 0x0 | Interrupt enable bit for error_irqs_front_fifo_overflow |

#### DPHY_LANE_CONTROL (0x40)

DPHY lane control for data and clock lanes enables and resets

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:17 | reserved | r | 0x0 | Reserved |
| 16 | cl_reset | rw | 0x0 | DPHY Clock lane Reset |
| 15 | dl3_reset | rw | 0x0 | DPHY data lane 3 Reset |
| 14 | dl2_reset | rw | 0x0 | DPHY data lane 2 Reset |
| 13 | dl1_reset | rw | 0x0 | DPHY data lane 1 Reset |
| 12 | dl0_reset | rw | 0x0 | DPHY data lane 0 Reset |
| 11:5 | reserved | r | 0x0 | Reserved |
| 4 | cl_enable | rw | 0x0 | DPHY Clock lane Enable |
| 3 | dl3_enable | rw | 0x0 | DPHY data lane 3 Enable |
| 2 | dl2_enable | rw | 0x0 | DPHY data lane 2 Enable |
| 1 | dl1_enable | rw | 0x0 | DPHY data lane 1 Enable |
| 0 | dl0_enable | rw | 0x0 | DPHY data lane 0 Enable |

#### DPHY_STATUS (0x48)

DPHY Clock and Data Lane mode status

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:23 | reserved | r | 0x0 | Reserved |
| 22 | dl3_RxULPSEsc | r | 0x0 | DPHY Data lane 3 ULPS Esc |
| 21 | dl3_ULPSActiveNot | r | 0x0 | DPHY Data lane 3 ULPSActiveNot |
| 20 | dl3_StopState | r | X | DPHY Data lane 3 Stop State |
| 19 | reserved | r | 0x0 | Reserved |
| 18 | dl2_RxULPSEsc | r | 0x0 | DPHY Data lane 2 ULPS Esc |
| 17 | dl2_ULPSActiveNot | r | 0x0 | DPHY Data lane 2 ULPSActiveNot |
| 16 | dl2_StopState | r | X | DPHY Data lane 2 Stop State |
| 15 | reserved | r | 0x0 | Reserved |
| 14 | dl1_RxULPSEsc | r | 0x0 | DPHY Data lane 1 ULPS Esc |
| 13 | dl1_ULPSActiveNot | r | 0x0 | DPHY Data lane 1 ULPSActiveNot |
| 12 | dl1_StopState | r | X | DPHY Data lane 1 Stop State |
| 11 | reserved | r | 0x0 | Reserved |
| 10 | dl0_RxULPSEsc | r | 0x0 | DPHY Data lane 0 ULPS Esc |
| 9 | dl0_ULPSActiveNot | r | 0x0 | DPHY Data lane 0 ULPSActiveNot |
| 8 | dl0_StopState | r | X | DPHY Data lane 0 Stop State |
| 7:3 | reserved | r | 0x0 | Reserved |
| 2 | cl_RxULPSClkNot | r | 0x0 | DPHY Clock lane RxULPSClkNot |
| 1 | cl_ULPSActiveNot | r | 0x0 | DPHY Clock lane ULPSActiveNot |
| 0 | cl_StopState | r | X | DPHY Clock lane Stop State |

#### DPHY_ERR_STATUS_IRQ (0x4c)

DPHY error interrupt status

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:21 | reserved | r | 0x0 | Reserved |
| 20 | dl3_ErrSotHS_irq | rw1tc | 0x0 | DPHY Data lane 3 ErrSotHS |
| 19:17 | reserved | r | 0x0 | Reserved |
| 16 | dl2_ErrSotHS_irq | rw1tc | 0x0 | DPHY Data lane 2 ErrSotHS |
| 15:13 | reserved | r | 0x0 | Reserved |
| 12 | dl1_ErrSotHS_irq | rw1tc | 0x0 | DPHY Data lane 1 ErrSotHS |
| 11:9 | reserved | r | 0x0 | Reserved |
| 8 | dl0_ErrSotHS_irq | rw1tc | 0x0 | DPHY Data lane 0 ErrSotHS |
| 7:0 | reserved | r | 0x0 | Reserved |

#### DPHY_ERR_IRQ_MASK_CFG (0x50)

DPHY error interrupt status

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:21 | reserved | r | 0x0 | Reserved |
| 20 | dl3_ErrSotHS_irqm | rw | 0x0 | DPHY Data lane 3 ErrSotHS mask |
| 19:17 | reserved | r | 0x0 | Reserved |
| 16 | dl2_ErrSotHS_irqm | rw | 0x0 | DPHY Data lane 2 ErrSotHS mask |
| 15:13 | reserved | r | 0x0 | Reserved |
| 12 | dl1_ErrSotHS_irqm | rw | 0x0 | DPHY Data lane 1 ErrSotHS mask |
| 11:9 | reserved | r | 0x0 | Reserved |
| 8 | dl0_ErrSotHS_irqm | rw | 0x0 | DPHY Data lane 0 ErrSotHS mask |
| 7:0 | reserved | r | 0x0 | Reserved |

#### INTEGRATION_DEBUG (0x60)

Used to observe the current data field, extracted by the protocol block from the last

short packet data field and FSM state.

FSM states are one-hot.

It also indicates what data type and virtual channels were used for that short packet.

This is primarily used during error condition debug.

Since the data can come from asynchronous domains the data coherency cannot be relied upon.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:28 | prot_fsm_state | r | X | csi2rx_fsm_state 0x1: WAIT_FOR_PACKET 0x2: PAYLOAD_DATA 0x4: PACKET_FOOTER_CHECK |
| 27:26 | reserved | r | 0x0 | Reserved |
| 25:22 | prot_vc | r | 0x0 | Protocol Virtual Channel |
| 21:16 | prot_dt | r | 0x0 | Protocol Datatype |
| 15:0 | prot_word_count | r | 0x0 | Protocol Word Count [Data Field] |

#### ERROR_DEBUG (0x74)

Error condition debug. After an error is detected by the CSI2RX, this register indicates which virtual channel,

datatype and data field is impacted.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | data_field | r | 0x0 | Indicates the Data Field for an invalid CRC/ECC/Data ID |
| 15:10 | reserved | r | 0x0 | Reserved |
| 9:6 | vc | r | 0x0 | Indicates the Virtual Channel for a invalid CRC/ECC/Data ID |
| 5:0 | dt | r | 0x0 | Indicates the Data Type for a invalid CRC/ECC/Data ID |

#### TEST_GENERIC (0x80)

Generic test control and status register that controls and reads primary I/O.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | status | r | 0x0 | Test status - Directly reflects, after resynchronisation into the pclk domain, the state of 'test_generic_status' primary inputs. |
| 15:0 | ctrl | rw | 0x0 | Test control - Directly controls primary outputs 'test_generic_ctrl' |

#### STREAM0_CTRL (0x100)

CSI2RX Stream Data output datapath control.

Start and Stop commands are independent for each output with the exception of pixel outputs that can never be

enabled together. If a pixel output is started while the other is already running,

the start command will be ignored. If both pixel outputs are enabled in a single register access,

then both start commands are ignored and no pixel output is started.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | reserved | r | 0x0 | Reserved |
| 4 | soft_rst | rw1tc | 0x0 | Writing 1'b1 will apply a synchronous soft reset of this stream registers/FIFO |
| 3 | reserved | r | 0x0 | Reserved |
| 2 | abort | w | 0x0 | Writing 1 this register will cause the csi2rx to stop streaming on the corresponding output immediately. This may corrupt the output protocol. stream_abort_irq is generated on completion of the abort operation. |
| 1 | stop | w | 0x0 | Writing 1 in this register will cause csi2rx to stop streaming on the corresponding output at the end of the current frame. If the command is issued during frame blanking, then the datapath will immediately stop streaming data on that output. stream_stop_irq is generated on completion of the stop operation. |
| 0 | start | w | 0x0 | Writing 1 in this register enables the corresponding datapath output. It will start streaming data at the start of the next frame that complies with the output configuration. stream_status[31] running indicates when data stream is enabled. |

#### STREAM0_STATUS (0x104)

CSI2 Slave Controller Status. Contains useful debug information such as

FSM states.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | running | r | 0x0 | The Stream is enabled |
| 30:9 | reserved | r | 0x0 | Reserved |
| 8 | ready_state | r | X | Indicates the state of the pushback signal pixel_ready_if for this stream |
| 7:4 | stream_fsm | r | 0x0 | Output to Stream FSM states: 0x0: STREAM_IDLE 0x1: STREAM_WAIT_CTRL_DATA // Expecting control data next 0x2: STREAM_CTRL // Check contents of Ctrl packet and extract header information 0x3: STREAM_DATA // Pixel stream pixel data unpacking 0x4: STREAM_CONV_PIX_NB // 1st cycle delay for byte2pixel conversion 0x5: STREAM_CONV_PIX_NB_EXT // 2nd cycle delay for byte2pixel conversion 0x6: STREAM_DATA_START // Assert Hsync 0x7: STREAM_DATA_END // De-assert Hysnc 0x8: STREAM_FILL_WAIT // Elastic Buffer cfg - wait until fill level is reached 0x9: STREAM_STOP // Stop at the end of Frame - used to set irq 0xA: STREAM_WAIT_CRC // Wait until CRC check has completed - Full Line cfg 0xB: STREAM_WAIT_PKT_DONE // Wait for CRC to complete before proceeding 0xC: STREAM_PKT_DONE // Packet complete - no error conditions after this point 0xD: STREAM_NULL // NULL pkt received 0xE: STREAM_FLUSH // Flush due to CRC error |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1:0 | protocol_fsm | r | 0x0 | Input to Stream FSM states: 0x0: PROT_IDLE 0x1: PROT_WAIT_CTRL 0x2: PROT_CTRL 0x3: PROT_DATA |

#### STREAM0_DATA_CFG (0x108)

Secondary CSI2 Slave Controller Data outputs configuration.

This register is used to configure the data types and virtual channels

are processed and output by this stream.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_select | rw | 0x0 | Selection of Virtual Channels to be processed: Default '0' -> All Virtual Channels are processed vc_select0[16] -> Virtual Channel Select 0 is processed vc_select1[17] -> Virtual Channel Select 1 is processed vc_select2[18] -> Virtual Channel Select 2 is processed vc_select3[19] -> Virtual Channel Select 3 is processed vc_select4[20] -> Virtual Channel Select 4 is processed vc_select5[21] -> Virtual Channel Select 5 is processed vc_select6[22] -> Virtual Channel Select 6 is processed vc_select7[23] -> Virtual Channel Select 7 is processed vc_select8[24] -> Virtual Channel Select 8 is processed vc_select9[25] -> Virtual Channel Select 9 is processed vc_select10[26] -> Virtual Channel Select 10 is processed vc_select11[27] -> Virtual Channel Select 11 is processed vc_select12[28] -> Virtual Channel Select 12 is processed vc_select13[29] -> Virtual Channel Select 13 is processed vc_select14[30] -> Virtual Channel Select 14 is processed vc_select15[31] -> Virtual Channel Select 15 is processed |
| 15 | enable_dt1 | rw | 0x0 | Enable processing of dt1 |
| 14 | reserved | r | 0x0 | Reserved |
| 13:8 | datatype_select1 | rw | 0x0 | Second data type format that this stream will process |
| 7 | enable_dt0 | rw | 0x0 | Enable processing of dt0 |
| 6 | reserved | r | 0x0 | Reserved |
| 5:0 | datatype_select0 | rw | 0x0 | First data type format that this stream will process |

#### STREAM0_CFG (0x10c)

Primary CSI2 Slave Controller Data pixel outputs configuration.

This register is used to configure the output mode.

It is also used to set up some Stream FIFO related settings.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | fifo_fill | rw | 0x0 | Set the FIFO_FILL_LEVEL which is used to hold data in the FIFO until this level is reached before allow data to be pulled. This setting is only used when fifo_mode is set for Large Buffer operation |
| 15 | reserved | r | 0x0 | Reserved |
| 14:12 | bpp_bypass | rw | 0x0 | Force unpacking of any Data type as selected RAW type. 0 - No bypass 1 - unpack as RAW6 2 - unpack as RAW7 3 - unpack as RAW8 4 - unpack as RAW10 5 - unpack as RAW12 6 - unpack as RAW14 7 - unpack as RAW16 |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:8 | fifo_mode | rw | 0x0 | Stream FIFO configuration, which must be set in accordance to FIFO sizing, flow control and the relationship between the link and pixel interface data rates. Refer to Use Case descriptions for further guidance on FIFO sizing and valid stream configuration options. 00: Full Line Buffer. Hold data in FIFO until CRC check completes. 01: Large Buffer [Fill Level Controlled]. Hold data in FIFO until FIFO_FILL_LEVEL is reached. 1x: Short Buffer. When pixel output data rate can match link data rate, a small buffer can be used to accommodate CDC, pixel data packing, and data rate matching |
| 7:6 | reserved | r | 0x0 | Reserved |
| 5:4 | num_pixels | rw | 0x0 | Number of pixels to output from the stream. Valid values are 1, 2, 4 and 8. The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1 | ls_le_mode | rw | 0x0 | Enable LS/LE control of HYSNC_VALID output. By default, LS and LE short packets are not required and HYSC_VALID will be generated from the start and end of payload data. |
| 0 | interface_mode | rw | 0x0 | Select the output configuration. Pixel = 0 [default] Packed = 1 |

#### STREAM0_MONITOR_CTRL (0x110)

Stream Monitor configuration.

This register is used to configure the CSI2RX Monitors:

Programmable Frame monitor to trigger an event if a truncated frame is detected,

Programmable Timer to trigger an event based on a clock cycle counter after a frame start

or frame end,

Programmable line/byte counters to trigger an event at a specific byte in a line.

This register is used to enable/disable CSI2RX programmable interrupt, select the virtual

channel for each programmable IT and select the point which will trigger the event.

This register should not be modified while the data path is enabled, nor should any of

settings be changed when the respective monitor/counter/timer is enabled.

In these cases, the behaviour is unpredictable.

Hard reset value is 0x00000000, all interrupt generators disabled on Virtual Channel 0.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_length | rw | 0x0 | Indicates the frame length in lines to detect truncated frames. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. 0x0000 means truncated frame detection feature disabled |
| 15 | frame_mon_en | rw | 0x0 | Enables monitor. This bit must only be set high after the frame_mon_vc and frame_length have been set. |
| 14:11 | frame_mon_vc | rw | 0x0 | Indicates virtual channel for monitor. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. |
| 10 | timer_eof | rw | 0x0 | Select the starting point of the timer: 0x0: Start of Frame event on selected virtual channel 0x1: End of Frame event on selected virtual channel. This value must not change while timer_en is enabled |
| 9 | timer_en | rw | 0x0 | Enables timer based interrupt. This bit must only be set high after the timer_eof and timer_vc have been set. |
| 8:5 | timer_vc | rw | 0x0 | Indicates which VC should be used to generate timer based interrupt. This value must not change while timer_en is enabled. |
| 4 | lb_en | rw | 0x0 | Enables line/byte counter. This bit must only be set high after the lb_vc, line_count and byte_count have been set. |
| 3:0 | lb_vc | rw | 0x0 | Indicates which VC should be used to generate line/byte counter interrupt. This value must not change while lb_en is enabled |

#### STREAM0_MONITOR_FRAME (0x114)

Stream Monitor Frame.

Used to observe the current frame number and packet size on monitored virtual channels.

These values are extracted by the CSI2RX from the last frame start short packet data field.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | packet_size | r | 0x0 | Size of the current payload |
| 15:0 | nb | r | 0x0 | Number of the last frame processed. |

#### STREAM0_MONITOR_LB (0x118)

Stream Monitor Line.

Used to specify the byte and line numbers that will generate an interrupt.

This register must only be modified when the corresponding line/byte enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | line_count | rw | 0x0 | Indicates the line number to generate an interrupt. [First line of a Frame is line number 0] |
| 15:0 | byte_count | rw | 0x0 | Indicates the byte number of the line to generate an interrupt. [First byte of a line is byte number 0] |

#### STREAM0_TIMER (0x11c)

Stream Timer.

Used to specify the number of clock cycles until the interrupt is triggered after frame

start or frame end.

This register must only be modified when the corresponding timer enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:25 | reserved | r | 0x0 | Reserved |
| 24:0 | count | rw | 0x0 | Number of clock cycles |

#### STREAM0_FCC_CFG (0x120)

Stream Frame Capture Control configuration.

Used to specify the frame count value when when the CSI2RX must generate interrupts

FCC_START and FCC_STOP.

This register must only be modified when the corresponding frame count enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_count_stop | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will stop outputting data on the pixel interface. [0x0000 will be continuous frames.] |
| 15:0 | frame_count_start | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will start outputting data on the pixel interface. [0x0000 will be the current frame.] |

#### STREAM0_FCC_CTRL (0x124)

Stream Frame Capture Counter control.

Used to enable / disable the FCC and specify which virtual channel it

should operate on.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_counter | r | 0x0 | Current Frame number being processed |
| 15:5 | reserved | r | 0x0 | Reserved |
| 4:1 | fcc_vc | rw | 0x0 | Indicates which VC should be used to generate FCC interrupts. This value must not change while fcc_en is enabled |
| 0 | fcc_en | rw | 0x0 | Frame Capture Counter enable. |

#### STREAM0_FIFO_FILL_LVL (0x128)

Stream FIFO fill level monitor.  This can operate in 2 modes:

1 - Monitor peak fill level until the stream is stopped.

2 - Monitor peak fill level when the first FIFO read is made during a frame.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:14 | reserved | r | 0x0 | Reserved |
| 13:12 | mode | rw | 0x0 | 00 -> Fill level detection disabled 01 -> Mode 1 10 -> Mode 2 11 -> Reserved |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:0 | count | r | 0x0 | Peak fill level of FIFO. |

#### STREAM1_CTRL (0x200)

CSI2RX Stream Data output datapath control.

Start and Stop commands are independent for each output with the exception of pixel outputs that can never be

enabled together. If a pixel output is started while the other is already running,

the start command will be ignored. If both pixel outputs are enabled in a single register access,

then both start commands are ignored and no pixel output is started.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | reserved | r | 0x0 | Reserved |
| 4 | soft_rst | rw1tc | 0x0 | Writing 1'b1 will apply a synchronous soft reset of this stream registers/FIFO |
| 3 | reserved | r | 0x0 | Reserved |
| 2 | abort | w | 0x0 | Writing 1 this register will cause the csi2rx to stop streaming on the corresponding output immediately. This may corrupt the output protocol. stream_abort_irq is generated on completion of the abort operation. |
| 1 | stop | w | 0x0 | Writing 1 in this register will cause csi2rx to stop streaming on the corresponding output at the end of the current frame. If the command is issued during frame blanking, then the datapath will immediately stop streaming data on that output. stream_stop_irq is generated on completion of the stop operation. |
| 0 | start | w | 0x0 | Writing 1 in this register enables the corresponding datapath output. It will start streaming data at the start of the next frame that complies with the output configuration. stream_status[31] running indicates when data stream is enabled. |

#### STREAM1_STATUS (0x204)

CSI2 Slave Controller Status. Contains useful debug information such as

FSM states.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | running | r | 0x0 | The Stream is enabled |
| 30:9 | reserved | r | 0x0 | Reserved |
| 8 | ready_state | r | X | Indicates the state of the pushback signal pixel_ready_if for this stream |
| 7:4 | stream_fsm | r | 0x0 | Output to Stream FSM states: 0x0: STREAM_IDLE 0x1: STREAM_WAIT_CTRL_DATA // Expecting control data next 0x2: STREAM_CTRL // Check contents of Ctrl packet and extract header information 0x3: STREAM_DATA // Pixel stream pixel data unpacking 0x4: STREAM_CONV_PIX_NB // 1st cycle delay for byte2pixel conversion 0x5: STREAM_CONV_PIX_NB_EXT // 2nd cycle delay for byte2pixel conversion 0x6: STREAM_DATA_START // Assert Hsync 0x7: STREAM_DATA_END // De-assert Hysnc 0x8: STREAM_FILL_WAIT // Elastic Buffer cfg - wait until fill level is reached 0x9: STREAM_STOP // Stop at the end of Frame - used to set irq 0xA: STREAM_WAIT_CRC // Wait until CRC check has completed - Full Line cfg 0xB: STREAM_WAIT_PKT_DONE // Wait for CRC to complete before proceeding 0xC: STREAM_PKT_DONE // Packet complete - no error conditions after this point 0xD: STREAM_NULL // NULL pkt received 0xE: STREAM_FLUSH // Flush due to CRC error |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1:0 | protocol_fsm | r | 0x0 | Input to Stream FSM states: 0x0: PROT_IDLE 0x1: PROT_WAIT_CTRL 0x2: PROT_CTRL 0x3: PROT_DATA |

#### STREAM1_DATA_CFG (0x208)

Secondary CSI2 Slave Controller Data outputs configuration.

This register is used to configure the data types and virtual channels

are processed and output by this stream.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_select | rw | 0x0 | Selection of Virtual Channels to be processed: Default '0' -> All Virtual Channels are processed vc_select0[16] -> Virtual Channel Select 0 is processed vc_select1[17] -> Virtual Channel Select 1 is processed vc_select2[18] -> Virtual Channel Select 2 is processed vc_select3[19] -> Virtual Channel Select 3 is processed vc_select4[20] -> Virtual Channel Select 4 is processed vc_select5[21] -> Virtual Channel Select 5 is processed vc_select6[22] -> Virtual Channel Select 6 is processed vc_select7[23] -> Virtual Channel Select 7 is processed vc_select8[24] -> Virtual Channel Select 8 is processed vc_select9[25] -> Virtual Channel Select 9 is processed vc_select10[26] -> Virtual Channel Select 10 is processed vc_select11[27] -> Virtual Channel Select 11 is processed vc_select12[28] -> Virtual Channel Select 12 is processed vc_select13[29] -> Virtual Channel Select 13 is processed vc_select14[30] -> Virtual Channel Select 14 is processed vc_select15[31] -> Virtual Channel Select 15 is processed |
| 15 | enable_dt1 | rw | 0x0 | Enable processing of dt1 |
| 14 | reserved | r | 0x0 | Reserved |
| 13:8 | datatype_select1 | rw | 0x0 | Second data type format that this stream will process |
| 7 | enable_dt0 | rw | 0x0 | Enable processing of dt0 |
| 6 | reserved | r | 0x0 | Reserved |
| 5:0 | datatype_select0 | rw | 0x0 | First data type format that this stream will process |

#### STREAM1_CFG (0x20c)

Primary CSI2 Slave Controller Data pixel outputs configuration.

This register is used to configure the output mode.

It is also used to set up some Stream FIFO related settings.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | fifo_fill | rw | 0x0 | Set the FIFO_FILL_LEVEL which is used to hold data in the FIFO until this level is reached before allow data to be pulled. This setting is only used when fifo_mode is set for Large Buffer operation |
| 15 | reserved | r | 0x0 | Reserved |
| 14:12 | bpp_bypass | rw | 0x0 | Force unpacking of any Data type as selected RAW type. 0 - No bypass 1 - unpack as RAW6 2 - unpack as RAW7 3 - unpack as RAW8 4 - unpack as RAW10 5 - unpack as RAW12 6 - unpack as RAW14 7 - unpack as RAW16 |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:8 | fifo_mode | rw | 0x0 | Stream FIFO configuration, which must be set in accordance to FIFO sizing, flow control and the relationship between the link and pixel interface data rates. Refer to Use Case descriptions for further guidance on FIFO sizing and valid stream configuration options. 00: Full Line Buffer. Hold data in FIFO until CRC check completes. 01: Large Buffer [Fill Level Controlled]. Hold data in FIFO until FIFO_FILL_LEVEL is reached. 1x: Short Buffer. When pixel output data rate can match link data rate, a small buffer can be used to accommodate CDC, pixel data packing, and data rate matching |
| 7:6 | reserved | r | 0x0 | Reserved |
| 5:4 | num_pixels | rw | 0x0 | Number of pixels to output from the stream. Valid values are 1, 2, 4 and 8. The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1 | ls_le_mode | rw | 0x0 | Enable LS/LE control of HYSNC_VALID output. By default, LS and LE short packets are not required and HYSC_VALID will be generated from the start and end of payload data. |
| 0 | interface_mode | rw | 0x0 | Select the output configuration. Pixel = 0 [default] Packed = 1 |

#### STREAM1_MONITOR_CTRL (0x210)

Stream Monitor configuration.

This register is used to configure the CSI2RX Monitors:

Programmable Frame monitor to trigger an event if a truncated frame is detected,

Programmable Timer to trigger an event based on a clock cycle counter after a frame start

or frame end,

Programmable line/byte counters to trigger an event at a specific byte in a line.

This register is used to enable/disable CSI2RX programmable interrupt, select the virtual

channel for each programmable IT and select the point which will trigger the event.

This register should not be modified while the data path is enabled, nor should any of

settings be changed when the respective monitor/counter/timer is enabled.

In these cases, the behaviour is unpredictable.

Hard reset value is 0x00000000, all interrupt generators disabled on Virtual Channel 0.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_length | rw | 0x0 | Indicates the frame length in lines to detect truncated frames. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. 0x0000 means truncated frame detection feature disabled |
| 15 | frame_mon_en | rw | 0x0 | Enables monitor. This bit must only be set high after the frame_mon_vc and frame_length have been set. |
| 14:11 | frame_mon_vc | rw | 0x0 | Indicates virtual channel for monitor. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. |
| 10 | timer_eof | rw | 0x0 | Select the starting point of the timer: 0x0: Start of Frame event on selected virtual channel 0x1: End of Frame event on selected virtual channel. This value must not change while timer_en is enabled |
| 9 | timer_en | rw | 0x0 | Enables timer based interrupt. This bit must only be set high after the timer_eof and timer_vc have been set. |
| 8:5 | timer_vc | rw | 0x0 | Indicates which VC should be used to generate timer based interrupt. This value must not change while timer_en is enabled. |
| 4 | lb_en | rw | 0x0 | Enables line/byte counter. This bit must only be set high after the lb_vc, line_count and byte_count have been set. |
| 3:0 | lb_vc | rw | 0x0 | Indicates which VC should be used to generate line/byte counter interrupt. This value must not change while lb_en is enabled |

#### STREAM1_MONITOR_FRAME (0x214)

Stream Monitor Frame.

Used to observe the current frame number and packet size on monitored virtual channels.

These values are extracted by the CSI2RX from the last frame start short packet data field.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | packet_size | r | 0x0 | Size of the current payload |
| 15:0 | nb | r | 0x0 | Number of the last frame processed. |

#### STREAM1_MONITOR_LB (0x218)

Stream Monitor Line.

Used to specify the byte and line numbers that will generate an interrupt.

This register must only be modified when the corresponding line/byte enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | line_count | rw | 0x0 | Indicates the line number to generate an interrupt. [First line of a Frame is line number 0] |
| 15:0 | byte_count | rw | 0x0 | Indicates the byte number of the line to generate an interrupt. [First byte of a line is byte number 0] |

#### STREAM1_TIMER (0x21c)

Stream Timer.

Used to specify the number of clock cycles until the interrupt is triggered after frame

start or frame end.

This register must only be modified when the corresponding timer enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:25 | reserved | r | 0x0 | Reserved |
| 24:0 | count | rw | 0x0 | Number of clock cycles |

#### STREAM1_FCC_CFG (0x220)

Stream Frame Capture Control configuration.

Used to specify the frame count value when when the CSI2RX must generate interrupts

FCC_START and FCC_STOP.

This register must only be modified when the corresponding frame count enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_count_stop | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will stop outputting data on the pixel interface. [0x0000 will be continuous frames.] |
| 15:0 | frame_count_start | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will start outputting data on the pixel interface. [0x0000 will be the current frame.] |

#### STREAM1_FCC_CTRL (0x224)

Stream Frame Capture Counter control.

Used to enable / disable the FCC and specify which virtual channel it

should operate on.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_counter | r | 0x0 | Current Frame number being processed |
| 15:5 | reserved | r | 0x0 | Reserved |
| 4:1 | fcc_vc | rw | 0x0 | Indicates which VC should be used to generate FCC interrupts. This value must not change while fcc_en is enabled |
| 0 | fcc_en | rw | 0x0 | Frame Capture Counter enable. |

#### STREAM1_FIFO_FILL_LVL (0x228)

Stream FIFO fill level monitor.  This can operate in 2 modes:

1 - Monitor peak fill level until the stream is stopped.

2 - Monitor peak fill level when the first FIFO read is made during a frame.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:14 | reserved | r | 0x0 | Reserved |
| 13:12 | mode | rw | 0x0 | 00 -> Fill level detection disabled 01 -> Mode 1 10 -> Mode 2 11 -> Reserved |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:0 | count | r | 0x0 | Peak fill level of FIFO. |

#### STREAM2_CTRL (0x300)

CSI2RX Stream Data output datapath control.

Start and Stop commands are independent for each output with the exception of pixel outputs that can never be

enabled together. If a pixel output is started while the other is already running,

the start command will be ignored. If both pixel outputs are enabled in a single register access,

then both start commands are ignored and no pixel output is started.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | reserved | r | 0x0 | Reserved |
| 4 | soft_rst | rw1tc | 0x0 | Writing 1'b1 will apply a synchronous soft reset of this stream registers/FIFO |
| 3 | reserved | r | 0x0 | Reserved |
| 2 | abort | w | 0x0 | Writing 1 this register will cause the csi2rx to stop streaming on the corresponding output immediately. This may corrupt the output protocol. stream_abort_irq is generated on completion of the abort operation. |
| 1 | stop | w | 0x0 | Writing 1 in this register will cause csi2rx to stop streaming on the corresponding output at the end of the current frame. If the command is issued during frame blanking, then the datapath will immediately stop streaming data on that output. stream_stop_irq is generated on completion of the stop operation. |
| 0 | start | w | 0x0 | Writing 1 in this register enables the corresponding datapath output. It will start streaming data at the start of the next frame that complies with the output configuration. stream_status[31] running indicates when data stream is enabled. |

#### STREAM2_STATUS (0x304)

CSI2 Slave Controller Status. Contains useful debug information such as

FSM states.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | running | r | 0x0 | The Stream is enabled |
| 30:9 | reserved | r | 0x0 | Reserved |
| 8 | ready_state | r | X | Indicates the state of the pushback signal pixel_ready_if for this stream |
| 7:4 | stream_fsm | r | 0x0 | Output to Stream FSM states: 0x0: STREAM_IDLE 0x1: STREAM_WAIT_CTRL_DATA // Expecting control data next 0x2: STREAM_CTRL // Check contents of Ctrl packet and extract header information 0x3: STREAM_DATA // Pixel stream pixel data unpacking 0x4: STREAM_CONV_PIX_NB // 1st cycle delay for byte2pixel conversion 0x5: STREAM_CONV_PIX_NB_EXT // 2nd cycle delay for byte2pixel conversion 0x6: STREAM_DATA_START // Assert Hsync 0x7: STREAM_DATA_END // De-assert Hysnc 0x8: STREAM_FILL_WAIT // Elastic Buffer cfg - wait until fill level is reached 0x9: STREAM_STOP // Stop at the end of Frame - used to set irq 0xA: STREAM_WAIT_CRC // Wait until CRC check has completed - Full Line cfg 0xB: STREAM_WAIT_PKT_DONE // Wait for CRC to complete before proceeding 0xC: STREAM_PKT_DONE // Packet complete - no error conditions after this point 0xD: STREAM_NULL // NULL pkt received 0xE: STREAM_FLUSH // Flush due to CRC error |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1:0 | protocol_fsm | r | 0x0 | Input to Stream FSM states: 0x0: PROT_IDLE 0x1: PROT_WAIT_CTRL 0x2: PROT_CTRL 0x3: PROT_DATA |

#### STREAM2_DATA_CFG (0x308)

Secondary CSI2 Slave Controller Data outputs configuration.

This register is used to configure the data types and virtual channels

are processed and output by this stream.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_select | rw | 0x0 | Selection of Virtual Channels to be processed: Default '0' -> All Virtual Channels are processed vc_select0[16] -> Virtual Channel Select 0 is processed vc_select1[17] -> Virtual Channel Select 1 is processed vc_select2[18] -> Virtual Channel Select 2 is processed vc_select3[19] -> Virtual Channel Select 3 is processed vc_select4[20] -> Virtual Channel Select 4 is processed vc_select5[21] -> Virtual Channel Select 5 is processed vc_select6[22] -> Virtual Channel Select 6 is processed vc_select7[23] -> Virtual Channel Select 7 is processed vc_select8[24] -> Virtual Channel Select 8 is processed vc_select9[25] -> Virtual Channel Select 9 is processed vc_select10[26] -> Virtual Channel Select 10 is processed vc_select11[27] -> Virtual Channel Select 11 is processed vc_select12[28] -> Virtual Channel Select 12 is processed vc_select13[29] -> Virtual Channel Select 13 is processed vc_select14[30] -> Virtual Channel Select 14 is processed vc_select15[31] -> Virtual Channel Select 15 is processed |
| 15 | enable_dt1 | rw | 0x0 | Enable processing of dt1 |
| 14 | reserved | r | 0x0 | Reserved |
| 13:8 | datatype_select1 | rw | 0x0 | Second data type format that this stream will process |
| 7 | enable_dt0 | rw | 0x0 | Enable processing of dt0 |
| 6 | reserved | r | 0x0 | Reserved |
| 5:0 | datatype_select0 | rw | 0x0 | First data type format that this stream will process |

#### STREAM2_CFG (0x30c)

Primary CSI2 Slave Controller Data pixel outputs configuration.

This register is used to configure the output mode.

It is also used to set up some Stream FIFO related settings.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | fifo_fill | rw | 0x0 | Set the FIFO_FILL_LEVEL which is used to hold data in the FIFO until this level is reached before allow data to be pulled. This setting is only used when fifo_mode is set for Large Buffer operation |
| 15 | reserved | r | 0x0 | Reserved |
| 14:12 | bpp_bypass | rw | 0x0 | Force unpacking of any Data type as selected RAW type. 0 - No bypass 1 - unpack as RAW6 2 - unpack as RAW7 3 - unpack as RAW8 4 - unpack as RAW10 5 - unpack as RAW12 6 - unpack as RAW14 7 - unpack as RAW16 |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:8 | fifo_mode | rw | 0x0 | Stream FIFO configuration, which must be set in accordance to FIFO sizing, flow control and the relationship between the link and pixel interface data rates. Refer to Use Case descriptions for further guidance on FIFO sizing and valid stream configuration options. 00: Full Line Buffer. Hold data in FIFO until CRC check completes. 01: Large Buffer [Fill Level Controlled]. Hold data in FIFO until FIFO_FILL_LEVEL is reached. 1x: Short Buffer. When pixel output data rate can match link data rate, a small buffer can be used to accommodate CDC, pixel data packing, and data rate matching |
| 7:6 | reserved | r | 0x0 | Reserved |
| 5:4 | num_pixels | rw | 0x0 | Number of pixels to output from the stream. Valid values are 1, 2, 4 and 8. The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1 | ls_le_mode | rw | 0x0 | Enable LS/LE control of HYSNC_VALID output. By default, LS and LE short packets are not required and HYSC_VALID will be generated from the start and end of payload data. |
| 0 | interface_mode | rw | 0x0 | Select the output configuration. Pixel = 0 [default] Packed = 1 |

#### STREAM2_MONITOR_CTRL (0x310)

Stream Monitor configuration.

This register is used to configure the CSI2RX Monitors:

Programmable Frame monitor to trigger an event if a truncated frame is detected,

Programmable Timer to trigger an event based on a clock cycle counter after a frame start

or frame end,

Programmable line/byte counters to trigger an event at a specific byte in a line.

This register is used to enable/disable CSI2RX programmable interrupt, select the virtual

channel for each programmable IT and select the point which will trigger the event.

This register should not be modified while the data path is enabled, nor should any of

settings be changed when the respective monitor/counter/timer is enabled.

In these cases, the behaviour is unpredictable.

Hard reset value is 0x00000000, all interrupt generators disabled on Virtual Channel 0.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_length | rw | 0x0 | Indicates the frame length in lines to detect truncated frames. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. 0x0000 means truncated frame detection feature disabled |
| 15 | frame_mon_en | rw | 0x0 | Enables monitor. This bit must only be set high after the frame_mon_vc and frame_length have been set. |
| 14:11 | frame_mon_vc | rw | 0x0 | Indicates virtual channel for monitor. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. |
| 10 | timer_eof | rw | 0x0 | Select the starting point of the timer: 0x0: Start of Frame event on selected virtual channel 0x1: End of Frame event on selected virtual channel. This value must not change while timer_en is enabled |
| 9 | timer_en | rw | 0x0 | Enables timer based interrupt. This bit must only be set high after the timer_eof and timer_vc have been set. |
| 8:5 | timer_vc | rw | 0x0 | Indicates which VC should be used to generate timer based interrupt. This value must not change while timer_en is enabled. |
| 4 | lb_en | rw | 0x0 | Enables line/byte counter. This bit must only be set high after the lb_vc, line_count and byte_count have been set. |
| 3:0 | lb_vc | rw | 0x0 | Indicates which VC should be used to generate line/byte counter interrupt. This value must not change while lb_en is enabled |

#### STREAM2_MONITOR_FRAME (0x314)

Stream Monitor Frame.

Used to observe the current frame number and packet size on monitored virtual channels.

These values are extracted by the CSI2RX from the last frame start short packet data field.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | packet_size | r | 0x0 | Size of the current payload |
| 15:0 | nb | r | 0x0 | Number of the last frame processed. |

#### STREAM2_MONITOR_LB (0x318)

Stream Monitor Line.

Used to specify the byte and line numbers that will generate an interrupt.

This register must only be modified when the corresponding line/byte enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | line_count | rw | 0x0 | Indicates the line number to generate an interrupt. [First line of a Frame is line number 0] |
| 15:0 | byte_count | rw | 0x0 | Indicates the byte number of the line to generate an interrupt. [First byte of a line is byte number 0] |

#### STREAM2_TIMER (0x31c)

Stream Timer.

Used to specify the number of clock cycles until the interrupt is triggered after frame

start or frame end.

This register must only be modified when the corresponding timer enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:25 | reserved | r | 0x0 | Reserved |
| 24:0 | count | rw | 0x0 | Number of clock cycles |

#### STREAM2_FCC_CFG (0x320)

Stream Frame Capture Control configuration.

Used to specify the frame count value when when the CSI2RX must generate interrupts

FCC_START and FCC_STOP.

This register must only be modified when the corresponding frame count enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_count_stop | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will stop outputting data on the pixel interface. [0x0000 will be continuous frames.] |
| 15:0 | frame_count_start | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will start outputting data on the pixel interface. [0x0000 will be the current frame.] |

#### STREAM2_FCC_CTRL (0x324)

Stream Frame Capture Counter control.

Used to enable / disable the FCC and specify which virtual channel it

should operate on.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_counter | r | 0x0 | Current Frame number being processed |
| 15:5 | reserved | r | 0x0 | Reserved |
| 4:1 | fcc_vc | rw | 0x0 | Indicates which VC should be used to generate FCC interrupts. This value must not change while fcc_en is enabled |
| 0 | fcc_en | rw | 0x0 | Frame Capture Counter enable. |

#### STREAM2_FIFO_FILL_LVL (0x328)

Stream FIFO fill level monitor.  This can operate in 2 modes:

1 - Monitor peak fill level until the stream is stopped.

2 - Monitor peak fill level when the first FIFO read is made during a frame.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:14 | reserved | r | 0x0 | Reserved |
| 13:12 | mode | rw | 0x0 | 00 -> Fill level detection disabled 01 -> Mode 1 10 -> Mode 2 11 -> Reserved |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:0 | count | r | 0x0 | Peak fill level of FIFO. |

#### STREAM3_CTRL (0x400)

CSI2RX Stream Data output datapath control.

Start and Stop commands are independent for each output with the exception of pixel outputs that can never be

enabled together. If a pixel output is started while the other is already running,

the start command will be ignored. If both pixel outputs are enabled in a single register access,

then both start commands are ignored and no pixel output is started.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:5 | reserved | r | 0x0 | Reserved |
| 4 | soft_rst | rw1tc | 0x0 | Writing 1'b1 will apply a synchronous soft reset of this stream registers/FIFO |
| 3 | reserved | r | 0x0 | Reserved |
| 2 | abort | w | 0x0 | Writing 1 this register will cause the csi2rx to stop streaming on the corresponding output immediately. This may corrupt the output protocol. stream_abort_irq is generated on completion of the abort operation. |
| 1 | stop | w | 0x0 | Writing 1 in this register will cause csi2rx to stop streaming on the corresponding output at the end of the current frame. If the command is issued during frame blanking, then the datapath will immediately stop streaming data on that output. stream_stop_irq is generated on completion of the stop operation. |
| 0 | start | w | 0x0 | Writing 1 in this register enables the corresponding datapath output. It will start streaming data at the start of the next frame that complies with the output configuration. stream_status[31] running indicates when data stream is enabled. |

#### STREAM3_STATUS (0x404)

CSI2 Slave Controller Status. Contains useful debug information such as

FSM states.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | running | r | 0x0 | The Stream is enabled |
| 30:9 | reserved | r | 0x0 | Reserved |
| 8 | ready_state | r | X | Indicates the state of the pushback signal pixel_ready_if for this stream |
| 7:4 | stream_fsm | r | 0x0 | Output to Stream FSM states: 0x0: STREAM_IDLE 0x1: STREAM_WAIT_CTRL_DATA // Expecting control data next 0x2: STREAM_CTRL // Check contents of Ctrl packet and extract header information 0x3: STREAM_DATA // Pixel stream pixel data unpacking 0x4: STREAM_CONV_PIX_NB // 1st cycle delay for byte2pixel conversion 0x5: STREAM_CONV_PIX_NB_EXT // 2nd cycle delay for byte2pixel conversion 0x6: STREAM_DATA_START // Assert Hsync 0x7: STREAM_DATA_END // De-assert Hysnc 0x8: STREAM_FILL_WAIT // Elastic Buffer cfg - wait until fill level is reached 0x9: STREAM_STOP // Stop at the end of Frame - used to set irq 0xA: STREAM_WAIT_CRC // Wait until CRC check has completed - Full Line cfg 0xB: STREAM_WAIT_PKT_DONE // Wait for CRC to complete before proceeding 0xC: STREAM_PKT_DONE // Packet complete - no error conditions after this point 0xD: STREAM_NULL // NULL pkt received 0xE: STREAM_FLUSH // Flush due to CRC error |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1:0 | protocol_fsm | r | 0x0 | Input to Stream FSM states: 0x0: PROT_IDLE 0x1: PROT_WAIT_CTRL 0x2: PROT_CTRL 0x3: PROT_DATA |

#### STREAM3_DATA_CFG (0x408)

Secondary CSI2 Slave Controller Data outputs configuration.

This register is used to configure the data types and virtual channels

are processed and output by this stream.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | vc_select | rw | 0x0 | Selection of Virtual Channels to be processed: Default '0' -> All Virtual Channels are processed vc_select0[16] -> Virtual Channel Select 0 is processed vc_select1[17] -> Virtual Channel Select 1 is processed vc_select2[18] -> Virtual Channel Select 2 is processed vc_select3[19] -> Virtual Channel Select 3 is processed vc_select4[20] -> Virtual Channel Select 4 is processed vc_select5[21] -> Virtual Channel Select 5 is processed vc_select6[22] -> Virtual Channel Select 6 is processed vc_select7[23] -> Virtual Channel Select 7 is processed vc_select8[24] -> Virtual Channel Select 8 is processed vc_select9[25] -> Virtual Channel Select 9 is processed vc_select10[26] -> Virtual Channel Select 10 is processed vc_select11[27] -> Virtual Channel Select 11 is processed vc_select12[28] -> Virtual Channel Select 12 is processed vc_select13[29] -> Virtual Channel Select 13 is processed vc_select14[30] -> Virtual Channel Select 14 is processed vc_select15[31] -> Virtual Channel Select 15 is processed |
| 15 | enable_dt1 | rw | 0x0 | Enable processing of dt1 |
| 14 | reserved | r | 0x0 | Reserved |
| 13:8 | datatype_select1 | rw | 0x0 | Second data type format that this stream will process |
| 7 | enable_dt0 | rw | 0x0 | Enable processing of dt0 |
| 6 | reserved | r | 0x0 | Reserved |
| 5:0 | datatype_select0 | rw | 0x0 | First data type format that this stream will process |

#### STREAM3_CFG (0x40c)

Primary CSI2 Slave Controller Data pixel outputs configuration.

This register is used to configure the output mode.

It is also used to set up some Stream FIFO related settings.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | fifo_fill | rw | 0x0 | Set the FIFO_FILL_LEVEL which is used to hold data in the FIFO until this level is reached before allow data to be pulled. This setting is only used when fifo_mode is set for Large Buffer operation |
| 15 | reserved | r | 0x0 | Reserved |
| 14:12 | bpp_bypass | rw | 0x0 | Force unpacking of any Data type as selected RAW type. 0 - No bypass 1 - unpack as RAW6 2 - unpack as RAW7 3 - unpack as RAW8 4 - unpack as RAW10 5 - unpack as RAW12 6 - unpack as RAW14 7 - unpack as RAW16 |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:8 | fifo_mode | rw | 0x0 | Stream FIFO configuration, which must be set in accordance to FIFO sizing, flow control and the relationship between the link and pixel interface data rates. Refer to Use Case descriptions for further guidance on FIFO sizing and valid stream configuration options. 00: Full Line Buffer. Hold data in FIFO until CRC check completes. 01: Large Buffer [Fill Level Controlled]. Hold data in FIFO until FIFO_FILL_LEVEL is reached. 1x: Short Buffer. When pixel output data rate can match link data rate, a small buffer can be used to accommodate CDC, pixel data packing, and data rate matching |
| 7:6 | reserved | r | 0x0 | Reserved |
| 5:4 | num_pixels | rw | 0x0 | Number of pixels to output from the stream. Valid values are 1, 2, 4 and 8. The width of the pixel interface and the bits per pixel for the selected datatype will determine how many pixels can be output in a single cycle. Default will be 1 pixel per clock. 00 -> 1 pixel per clock 01 -> 2 pixels per clock 10 -> 4 pixels per clock 11 -> 8 pixels per clock [Reserved] |
| 3:2 | reserved | r | 0x0 | Reserved |
| 1 | ls_le_mode | rw | 0x0 | Enable LS/LE control of HYSNC_VALID output. By default, LS and LE short packets are not required and HYSC_VALID will be generated from the start and end of payload data. |
| 0 | interface_mode | rw | 0x0 | Select the output configuration. Pixel = 0 [default] Packed = 1 |

#### STREAM3_MONITOR_CTRL (0x410)

Stream Monitor configuration.

This register is used to configure the CSI2RX Monitors:

Programmable Frame monitor to trigger an event if a truncated frame is detected,

Programmable Timer to trigger an event based on a clock cycle counter after a frame start

or frame end,

Programmable line/byte counters to trigger an event at a specific byte in a line.

This register is used to enable/disable CSI2RX programmable interrupt, select the virtual

channel for each programmable IT and select the point which will trigger the event.

This register should not be modified while the data path is enabled, nor should any of

settings be changed when the respective monitor/counter/timer is enabled.

In these cases, the behaviour is unpredictable.

Hard reset value is 0x00000000, all interrupt generators disabled on Virtual Channel 0.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_length | rw | 0x0 | Indicates the frame length in lines to detect truncated frames. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. 0x0000 means truncated frame detection feature disabled |
| 15 | frame_mon_en | rw | 0x0 | Enables monitor. This bit must only be set high after the frame_mon_vc and frame_length have been set. |
| 14:11 | frame_mon_vc | rw | 0x0 | Indicates virtual channel for monitor. This value must not change while monitor is enabled, i.e. it must only be changed when the frame_mon_en bit is low. |
| 10 | timer_eof | rw | 0x0 | Select the starting point of the timer: 0x0: Start of Frame event on selected virtual channel 0x1: End of Frame event on selected virtual channel. This value must not change while timer_en is enabled |
| 9 | timer_en | rw | 0x0 | Enables timer based interrupt. This bit must only be set high after the timer_eof and timer_vc have been set. |
| 8:5 | timer_vc | rw | 0x0 | Indicates which VC should be used to generate timer based interrupt. This value must not change while timer_en is enabled. |
| 4 | lb_en | rw | 0x0 | Enables line/byte counter. This bit must only be set high after the lb_vc, line_count and byte_count have been set. |
| 3:0 | lb_vc | rw | 0x0 | Indicates which VC should be used to generate line/byte counter interrupt. This value must not change while lb_en is enabled |

#### STREAM3_MONITOR_FRAME (0x414)

Stream Monitor Frame.

Used to observe the current frame number and packet size on monitored virtual channels.

These values are extracted by the CSI2RX from the last frame start short packet data field.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | packet_size | r | 0x0 | Size of the current payload |
| 15:0 | nb | r | 0x0 | Number of the last frame processed. |

#### STREAM3_MONITOR_LB (0x418)

Stream Monitor Line.

Used to specify the byte and line numbers that will generate an interrupt.

This register must only be modified when the corresponding line/byte enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | line_count | rw | 0x0 | Indicates the line number to generate an interrupt. [First line of a Frame is line number 0] |
| 15:0 | byte_count | rw | 0x0 | Indicates the byte number of the line to generate an interrupt. [First byte of a line is byte number 0] |

#### STREAM3_TIMER (0x41c)

Stream Timer.

Used to specify the number of clock cycles until the interrupt is triggered after frame

start or frame end.

This register must only be modified when the corresponding timer enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:25 | reserved | r | 0x0 | Reserved |
| 24:0 | count | rw | 0x0 | Number of clock cycles |

#### STREAM3_FCC_CFG (0x420)

Stream Frame Capture Control configuration.

Used to specify the frame count value when when the CSI2RX must generate interrupts

FCC_START and FCC_STOP.

This register must only be modified when the corresponding frame count enable is disabled.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_count_stop | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will stop outputting data on the pixel interface. [0x0000 will be continuous frames.] |
| 15:0 | frame_count_start | rw | 0x0 | Indicates the frame number on which the interrupt should be generated and the stream will start outputting data on the pixel interface. [0x0000 will be the current frame.] |

#### STREAM3_FCC_CTRL (0x424)

Stream Frame Capture Counter control.

Used to enable / disable the FCC and specify which virtual channel it

should operate on.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | frame_counter | r | 0x0 | Current Frame number being processed |
| 15:5 | reserved | r | 0x0 | Reserved |
| 4:1 | fcc_vc | rw | 0x0 | Indicates which VC should be used to generate FCC interrupts. This value must not change while fcc_en is enabled |
| 0 | fcc_en | rw | 0x0 | Frame Capture Counter enable. |

#### STREAM3_FIFO_FILL_LVL (0x428)

Stream FIFO fill level monitor.  This can operate in 2 modes:

1 - Monitor peak fill level until the stream is stopped.

2 - Monitor peak fill level when the first FIFO read is made during a frame.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:14 | reserved | r | 0x0 | Reserved |
| 13:12 | mode | rw | 0x0 | 00 -> Fill level detection disabled 01 -> Mode 1 10 -> Mode 2 11 -> Reserved |
| 11:10 | reserved | r | 0x0 | Reserved |
| 9:0 | count | r | 0x0 | Peak fill level of FIFO. |

#### ASF_INT_STATUS (0x900)

ASF Interrupt Status Register. This register indicates the source of ASF interrupts. The corresponding bit in the mask register must be clear for a bit to be set. If any bit is set in this register the asf_int_fatal or asf_int_nonfatal signal will be asserted. Writing to either raw or masked status registers, clear both registers. For test purposes, trigger signal interrupt event by writing to the ASF interrupt status test register.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err | rw1tc | 0x0 | Integrity error interrupt |
| 5 | asf_protocol_err | rw1tc | 0x0 | Protocol error interrupt |
| 4 | asf_trans_to_err | rw1tc | 0x0 | Transaction timeouts error interrupt |
| 3 | asf_csr_err | rw1tc | 0x0 | Configuration and status registers error interrupt |
| 2 | asf_dap_err | rw1tc | 0x0 | Data and address paths parity error interrupt |
| 1 | asf_sram_uncorr_err | rw1tc | 0x0 | SRAM uncorrectable error interrupt |
| 0 | asf_sram_corr_err | rw1tc | 0x0 | SRAM correctable error interrupt |

#### ASF_INT_RAW_STATUS (0x904)

ASF Interrupt Raw Status Register. A bit set in this raw register indicates a source of ASF fault in the corresponding feature. Writing to either raw or masked status registers, clear both registers. For test purposes, trigger signal interrupt event by writing to the ASF interrupt status test register.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err | rw1tc | 0x0 | Integrity error interrupt |
| 5 | asf_protocol_err | rw1tc | 0x0 | Protocol error interrupt |
| 4 | asf_trans_to_err | rw1tc | 0x0 | Transaction timeouts error interrupt |
| 3 | asf_csr_err | rw1tc | 0x0 | Configuration and status registers error interrupt |
| 2 | asf_dap_err | rw1tc | 0x0 | Data and address paths parity error interrupt |
| 1 | asf_sram_uncorr_err | rw1tc | 0x0 | SRAM uncorrectable error interrupt |
| 0 | asf_sram_corr_err | rw1tc | 0x0 | SRAM correctable error interrupt |

#### ASF_INT_MASK (0x908)

The ASF interrupt mask register indicating which interrupt bits in the ASF interrupt status register are masked. All bits are set at reset. Clear the individual bit to enable the corresponding interrupt.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err_mask | rw | 0x1 | Mask bit for integrity error interrupt |
| 5 | asf_protocol_err_mask | rw | 0x1 | Mask bit for protocol error interrupt. |
| 4 | asf_trans_to_err_mask | rw | 0x1 | Mask bit for transaction timeouts error interrupt. |
| 3 | asf_csr_err_mask | rw | 0x1 | Mask bit for configuration and status registers error interrupt. |
| 2 | asf_dap_err_mask | rw | 0x1 | Mask bit for data and address paths parity error interrupt. |
| 1 | asf_sram_uncorr_err_mask | rw | 0x1 | Mask bit for SRAM uncorrectable error interrupt. |
| 0 | asf_sram_corr_err_mask | rw | 0x1 | Mask bit for SRAM correctable error interrupt. |

#### ASF_INT_TEST (0x90c)

The ASF interrupt test register emulate hardware even. Write one to individual bit to trigger single event in (masked and raw) status registers according to mask and will generate interrupt accordingly.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err_test | w | 0x0 | Test bit for integrity error interrupt |
| 5 | asf_protocol_err_test | w | 0x0 | Test bit for protocol error interrupt. |
| 4 | asf_trans_to_err_test | w | 0x0 | Test bit for transaction timeouts error interrupt. |
| 3 | asf_csr_err_test | w | 0x0 | Test bit for configuration and status registers error interrupt. |
| 2 | asf_dap_err_test | w | 0x0 | Test bit for data and address paths parity error interrupt. |
| 1 | asf_sram_uncorr_err_test | w | 0x0 | Test bit for SRAM uncorrectable error interrupt. |
| 0 | asf_sram_corr_err_test | w | 0x0 | Test bit for SRAM correctable error interrupt. |

#### ASF_FATAL_NONFATAL_SELECT (0x910)

The fatal or non-fatal interrupt register selects whether a fatal (asf_int_fatal) or non-fatal (asf_int_nonfatal) interrupt is triggered. If the bit of the event will be set to one then fatal interrupt (asf_int_fatal) will be triggered. Otherwise the non-fatal interrupt (asf_int_nonfatal) will be triggered.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:7 | reserved_31_7 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 6 | asf_integrity_err | rw | 0x1 | Enable integrity error interrupt as fatal |
| 5 | asf_protocol_err | rw | 0x1 | Enable protocol error interrupt as fatal. |
| 4 | asf_trans_to_err | rw | 0x1 | Enable transaction timeouts error interrupt as fatal. |
| 3 | asf_csr_err | rw | 0x1 | Enable configuration and status registers error interrupt as fatal. |
| 2 | asf_dap_err | rw | 0x1 | Enable data and address paths parity error interrupt as fatal. |
| 1 | asf_sram_uncorr_err | rw | 0x1 | Enable SRAM uncorrectable error interrupt as fatal. |
| 0 | asf_sram_corr_err | rw | 0x1 | Enable SRAM correctable error interrupt as fatal. |

#### ASF_SRAM_CORR_FAULT_STATUS (0x920)

Status register for SRAM correctable fault. These fields are updated whenever asf_sram_corr_fault input is active.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | asf_sram_corr_fault_inst | r | 0x0 | Last SRAM instance that generated fault. |
| 23:0 | asf_sram_corr_fault_addr | r | 0x0 | Last SRAM address that generated fault. |

#### ASF_SRAM_UNCORR_FAULT_STATUS (0x924)

Status register for SRAM uncorrectable fault. These fields are updated whenever asf_sram_uncorr_fault input is active.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:24 | asf_sram_uncorr_fault_inst | r | 0x0 | Last SRAM instance that generated fault. |
| 23:0 | asf_sram_uncorr_fault_addr | r | 0x0 | Last SRAM address that generated fault. |

#### ASF_SRAM_FAULT_STATS (0x928)

Statistics register for SRAM faults. Note that this register clears when software writes to any field.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | reserved_31_16 | r | 0x0 | Reserved, read as 0, ignored on write. |
| 15:0 | asf_sram_fault_corr_stats | rw1tc | 0x0 | Count of number of correctable errors if implemented. Count value will saturate at 0xffff. |

#### ASF_TRANS_TO_CTRL (0x930)

Control register to configure the ASF transaction timeout monitors.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31 | asf_trans_to_en | rw | 0x0 | Enable transaction timeout monitoring. |
| 30:16 | reserved | r | 0x0 | Reserved |
| 15:0 | asf_trans_to_ctrl | rw | 0x0 | Timer value to use for transaction timeout monitor. |

#### ASF_TRANS_TO_FAULT_MASK (0x934)

Control register to mask out ASF transaction timeout faults from triggering interrupts. On reset, all bits are set to mask out all sources. Clear the corresponding bit to enable the interrupt source. The width of this field is parameterisable and the bit definitions are implementation specific.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | asf_trans_to_fault_0_mask | rw | 0x1 | Mask register for each ASF transaction timeout fault source. |

#### ASF_TRANS_TO_FAULT_STATUS (0x938)

Status register for transaction timeouts fault. If a fault occurs the revelant status bit will be set to 1. Each bit can be cleared by software writing 1 to each bit.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | asf_trans_to_fault_0_status | rw1tc | 0x0 | Status bits for transaction timeouts faults. |

#### ASF_PROTOCOL_FAULT_MASK (0x940)

Control register to mask out ASF Protocol faults from triggering interrupts. On reset, all bits are set to mask out all sources. Clear the corresponding bit to enable the interrupt source. The width of this field is parameterisable and the bit definitions are implementation specific.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:14 | reserved | r | 0x0 | Reserved |
| 13 | asf_protocol_fault_13_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 12 | asf_protocol_fault_12_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 11 | asf_protocol_fault_11_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 10 | asf_protocol_fault_10_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 9 | asf_protocol_fault_9_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 8 | asf_protocol_fault_8_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 7 | asf_protocol_fault_7_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 6 | asf_protocol_fault_6_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 5 | asf_protocol_fault_5_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 4 | asf_protocol_fault_4_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 3 | asf_protocol_fault_3_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 2 | asf_protocol_fault_2_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 1 | asf_protocol_fault_1_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |
| 0 | asf_protocol_fault_0_mask | rw | 0x1 | Mask register for each ASF protocol fault source. |

#### ASF_PROTOCOL_FAULT_STATUS (0x944)

Status register for protocol faults. If a fault occurs the revelant status bit will be set to 1. Each bit can be cleared by software writing 1 to each bit

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:14 | reserved | r | 0x0 | Reserved |
| 13 | asf_protocol_fault_13_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 12 | asf_protocol_fault_12_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 11 | asf_protocol_fault_11_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 10 | asf_protocol_fault_10_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 9 | asf_protocol_fault_9_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 8 | asf_protocol_fault_8_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 7 | asf_protocol_fault_7_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 6 | asf_protocol_fault_6_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 5 | asf_protocol_fault_5_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 4 | asf_protocol_fault_4_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 3 | asf_protocol_fault_3_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 2 | asf_protocol_fault_2_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 1 | asf_protocol_fault_1_status | rw1tc | 0x0 | Status bits for protocol faults. |
| 0 | asf_protocol_fault_0_status | rw1tc | 0x0 | Status bits for protocol faults. |

#### ID_PROD_VER (0xffc)

This register is hard-coded in order to allow software to identify the product and its

release version.  The product ID will be fixed for all versions, while the version will

be updated as new releases for the CSI2RX product are made.

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:16 | product_id | r | 0x5022 | Product Identification Number [IP5022/IP5022A]. |
| 15:0 | version_id | r | 0x200 | Product Version Number [R200]. |

## MMR Region: cp_intd__intd_cfg__intd_cfg

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | REVISION |  |
| 0x4 | CONTROL |  |
| 0x10 | EOI_REG |  |
| 0x14 | INTR_VECTOR_REG |  |
| 0x100 | ENABLE_REG_LEVEL_0 |  |
| 0x104 | ENABLE_REG_PULSE_0 |  |
| 0x300 | ENABLE_CLR_REG_LEVEL_0 |  |
| 0x304 | ENABLE_CLR_REG_PULSE_0 |  |
| 0x500 | STATUS_REG_LEVEL_0 |  |
| 0x504 | STATUS_REG_PULSE_0 |  |
| 0x700 | STATUS_CLR_REG_LEVEL_0 |  |
| 0x704 | STATUS_CLR_REG_PULSE_0 |  |
| 0xa80 | INTR_VECTOR_REG_LEVEL |  |
| 0xa84 | INTR_VECTOR_REG_PULSE |  |

### Register Definitions

#### REVISION (0x0)

Revision Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:30 | SCHEME | r | 0x1 | Scheme |
| 29:28 | BU | r | 0x2 | BU |
| 27:16 | FUNCTION | r | 0x690 | Module ID |
| 15:11 | RTLVER | r | 0x14 | RTL revisions |
| 10:8 | MAJREV | r | 0x2 | Major revision |
| 7:6 | CUSTOM | r | 0x0 | Custom revision |
| 5:0 | MINREV | r | 0x0 | Minor revision |

#### EOI_REG (0x10)

End of Interrupt Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:8 | reserved | r | 0x0 | Reserved |
| 7:0 | eoi_vector | rw | 0x0 | End of Interrupt Vector |

#### INTR_VECTOR_REG (0x14)

Interrupt Vector Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | intr_vector | r | 0x0 | Interrupt Vector Register |

#### ENABLE_REG_LEVEL_0 (0x100)

Enable Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | enable_level_en_int_vp1_errInLnFrm | rw1ts | 0x0 | Enable Set for level_en_int_vp1_errInLnFrm |
| 1 | enable_level_en_int_vp0_errInLnFrm | rw1ts | 0x0 | Enable Set for level_en_int_vp0_errInLnFrm |
| 0 | enable_level_en_fifo_overflow | rw1ts | 0x0 | Enable Set for level_en_fifo_overflow |

#### ENABLE_REG_PULSE_0 (0x104)

Enable Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | enable_pulse_en_int_vp1_errInLnFrm | rw1ts | 0x0 | Enable Set for pulse_en_int_vp1_errInLnFrm |
| 1 | enable_pulse_en_int_vp0_errInLnFrm | rw1ts | 0x0 | Enable Set for pulse_en_int_vp0_errInLnFrm |
| 0 | enable_pulse_en_fifo_overflow | rw1ts | 0x0 | Enable Set for pulse_en_fifo_overflow |

#### ENABLE_CLR_REG_LEVEL_0 (0x300)

Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | enable_level_en_int_vp1_errInLnFrm_clr | rw1tc | 0x0 | Enable Clear for level_en_int_vp1_errInLnFrm |
| 1 | enable_level_en_int_vp0_errInLnFrm_clr | rw1tc | 0x0 | Enable Clear for level_en_int_vp0_errInLnFrm |
| 0 | enable_level_en_fifo_overflow_clr | rw1tc | 0x0 | Enable Clear for level_en_fifo_overflow |

#### ENABLE_CLR_REG_PULSE_0 (0x304)

Enable Clear Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | enable_pulse_en_int_vp1_errInLnFrm_clr | rw1tc | 0x0 | Enable Clear for pulse_en_int_vp1_errInLnFrm |
| 1 | enable_pulse_en_int_vp0_errInLnFrm_clr | rw1tc | 0x0 | Enable Clear for pulse_en_int_vp0_errInLnFrm |
| 0 | enable_pulse_en_fifo_overflow_clr | rw1tc | 0x0 | Enable Clear for pulse_en_fifo_overflow |

#### STATUS_REG_LEVEL_0 (0x500)

Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | status_level_int_vp1_errInLnFrm | rw1ts | 0x0 | Status ,write 1 to set, for level_en_int_vp1_errInLnFrm |
| 1 | status_level_int_vp0_errInLnFrm | rw1ts | 0x0 | Status ,write 1 to set, for level_en_int_vp0_errInLnFrm |
| 0 | status_level_fifo_overflow | rw1ts | 0x0 | Status ,write 1 to set, for level_en_fifo_overflow |

#### STATUS_REG_PULSE_0 (0x504)

Status Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | status_pulse_int_vp1_errInLnFrm | rw1ts | 0x0 | Status ,write 1 to set, for pulse_en_int_vp1_errInLnFrm |
| 1 | status_pulse_int_vp0_errInLnFrm | rw1ts | 0x0 | Status ,write 1 to set, for pulse_en_int_vp0_errInLnFrm |
| 0 | status_pulse_fifo_overflow | rw1ts | 0x0 | Status ,write 1 to set, for pulse_en_fifo_overflow |

#### STATUS_CLR_REG_LEVEL_0 (0x700)

Status Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | status_level_int_vp1_errInLnFrm_clr | rw1tc | 0x0 | Status ,write 1 to clear, for level_en_int_vp1_errInLnFrm |
| 1 | status_level_int_vp0_errInLnFrm_clr | rw1tc | 0x0 | Status ,write 1 to clear, for level_en_int_vp0_errInLnFrm |
| 0 | status_level_fifo_overflow_clr | rw1tc | 0x0 | Status ,write 1 to clear, for level_en_fifo_overflow |

#### STATUS_CLR_REG_PULSE_0 (0x704)

Status Clear Register 1

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:3 | reserved | r | 0x0 | Reserved |
| 2 | status_pulse_int_vp1_errInLnFrm_clr | rw1tc | 0x0 | Status ,write 1 to clear, for pulse_en_int_vp1_errInLnFrm |
| 1 | status_pulse_int_vp0_errInLnFrm_clr | rw1tc | 0x0 | Status ,write 1 to clear, for pulse_en_int_vp0_errInLnFrm |
| 0 | status_pulse_fifo_overflow_clr | rw1tc | 0x0 | Status ,write 1 to clear, for pulse_en_fifo_overflow |

#### INTR_VECTOR_REG_LEVEL (0xa80)

Interrupt Vector for level

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | intr_vector_level | r | 0x0 | Interrupt Vector |

#### INTR_VECTOR_REG_PULSE (0xa84)

Interrupt Vector for pulse

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:0 | intr_vector_pulse | r | 0x0 | Interrupt Vector |

## MMR Region: ecc_aggr__cfg__regs

### Register Map

| Address Offset | Register Mnemonic | Register Name |
| --- | --- | --- |
| 0x0 | AGGR_REVISION | Aggregator Revision Register |
| 0x8 | ECC_VECTOR | ECC Vector Register |
| 0xc | MISC_STATUS | Misc Status |
| 0x3c | SEC_EOI_REG | EOI Register |
| 0x40 | SEC_STATUS_REG0 | Interrupt Status Register 0 |
| 0x80 | SEC_ENABLE_SET_REG0 | Interrupt Enable Set Register 0 |
| 0xc0 | SEC_ENABLE_CLR_REG0 | Interrupt Enable Clear Register 0 |
| 0x13c | DED_EOI_REG | EOI Register |
| 0x140 | DED_STATUS_REG0 | Interrupt Status Register 0 |
| 0x180 | DED_ENABLE_SET_REG0 | Interrupt Enable Set Register 0 |
| 0x1c0 | DED_ENABLE_CLR_REG0 | Interrupt Enable Clear Register 0 |
| 0x200 | AGGR_ENABLE_SET | AGGR interrupt enable set Register |
| 0x204 | AGGR_ENABLE_CLR | AGGR interrupt enable clear Register |
| 0x208 | AGGR_STATUS_SET | AGGR interrupt status set Register |
| 0x20c | AGGR_STATUS_CLR | AGGR interrupt status clear Register |

### Register Definitions

#### AGGR_REVISION (Aggregator Revision Register), (0x0)

Revision parameters

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:30 | scheme | r | 0x1 | Scheme |
| 29:28 | bu | r | 0x2 | bu |
| 27:16 | module_id | r | 0x6a0 | Module ID |
| 15:11 | revrtl | r | 0x1c | RTL version |
| 10:8 | revmaj | r | 0x2 | Major version |
| 7:6 | custom | r | 0x0 | Custom version |
| 5:0 | revmin | r | 0x0 | Minor version |

#### ECC_VECTOR (ECC Vector Register), (0x8)

ECC Vector Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:25 | reserved | r | 0x0 | Reserved |
| 24 | rd_svbus_done | rw1tc | 0x0 | Status to indicate if read on serial VBUS is complete, write of any value will clear this bit. |
| 23:16 | rd_svbus_address | rw | 0x0 | Read address |
| 15 | rd_svbus | rw1ts | 0x0 | Write 1 to trigger a read on the serial VBUS |
| 14:11 | reserved | r | 0x0 | Reserved |
| 10:0 | ecc_vector | rw | 0x0 | Value written to select the corresponding ECC RAM for control or status |

#### MISC_STATUS (Misc Status), (0xc)

Misc Status

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:11 | reserved | r | 0x0 | Reserved |
| 10:0 | num_rams | r | 0x6 | Indicates the number of RAMS serviced by the ECC aggregator |

#### SEC_EOI_REG (EOI Register), (0x3c)

EOI Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | eoi_wr | rw1ts | 0x0 | EOI Register |

#### SEC_STATUS_REG0 (Interrupt Status Register 0), (0x40)

Interrupt Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | psil_fifo_ramecc_pend | rw1ts | 0x0 | Interrupt Pending Status for psil_fifo_ramecc_pend |
| 0 | busecc_pend | rw1ts | 0x0 | Interrupt Pending Status for busecc_pend |

#### SEC_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x80)

Interrupt Enable Set Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | psil_fifo_ramecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for psil_fifo_ramecc_pend |
| 0 | busecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for busecc_pend |

#### SEC_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0xc0)

Interrupt Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | psil_fifo_ramecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for psil_fifo_ramecc_pend |
| 0 | busecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for busecc_pend |

#### DED_EOI_REG (EOI Register), (0x13c)

EOI Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:1 | reserved | r | 0x0 | Reserved |
| 0 | eoi_wr | rw1ts | 0x0 | EOI Register |

#### DED_STATUS_REG0 (Interrupt Status Register 0), (0x140)

Interrupt Status Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | psil_fifo_ramecc_pend | rw1ts | 0x0 | Interrupt Pending Status for psil_fifo_ramecc_pend |
| 0 | busecc_pend | rw1ts | 0x0 | Interrupt Pending Status for busecc_pend |

#### DED_ENABLE_SET_REG0 (Interrupt Enable Set Register 0), (0x180)

Interrupt Enable Set Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | psil_fifo_ramecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for psil_fifo_ramecc_pend |
| 0 | busecc_enable_set | rw1ts | 0x0 | Interrupt Enable Set Register for busecc_pend |

#### DED_ENABLE_CLR_REG0 (Interrupt Enable Clear Register 0), (0x1c0)

Interrupt Enable Clear Register 0

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | psil_fifo_ramecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for psil_fifo_ramecc_pend |
| 0 | busecc_enable_clr | rw1tc | 0x0 | Interrupt Enable Clear Register for busecc_pend |

#### AGGR_ENABLE_SET (AGGR interrupt enable set Register), (0x200)

AGGR interrupt enable set Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | timeout | rw1ts | 0x0 | interrupt enable set for svbus timeout errors |
| 0 | parity | rw1ts | 0x0 | interrupt enable set for parity errors |

#### AGGR_ENABLE_CLR (AGGR interrupt enable clear Register), (0x204)

AGGR interrupt enable clear Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:2 | reserved | r | 0x0 | Reserved |
| 1 | timeout | rw1tc | 0x0 | interrupt enable clear for svbus timeout errors |
| 0 | parity | rw1tc | 0x0 | interrupt enable clear for parity errors |

#### AGGR_STATUS_SET (AGGR interrupt status set Register), (0x208)

AGGR interrupt status set Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3:2 | timeout | rwi | 0x0 | interrupt status set for svbus timeout errors |
| 1:0 | parity | rwi | 0x0 | interrupt status set for parity errors |

#### AGGR_STATUS_CLR (AGGR interrupt status clear Register), (0x20c)

AGGR interrupt status clear Register

| Bit | Name | Type | Reset | Description |
| --- | --- | --- | --- | --- |
| 31:4 | reserved | r | 0x0 | Reserved |
| 3:2 | timeout | rwd | 0x0 | interrupt status clear for svbus timeout errors |
| 1:0 | parity | rwd | 0x0 | interrupt status clear for parity errors |

# Submodule Detail

## CSI Protocol Details

CSI provide a series of short command packets to frame pixel data.  Optionally, pixel data can be either  frame based or line based.  In Line mode, both frame and line control packets frame data packets and lines are whole data packets.  In Frame mode, only frame control packets are provided and data packets are whole frames.  In any mode, CSI will burst an entire packet prior to context switching to another channel.  There is no back-pressure or rate control of data transfer so the receiver needs to consume at full rate.  The FS/LS control packets do contain and frame/line index, however the Cadence CSI_RX circuit does not provide this information to the DMA/VP circuits at the SOC infrastructure side.  The Line/Frame data packets contain a length field in their packet header, however the Cadence CSI_RX does not supply this information to the DMA/VP layer either.

Bit errors in the large data packets are acceptable.  The Data Packet header is ECC protected and contains the packet length.  In a worst cases scenario, some pixel data is corrupted which is detected, but not corrected by a trailing CRC.  Vision systems are robust to this level of pixel noise and most importantly, there is no way the length of the packet can be corrupted.

Between each packet, CSI Lanes go into a low power state.  This makes for a robust protocol for bit alignment, but also waists some BW between packets.   Because of this, the CSI provided LE/FE occurs many clock cycles after the last data.

CSI is a multi-lane serial interface with a statistical probability of random bit errors.  The header of each packet is ECC protected for purposes of correcting a single bit error or detecting a dual bit error.  Statistically, the probability of a dual bit error in a packet header is extremely low; however when this dual bit error does occur.  Information is lost.  The CSI_RX circuit and SOC in general are robust of these unlikely error conditions.

(Strong assumption is that Cadence CSI-RX circuit will report all CSI bit errors and protocol violations)

In gross failure conditions where a cable is hot swapped or if a connection is corrupted in other ways, the CSI_RX/SOC will not operate recover gracefully.

### CSI Statistical probability of bit errors

- Assumptions:

  - 10-12  DPHY worst BER (prob of a single bit being an error)

    - 1 bit error per 100sec

  - 10+10 bits per sec MAX

  - 16 virtual streams MAX

  - 32 streams Max  (pessimistic)

  - 32bit per short packet or long packet header (EccHdr for short)

  - 1000 lines per frame (estimate/pessimistic)

  - 30 frames per sec (estimate)

- Length of time between bit errors

  - 1/100 error per sec = 10+10 * 10-12  -OR-

  - 100 seconds between bit errors

- Number of EccHdr per second

  - 960 vs/ve	=16*30*2

  - 960,000 hs/he =16*1000*30*2

  - 960,000 longPkt =32*1000*30*1

  - 1,920,960 Number of EccHdr per sec = 960 + 960,000 + 960,000

- Probability of dual bit error for a given 32bit header

  - 4.96 * 10-22 = 496 *10-12 *10-12  (496 = 31+30+29+…+1)

- Probability of dual bit error per second

  - 9.53 * 10-16 = 4.96 * 10-22 * 1, 920,960

  - 1.05*10+15 sec between EccHdr dual bit err

  - ~10+9 year between EccHdr dual bit err

### CSI dual bit error conditions

- Corrupted/missing FS short packet

  - Recoverable: any next packet starts operation

- Corrupted/missing LS short packet

  - Recoverable: next Long Data Packet used like normal

- Corrupted/missing Data long packet

  - DMA: will see 1 line too few and will simply compress out the missing line with a line gap at the DMA end.  DMA will recover to next frame boundary on next good frame

  - VP: will present 1 line too few to VISS.  VISS cannot process short frames and will use extra clock cycles to patch in a zero line.  During this patch time, VISS will back-pressure CSI which will very likely result in CAL/CSI input FIFO overflow.  This overflow condition will trigger CAL/CSI to truncate this next frame, flush all remaining frame data for that context, then start fresh on the next clean FS.

- Corrupted/Missing LE

  - LE is redundant with long packet length.  Cadence CSI_RX will trigger HorzSync.  DMA/VP will have no adverse operation

- Corrupted/Missing FE

  - DMA: UDMA_P will have already closed out frame since it’s tables will have been programmed with length information.  The TI Shim will see the next FS.  When it sees this FS, it will stall the input FIFO for 1 clock, issue an EOF, then process the next frame.

  - VP: The TI shim is programmed with frame length/line length and will count through the pixels of the packet.  The shim will issue the FE automatically.

- General

  - In all cases, either the TI shim or the Cadence CSI_RX will alert the SOC that a protocol violation occurred and optionally trigger a system event.

## DMA (PSI/PSIL) Interface

- 32 channels/contexts/Thread id’s

- CSI conversion

  - Conversion from CSI virtualChan/dataType to thread id

  - Conversion from vsync/hsync level signals to SOP/EOP/SOL/EOL

    - Vsync/hsync is per virtual channel (applies to all cntx with same virtChan)

- QOS

  - FIFO order of received data from csi interface to psil output. No priority for specific virtual channels or data types.

- It is desirable to have all same data types on csi . If not then this interface may be inefficient due to programming requirements on csi rx controller for various data types.

- 32bit-to-128 bit packer

  - Internal CSIRX clock domain crossing fifo is 32bits, DMA FIFO is 128 bits

  - Due to limitations in how csirx controller exports data there is logic to reorganize data into TI/standard formats. See memory organization section for more details.

    - Mmr configurations must be set to unpack csi formats into TI formats. See programming section for more details

  - Error handling

    - Resync frames on fifo overflow condition.

- EOL/EOF

  - The cadence ip indicates EOL or EOF after the last data is sent. Due to this the TI logic will hold the last line of data and ether a new line or EOP will force the last line out. EOL and EOP will always be in the same packet. It is possible for EOL and EOP packet to have a zero byte data.

- Line and Frame sizes.

  - The line and frame size is not known outside the cadence ip. Therefore any mismatches in system level programming cannot be known. If the dma line/frame size does not match the csi line/frame size it is assumed the dma will not result in any adverse side affects as a result of not enough data or too much data.

The DMA interface module is primarily an elasticity buffer followed by a simple interface conversion to PSI.  Last is the PSI2PSIL module, provided by the Veriperl infrastructure. TI external interface to UDMA is PSIL.

The RAM contains both the 128bit data path, plus sideband control signals.  Sideband control are:

- 5 bit channel context index

- Eol, sol, eof, sof

- Byte count in 128 packet.

### DMA framing

CDNS stream interface provides VSYNC/HSYNC.  The signals are per virtual channel where the transition from 1-to-0 or 0-to-1 represents SOF/EOF or SOL/EOL. VSYNC/HSYNC transition is usually not coincident with a valid data phase, though there are CSI use cases where it can be. Because of the nature of CSI, the VSYNC and HSYNC transactions require at least a single clk cycle and can only transition sequentially (important fact in handling of VSYNC events)

There is an uncommonly used mode of CSI where an entire frame is passed in a single packets without line breaks.  In this mode, there are no HSYNC transitions at line starts/ends.

Last there is redundant information provided with each CSI packet where the packet length is supplied in bytes.

TI solution for framing involves the concept of passing metadata through the DMA FIFO.  Anytime metadata is inserted into the FIFO, the CSIRX stream is stalled for a clk cycle.  The forms of supported Meta are:

- SOF for a given virtChan

- EOF for a given virtChan

- Packet length in bytes for a given dmaCntx

TI solution will disregard the HSYNC signal entirely.  The long packet beginning is used as SOL and the EOL will be “counted” using the packet length provided.  In the special case where packets are of FRAME length, the SOL/EOL handling is identical.

TI solution for VSYNC handling will be to create the concept of metadata FIFO entry which is indicated by a metadata bit.  The SOF or EOF VSYN information plus the virtualChan index is fed into the FIFO.  On the output of the FIFO, the SOF is saved for each context of that virtualChan type.  The very next data phase of that channel context will be marked as SOF.   Additionally a state bit per context is maintained indicating the channel is MOP (middle of packet).  Once the EOF arrives at the FIFO output, all contexts of that type virtual channel (and currently in MOP state) will receive a PSI_EOP with zero active bytes of data.

### DMA Error handling due to fifo overflow

- context cleanup

  - cycle through each context index checking if context is in MOL or MOP.

  - Close out MOP with EOP and MOL&MOP with EOL&EOP

- dropOnFloor SOP if currently in MOPstate

- dropOnFloor EOP if not currently in MOPstate

- dropOnFloor FIFO data

- EOL context if EOP and MOLstate

- After closing out all open contexts the TI logic will then wait till end of frame per virtual channel. Once a new frame starts it will then start sending out data from that new frame.

This error handling is broadly referred to as a PSI protocol enforcer.  The goal is to prevent hang of PSI/PSIL.  Unnatural packet size should not cause hang so only SOP/SOL/EOL/EOP framing is enforced.

### DMA safety

The primary concern for safety is preventing illegal protocol on the PSI interface.  It is envisioned that the primary source of internal error would be a flipped RAM bit.  The 128bit of data path is ecc protected as it contains short packet information.

The TI implementation enforces PSI protocol at the output of the RAM.  This means that all SOP/SOL are terminated with EOP/EOL.  Never issue EOx if SOx has not previously been issued and no new SOx until an EOx has been issued.  With this protocol enforcement, this circuit will not hang PSI.  And unexpected packet /line lengths are handled gracefully with the external UDMA.

## VPORT Interface

- 2 pixels wide

  - 32bit wide

- Slight protocol conversion

  - VPORT VS,VE,HS,HE supplied with data phases

  - STREAM vsync/hsync precede data phases (no VE/HE)

- VISS (consumer of data) will read data max every other clock cycle.

  - Average rate 1 pixel per clk

- Clk asynchronous to main clock, but synchronous to viss clock

- VPORT FSM forces correct protocol onto VP output

- Error conditions

  - Vsync/hsync disagree with FSM

- VISS Data Format

  - RAW16-RAW8

See VISS spec for VP protocol.

### VPORT Error Handling

The following are possible protocol violations.

- Wrong Line Length

  - Statistically, likelihood of this error occurring due to a bit error is zero.  The ECC protection on the packet header is extremely robust.

  - Programmer error is the most likely cause of this error

  - For short line from csi compared to mmr config. video port interface will send short line, will not pad out data.

  - For line exceeding mmr, terminate based on mmr and drop data till next line.

- Mismatch on frame

  - Caused by multiple CSI bit errors in a CSI long data packet.  Statistically extremely unlikely.  Programmer error is most likely cause.

  - Csi sends less lines than mmr config. Will just terminate with no padding. This will likely lead to a VE that is not lined up with HE.

  - For more csi lines than configured in mmr terminate frame based on configuration then drop extra lines till start of next frame.

- Overflow

  - Immediately closes out any open lines and frames. No padding. This could lead to VE without HE.

### Configuration parameters for CSI RX controller

| Parameter name | value | comments |
| --- | --- | --- |
| STREAM_FIFO_AD_WIDTH0 | 32'd6 | Stream 0 FIFO address width |
| STREAM_FIFO_DA_WIDTH0 | 32'd36 | Stream 0 FIFO Data width |
| STREAM_FIFO_DEPTH0 | 32'd64 | Stream 0 FIFO depth |
| STREAM_FIFO_FL_WIDTH0 | 32'd6 | Stream 0 FIFO fill level width |
| STREAM_FIFO_AD_WIDTH1 | 32'd6 | Stream 1 FIFO address width |
| STREAM_FIFO_DA_WIDTH1 | 32'd36 | Stream 1 FIFO Data width |
| STREAM_FIFO_DEPTH1 | 32'd64 | Stream 1 FIFO depth |
| STREAM_FIFO_FL_WIDTH1 | 32'd6 | Stream 1 FIFO fill level width |
| STREAM_FIFO_AD_WIDTH2 | 32'd11 | Stream 2 FIFO address width |
| STREAM_FIFO_DA_WIDTH2 | 32'd36 | Stream 2 FIFO Data width |
| STREAM_FIFO_DEPTH2 | 32'd2048 | Stream 2 FIFO depth |
| STREAM_FIFO_FL_WIDTH2 | 32'd11 | Stream 2 FIFO fill level width |
| STREAM_FIFO_AD_WIDTH3 | 32'd11 | Stream 3 FIFO address width |
| STREAM_FIFO_DA_WIDTH3 | 32'd36 | Stream 3 FIFO Data width |
| STREAM_FIFO_DEPTH3 | 32'd2048 | Stream 3 FIFO depth |
| STREAM_FIFO_FL_WIDTH3 | 32'd11 | Stream 3 FIFO fill level width |
| PIXEL_WIDTH_IF0 | 32'd32 | Stream 0 pixel if data width |
| PIXEL_WIDTH_IF1 | 32'd32 | Stream 1 pixel if data width |
| PIXEL_WIDTH_IF2 | 32'd32 | Stream 2 pixel if data width |
| PIXEL_WIDTH_IF3 | 32'd32 | Stream 3 pixel if data width |
| FULL_PSLVERR | 32'd0 | Full granularity APB slave error |
| NUM_VCS | 32'd16 | Number of sequential VCs to be supported (default (4)= VCs 0,1,2,3) |
| PHY_DATA_WIDTH | 32'd8 | width of the data from phy - 8 for DPHY 16 for CPHY |
| PHY_NUM_LANES | 32'd4 | Number of Data Lanes on phy - Max 8 for DPHY, 4 for CPHY |
| NUM_STREAMS | 32'd4 | Number of streams |
| NUM_MONITORS | 32'd4 | Number of Monitors |
| SYNC_DPHY_SYS | 1'b0 | Indicates if ppi_rx_byte_hs_clk and sys_clk are synchronous |
| SYNC_PCLK_SYS | 1'b0 | Indicates if pclk and sys_clk are synchronous |
| SYNC_PIXEL_IF0_SYS | 1'b0 | Indicates if pixel_clk_if0 and sys_clk are synchronous |
| SYNC_PIXEL_IF1_SYS | 1'b0 | Indicates if pixel_clk_if1 and sys_clk are synchronous |
| SYNC_PIXEL_IF2_SYS | 1'b0 | Indicates if pixel_clk_if2 and sys_clk are synchronous |
| SYNC_PIXEL_IF3_SYS | 1'b0 | Indicates if pixel_clk_if3 and sys_clk are synchronous |
| SYNC_PIXEL_IF0_PCLK | 1'b0 | Indicates if pixel_clk_if0 and pclk are synchronous |
| SYNC_PIXEL_IF1_PCLK | 1'b0 | Indicates if pixel_clk_if1 and pclk are synchronous |
| SYNC_PIXEL_IF2_PCLK | 1'b0 | Indicates if pixel_clk_if2 and pclk are synchronous |
| SYNC_PIXEL_IF3_PCLK | 1'b0 | Indicates if pixel_clk_if3 and pclk are synchronous |
| STREAM_IF1 | 1'b1 | Stream if1 |
| STREAM_IF2 | 1'b1 | Stream if2 |
| STREAM_IF3 | 1'b1 | Stream if3 |
| CDNS_DPHY | 1'b1 | Uses Cadence DPHY |
| ASF | 1'b1 | ASF usage/Safety used |

## DV Tap Points

SECTION II: INTEGRATION DETAILS

# Submodule details

# Integration Overview

## Parameterization Requirements

N/A  No parameterization

## Memories Embedded in the Peripheral

|  | Qty | RAM | RAM Name | Comments |
| --- | --- | --- | --- | --- |
|  | 2 |  | saculs0c4s2p2048x64m4b2w0c0p1d1r1s2z1rw10. |  |
|  | 4 |  | sadrls0c4l2p1024x45m4b8w0c0p1d1r1s2z1rw10 |  |
|  | 1 |  | saculs0c4s2p2048x24m4b2w0c0p1d1r1s2z1rw10 |  |

Table 3 : RAM

*rams associated with cadence ip have parity. All other rams have full ecc using ti ecc wrappers.

### Error Correcting Code (ECC) Support

ECC is a mechanism for providing increased system reliability (via reduction of memory soft errors) by allowing single bit errors to be detected and corrected and double bit errors to be detected.

One or more memories within this subsystem are ECC protected using an ECC Memory Wrapper which implements Single Error Correction and Double Error Detection (SECDED). Refer to the “ECC Wrapper Functional Specification” for complete details on the features and functions of the ECC Memory Wrapper. Refer to section 6.2.2 in this document for specific information on the ECC protected memories within this subsystem.

The ECC wrapper provides Single Error Detection and Correction (SED/SEC). This logic detects and corrects a single bit error (1 bit error per ECC word or per ECC data segment). For memories that contain critical and/or persistent data, automatic (immediate or delayed) write-back of the corrected data to the corresponding memory address is supported. In addition, the ECC wrapper also supports multiple options for partial word writes, such as read-modify-write or multiple ECC code segments per word.

The ECC wrapper also provides Double Error Detection (DED). This logic only detects (does not correct) double errors (2 bit errors per ECC word or per ECC data segment).

An ECC Aggregator at the IP subsystem level consolidates the ECC configuration and status bits for all the ECC supported memories in the subsystem. It provides a single EOI-handshake based interrupt to the host (for both single and double error detections) and a standard 32-bit VBUSP interface for configuring and querying the various ECC wrappers via their ECC register set. Refer to the “ECC Aggregator Functional Specification” for complete details on the features and functions of the ECC Aggregator.

### ECC Memories

The following table lists the memories within this subsystem that are ECC protected, along with ECC attributes for each memory.

Table 4 : ECC Memory Table

| RAM Wrapper | Type | ECC width (bits) | Fast_Q mode | Write-back (Immediate, Delayed, or None) | Partial- word writes |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | rmw | Data splitting |
| csi_rx_if_rx_shim_dma_psil_fifo_tpram_2048x152_sww_sr | tpram | 9 | N | None | N | N |

## PLL Requirements

There are no special PLL requirements for this module.   All clocks generation, gating, distribution is performs by parent modules.

## Overall Design For Test

## Target Processes

The initial target process is 16nm finFET.  While developing RTL, 28nm RAM will be used to DV.

## Area Requirements

.7mm2 in 16ffc

## Power Requirements

## Performance Requirements

#### MAIN CLK

- Target Clock Rate

  - 500MHz 16ffc, 650 Max.

  - f(rx_byte_clk) * nb_lanes / 4 where f(rx_byte_clk) is the minimum frequency of rx_byte_clk_hs. See cadence spec for sys clock requirements.

When MAIN CLK is operating lower than 312.5MHz, then the clock is essentially limiting the clock rate of the DPHY.  Said another way, CLK must be at least the Byte_clk rate else FIFOs will overflow.

#### VBUS_CLK

The VBUS_CLK is a /2 of the main clock.

#### VP_CLK

This is the video port interface clock. It must run at the same speed or higher than main_clk. it can be async to main clock. Must be sync to vpac video clock. 720MHz max rate

#### RX Byte Clock

The Byte clock is the clock supplied by the DPHY.  DPHY is speced for max of 10gbps.  This translates to a max byte clock of 312.5MHz. See cadence spec for more details.

# I/O Description

## Module I/O

The following table provides a summary of the interfaces on this module:

| Interface Name | Description | Clock | Reset |
| --- | --- | --- | --- |
| main_rstn | synchronous Reset Interface For csi | main_clk_clk |  |
| main_clk | free running clock for csi_rx_if |  |  |
| vbus_clk | free running clock for csi_rx_if |  |  |
| vp_clk | free running clock for video port interface. can be async to main clock. restrictions apply |  |  |
| pwr_stop | Clock Power Stop | async | async |
| pwr_clken | Power Clock Enable for csi | async | async |
| csi_dft_local | DFT clock for dphy_rx_byte_clk Clock Domain during Test... muxed in for DFT |  |  |
| dft | DFT Interface |  |  |
| vp0 | master interface for control and data interface for VP output | vp_clk_clk |  |
| vp1 | master interface for control and data interface for VP output | vp_clk_clk |  |
| csi_pulse | Interrupt servicing all CSI TI generated RTL aggregated by INTD | vbus_clk_clk | main_rstn_mod_g_rst_n |
| csi_level | Interrupt servicing all CSI TI generated RTL aggregated by INTD | vbus_clk_clk | main_rstn_mod_g_rst_n |
| csi_irq | CSI irq from CSI2RX submodule | main_clk_clk | main_rstn_mod_g_rst_n |
| csi_err_irq | CSI err_irq from CSI2RX submodule ERROR Interrupts | main_clk_clk | main_rstn_mod_g_rst_n |
| csi_fatal | CSI fatal interrupt from CSI2RX submodule | main_clk_clk | main_rstn_mod_g_rst_n |
| csi_nonfatal | CSI non fatal interrupt from CSI2RX submodule | main_clk_clk | main_rstn_mod_g_rst_n |
| vbusp | Vbusp configuration port | vbus_clk_clk | main_rstn_mod_g_rst_n |
| retran |  | main_clk_clk | main_rstn_mod_g_rst_n |
| ppi |  |  | main_rstn_mod_g_rst_n |
| strm | PSI-L Streaming Interface | main_clk_clk | main_rstn_mod_g_rst_n |
| psil_fifo_ramdft | mem gpio interface | main_clk_clk |  |
| uncorr_pulse | CPU 2 uncorrectable error, pulse | vbus_clk_clk | main_rstn_mod_g_rst_n |
| uncorr_level | CPU 2 uncorrectable error, level | vbus_clk_clk | main_rstn_mod_g_rst_n |
| ram2_ramdft | mem gpio interface | vp_clk_clk |  |
| ram3_ramdft | mem gpio interface | vp_clk_clk |  |

The following sections provide pin level detail for each of the interfaces on this module:

### synchronous Reset Interface For csi

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| main_rstn_mod_g_rst_n | In | 1'd1 | Module level main reset |

### free running clock for csi_rx_if

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| main_clk_clk | In | 1'd0 | Single ended clock |
| main_clk_cs_func_clk_en | In | 1'd0 | catscan enable on kp_ti_ipg_clk_l1 |

### free running clock for csi_rx_if

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| vbus_clk_clk | In | 1'd0 | Single ended clock |

### free running clock for video port interface. can be async to main clock. restrictions apply

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| vp_clk_clk | In | 1'd0 | Single ended clock |

### Clock Power Stop

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_clkstop_req | In | 1'd0 | Clock stop request |
| csi_clkstop_ack | Out | pwr_stop_clkstop_req | Clock stop acknowledge |

### Power Clock Enable for csi

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_clk_en | In | 1'd1 | Clock-gate enable to IP |
| csi_clk_en_ack | Out | pwr_clken_clk_en | Clock-gate enable acknowledge from IP |

### DFT clock for dphy_rx_byte_clk Clock Domain during Test... muxed in for DFT

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_dft_local_clk | In | 1'd0 | Single ended clock |

### DFT Interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| dft_clk_force_en | In | 1'd0 | dft_clk_force_en |
| dft_partition_en | In | {1{1'b1}} | dft_partition_en |
| dft_scan_en | In | 1'd0 | dft_scan_en |
| dft_async_rst_sel | In | 1'd0 | dft_async_rst_sel |
| dft_test_async_rst_n | In | 1'd1 | dft_test_async_rst_n |
| dft_tft_mcp_dis | In | 1'd0 | Assert High to block MCP path |
| dft_local_clk_en | In | 1'd0 | dft_local_clk_en |

### master interface for control and data interface for VP output

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| vp0_vp_data[31:0] | Out | 32'd0 | Parallel output data |
| vp0_vp_vs | Out | 1'd0 | Frame start trigger output signal |
| vp0_vp_ve | Out | 1'd0 | Frame end trigger output signal |
| vp0_vp_hs | Out | 1'd0 | Line start trigger output signal |
| vp0_vp_he | Out | 1'd0 | Line end trigger output signal |
| vp0_vp_stall | In | 1'd0 | input port to stall video data sending |
| vp0_vp_clken | Out | 1'd0 | instead of sending gated clock along with video port data/markers, send clock enable instead. |

### master  interface for control and data interface for VP output

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| vp1_vp_data[31:0] | Out | 32'd0 | Parallel output data |
| vp1_vp_vs | Out | 1'd0 | Frame start trigger output signal |
| vp1_vp_ve | Out | 1'd0 | Frame end trigger output signal |
| vp1_vp_hs | Out | 1'd0 | Line start trigger output signal |
| vp1_vp_he | Out | 1'd0 | Line end trigger output signal |
| vp1_vp_stall | In | 1'd0 | input port to stall video data sending |
| vp1_vp_clken | Out | 1'd0 | instead of sending gated clock along with video port data/markers, send clock enable instead. |

### Interrupt servicing all CSI TI generated RTL aggregated by INTD

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_pulse_intr | Out | 1'd0 | Interrupt signal |

### Interrupt servicing all CSI TI generated RTL aggregated by INTD

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_level_intr | Out | 1'd0 | Interrupt signal |

### CSI irq from CSI2RX submodule

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_irq_intr | Out | 1'd0 | Interrupt signal |

### CSI err_irq from CSI2RX submodule  ERROR Interrupts

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_err_irq_intr | Out | 1'd0 | Interrupt signal |

### CSI fatal interrupt from CSI2RX submodule

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_fatal_intr | Out | 1'd0 | Interrupt signal |

### CSI non fatal interrupt from CSI2RX submodule

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| csi_nonfatal_intr | Out | 1'd0 | Interrupt signal |

### Vbusp configuration port

This is the slave MMR configuration interface with 4Byte addressable range

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| vbusp_req | In | 1'b0 | Request |
| vbusp_rsel[3:0] | In | 4'd0 | Region Select |
| vbusp_dir | In | 1'b0 | Direction |
| vbusp_address[13:0] | In | 14'd0 | Address |
| vbusp_xcnt[2:0] | In | 3'd4 | Good Byte Count |
| vbusp_byten[3:0] | In | {4{1'b1}} | Byte Enables |
| vbusp_wdata[31:0] | In | 32'd0 | Write Data |
| vbusp_wready | Out | 1'b1 | Write Ready |
| vbusp_rdatap[31:0] | Out | 32'd0 | Read Data (Pipelined) |
| vbusp_rready | Out | 1'b1 | Read Ready |

### retran

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| retran_pixel_ready_if | In | 1'd0 | CSI retransmit Data enable |
| retran_vsync_valid_if[15:0] | Out | 16'd0 | CSI retransmit Vertical sync |
| retran_hsync_valid_if | Out | 1'd0 | CSI retransmit Horizontal sync |
| retran_pixel_da_vld_if[3:0] | Out | 4'd0 | CSI retransmit Data enable |
| retran_pixel_byte_vld_if[3:0] | Out | 4'd0 | CSI retransmit Byte enable |
| retran_pixel_data_if[31:0] | Out | 32'd0 | CSI retransmit Pixel data |
| retran_virtual_channel_if[3:0] | Out | 4'd0 | CSI retransmit Virtual Channel |
| retran_pixel_datatype_if[5:0] | Out | 6'd0 | CSI retransmit Data Type |
| retran_pixel_wordcount_if[15:0] | Out | 16'd0 | CSI retransmit Word Count |
| retran_packet_error_if | Out | 1'd0 | CSI retransmit Packet Error |
| retran_stream_idle_if | Out | 1'd0 | CSI retransmit Stream Idle |

### ppi

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| ppi_rx_byte_clk | In | 1'd0 | byte clock |
| ppi_d_rx_active_hs[3:0] | In | 4'd0 | ppi_rx_byte_hs_clk rx_active_hs_dl signal from PHY module |
| ppi_d_rx_sync_hs[3:0] | In | 4'd0 | ppi_rx_byte_hs_clk rx_sync_hs_dl signal from PHY module |
| ppi_d_rx_valid_hs[3:0] | In | 4'd0 | ppi_rx_byte_hs_clk rx_valid_hs_dl signal from PHY module |
| ppi_d_rx_data_hs_0[7:0] | In | 8'd0 | ppi_rx_byte_hs_clk rx_data_hs_dl bus from PHY module lane 0 |
| ppi_d_rx_data_hs_1[7:0] | In | 8'd0 | ppi_rx_byte_hs_clk rx_data_hs_dl bus from PHY module lane 1 |
| ppi_d_rx_data_hs_2[7:0] | In | 8'd0 | ppi_rx_byte_hs_clk rx_data_hs_dl bus from PHY module lane 2 |
| ppi_d_rx_data_hs_3[7:0] | In | 8'd0 | ppi_rx_byte_hs_clk rx_data_hs_dl bus from PHY module lane 3 |
| ppi_d_rx_ulps_esc[3:0] | In | 4'd0 | Asynchronous ,resynchronized into sys_clk, DPHY Data Lane ULPS Escape mode signal |
| ppi_d_stop_state[3:0] | In | 4'd0 | Asynchronous ,resynchronized into sys_clk, DPHY Data Lane Stop State signal |
| ppi_d_rx_ulps_active_not[3:0] | In | 4'd0 | Asynchronous ,resynchronized into sys_clk, DPHY Data Lane ULPS Active signal |
| ppi_c_stop_state | In | 1'd0 | Asynchronous ,resynchronized into sys_clk, DPHY Clock Lane StopState signal |
| ppi_c_ulps_not | In | 1'd0 | Asynchronous ,resynchronized into sys_clk, DPHY Clock Lane ULPS signal |
| ppi_c_ulps_active_not | In | 1'd0 | Asynchronous ,resynchronized into sys_clk, DPHY Clock Lane ULPS Active signal |
| ppi_d_rx_skewcal_hs[3:0] | In | 4'd0 | ppi_rx_byte_hs_clk DPHY Data lanes skew control DPHY Error Condition |
| ppi_d_errsot_hs[3:0] | In | 4'd0 | Asynchronous ,resynchronized into pclk, DPHY Data Lane High Speed Sync Packet |
| ppi_clock_valid | In | 1'd0 | Asynchronous, byte clock is valid |
| ppi_c_rstb | Out | 1'd0 | DPHY Clock Lane Reset |
| ppi_d_rstb[3:0] | Out | 4'd0 | DPHY Data Lane Reset |
| ppi_c_enable | Out | 1'd0 | DPHY Clock Lane Control |
| ppi_d_enable[3:0] | Out | 4'd0 | DPHY Data Lane Control |

### PSI-L Streaming Interface

This interface is a PSI-L endpoint and is used to send and receive data phases from the PSI-L interconnect.

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| strm_i_link | Out | 1'd0 | Link alive indicator |
| strm_i_req | In | 1'd0 | Request |
| strm_i_ready | Out | 1'd0 | Ready |
| strm_i_thread_id[15:0] | In | 16'd0 | Destination Thread ID |
| strm_i_src_id[15:0] | In | 16'd0 | Source Thread ID |
| strm_i_data_type[7:0] | In | 8'd0 | Data type |
| strm_i_wnum[4:0] | In | 5'd0 | Word Number |
| strm_i_lastw | In | 1'd0 | Last Word Indicator |
| strm_i_xcnt[4:0] | In | ALL1S | Valid Data Byte Count |
| strm_i_worden[3:0] | In | ALL1S | Word enables |
| strm_i_data[127:0] | In | 128'd0 | Data |
| strm_i_sop | In | 1'd0 | Start of Packet Indicator |
| strm_i_eop | In | 1'd0 | End of Packet Indicator |
| strm_i_tdown | In | 1'd0 | Channel Teardown Indicator |
| strm_i_interest | In | 1'b0 | Debug marker |
| strm_i_sreq | Out | 1'b0 | Response Request |
| strm_i_sready | In | 1'b1 | Response Ready |
| strm_i_ssrc_id[15:0] | Out | 16'd0 | Response Source Thread ID |
| strm_i_sthread_id[15:0] | Out | 16'd0 | Reaponse Destination Thread ID |
| strm_i_sccnt[7:0] | Out | 8'd0 | Response data phase credit count return |
| strm_o_link | In | 1'd0 | Link alive indicator |
| strm_o_req | Out | 1'd0 | Request |
| strm_o_ready | In | 1'd0 | Ready |
| strm_o_thread_id[15:0] | Out | 16'd0 | Destination Thread ID |
| strm_o_src_id[15:0] | Out | 16'd0 | Source Thread ID |
| strm_o_data_type[7:0] | Out | 8'd0 | Data type |
| strm_o_wnum[4:0] | Out | 5'd0 | Word Number |
| strm_o_lastw | Out | 1'd0 | Last Word Indicator |
| strm_o_xcnt[4:0] | Out | ALL1S | Valid Data Byte Count |
| strm_o_worden[3:0] | Out | ALL1S | Word enables |
| strm_o_data[127:0] | Out | 128'd0 | Data |
| strm_o_sop | Out | 1'd0 | Start of Packet Indicator |
| strm_o_eop | Out | 1'd0 | End of Packet Indicator |
| strm_o_sol | Out | 1'd0 | Start of Line Indicator |
| strm_o_eol | Out | 1'd0 | End of Line Indicator |
| strm_o_tdown | Out | 1'd0 | Channel Teardown Indicator |
| strm_o_pkt_error[3:0] | Out | 4'd0 | Packet Error Indicator |
| strm_o_interest | Out | 1'b0 | Debug marker |
| strm_o_sreq | In | 1'b0 | Response Request |
| strm_o_sready | Out | 1'b1 | Response Ready |
| strm_o_ssrc_id[15:0] | In | 16'd0 | Response Source Thread ID |
| strm_o_sthread_id[15:0] | In | 16'd0 | Reaponse Destination Thread ID |
| strm_o_sccnt[7:0] | In | 8'd0 | Response data phase credit count return |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| psil_fifo_ramdft_gpi[2999:0] | In | 3000'd0 | BIST GPI |
| psil_fifo_ramdft_gpo[2999:0] | Out | 3000'd0 | BIST GPO |

### CPU 2 uncorrectable error, pulse

CPU 2 uncorrectable error, pulse

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| uncorr_pulse_intr | Out | 1'd0 | Interrupt signal |

### CPU 2 uncorrectable error, level

CPU 2 uncorrectable error, level

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| uncorr_level_intr | Out | 1'd0 | Interrupt signal |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| ram2_ramdft_gpi[1999:0] | In | 2000'd0 | BIST GPI |
| ram2_ramdft_gpo[1999:0] | Out | 2000'd0 | BIST GPO |

### mem gpio interface

| Signal Name | Dir | Default Val | Description |
| --- | --- | --- | --- |
| ram3_ramdft_gpi[1999:0] | In | 2000'd0 | BIST GPI |
| ram3_ramdft_gpo[1999:0] | Out | 2000'd0 | BIST GPO |

## External Pin IO

None

## Data Flow Diagram

(Please See Figure 1: Block Diagram)

Figure 10 : Data Flow Diagram

CSI_RX_IF Data Flow

- Input

  - Data is supplied from DPHY to CSI_RX_IF via the PPI interface using CSI2 protocol

  - CSI2 Protocol serially transfers 1 context at a time with minimum granularity of 1 row per burst transfer prior to switching context.

  - TI implementation supports 32 max contexts

- 4 output paths (Stream0/1/2/3)

  - All 4 interfaces can be active simultaneously (unlike CSI_TX_IF)

  - CSI2RX will serially burst on appropriate stream and can even multicast same information

  - Stream0 High BW DMA

    - Up to 32 channel contexts

    - TI add FIFO elasticity buffering

    - Dumps into PSIL which talks to UDMAP

  - Stream1 Retransmit path RX-to-TX

    - Unlimited channel contexts

    - Minimal buffering

  - Stream2 Video Port to VISS

    - 1 single channel context

    - Full line length elasticity FIFO buffer

  - Stream3 Video Port to VISS

    - (same)

# Physical Implementation

## Clocking

There are 4 functional clocks.

- main_clk_clk

  - main module clock that clocks csirx cadence ip.

  - PSIL DMA

  - Re-transmit to CSI_TX_IF

  - 312.5MHz Min clock rate when DPHY is at Max Data rate

    - Slower clock permissible when DPHY is at lower data rates

- Vbus_clk_clk

  - Config interface

- Vp_clk_clk

  - Video Port interface to ISS

- dphy_rx_byte_clk

  - PHY clock domain

  - Supplied by DPHY PLL, divided down to byte clock

  - Max Freq 312.5MHz

  - Inactive clk when PHY is not in HS operation

  - TI implements DFT Mux to take over clock domain for DFT operation

Table 5 : Inter-clock dependency table

|  | Main_clk_clk | Vbus_clk_clk | Vp_clk | Dphy_rx_byte_clk |
| --- | --- | --- | --- | --- |
| Min. freq | Dphy_rx_byte_clk | /2 of main_clk_clk | Main_clk | N/A |
| Max. freq | 500MHz | /2 of main_clk_clk | 720MHz | 312.5MHz |

*see cadence spec for additional clock requirements.

** csi rx/csi tx main clocks should be the same clocks.

## Clock Diagram

CSI Clocking and Reset Diagram

## Reset

Vendor circuits require async resets.  (Please see CLK diagram which contains RST information)

## Latches

There are no latches in the module.

## Placement and Hierarchy Information

TBD

## Hard Macro Information

- CSI_RX_IF: contains no hard macros (with the exception of RAM)

## Power / Voltage

TBD

## Interrupt

- Nothing special

# DFT

## Scan Implementation

Scan implementation details

## IDDQ Modes

There are no specific IDDQ modes or sequences for IDDQ testing.

## Memory BIST

Memory BIST is not included in this module, but can be facilitated by the inclusion of the RAM_GPI and RAM_GPO I/Os.

## I/O DFT

SECTION III: SOFTWARE USAGE GUIDE

# Programmer’s guide

### csirx programming details

- Full line buffer mode is not supported. This is a cadence ip configuration that is not supported.

- It is a restriction that LS/LE detection is disabled.

- Can only use Pixel transmit mode, never packet mode.

- Lsb alignment

- In csirx to csi tx loop back  need to have the same mode on both controllers(single, dual, quad). Can not mix dual on csi rx and single on csi tx.

- Video port stream interfaces must be programmed in the csi rx controller to meet restrictions.

  - Raw 8-16 formats only

  - Dual pixel mode

  - Filtering for single virtual channel and single data type.

- In pixel mode you can only have single, dual, or quad mode. It is not allowed to mix these based on data format or virtual channels(cadence ip restriction).

- NOTE! Csi rx controller has a limitation that any line must fully be exported on it’s stream interface before and new line or even end of frame is sent on the DPHY interface. To meet this requirement you will need to take into account export rates on stream interface(single, dual, quad modes) at 500MHz versus dphy 32-bits @ byte clock frequency to know when a new line or end of frame can be sent in after last line was sent over dphy interface.

Table 6 Programming requirements for pixel modes and data sizes

| format | Pixel transmit mode | size | Comments |
| --- | --- | --- | --- |
| Raw6-8, user defined 8-bit | single | 0 |  |
| Raw6-8, user defined 8-bit | Dual | 1 |  |
| Raw6-8, user defined 8-bit | quad | 2 |  |
| Raw10-16, user defined 16-bit | single | 1 |  |
| Raw10-16, user defined 16-bit | dual | 2 |  |
| Raw 20 | single | 2 |  |
| Yuv422-8 | single | 0 | must set dual mode=0 |
| Yuv422-8 | dual | 0 | must set dual mode=1 |
| Yuv422-10 | single | 1 |  |
| Yuv420-8 | single | 0 | must set dual mode=0 |
| Yuv420-8 | dual | 0 | must set dual mode=1 |
| Yuv420-10 | single | 1 |  |

## Initialization sequence

## Real-time operating requirements

NOTE: Care must be taken on blanking requirements for next line, end of frame, or start of frame. The previous packets(long or short) must be fully consumed before any new line, end of frame, or start frame can be sent over csi interface. You will get various error conditions if these are not met. Below are some sample equations to determine this blanking time. Keep in mind that this is not blanking data. High level details are that the internal fifo must be completely empty prior to sending any thing new into it. below are some sample equations to determine how much time is needed to empty the fifo and blanking time needed. byte clock rates, pixel data type, and export modes play into the calculation.

Byte_clock_rate*8*#lanes*Tcsi = pixel_clock_rate*BPP*Tpix = total bits

BPP = Bits Per Pixel. This number is how many bits are sent per pixel clock cycle is based on single, dual, quad modes and data type. See cadence spec section Multi Pixel Support for Data Types on how to obtain this number.

Tcsi = Time for csi interface to complete 1 line

Tpix = Time for pixel interface to complete 1 line

Tpix16 = Tpix+16/pixel_clock_rate

(Tpix16-Tcsi) = required blanking time on CSI interface

Tpix16-Tcsi<=0 implies 0.

This is an ideal number and likely needs to add around 20% margin in blanking time to be on the safe side to the Tpix16 number.

## Power up/down sequence